3GPP TSG-RAN WG4 Meeting # 103e	R4-2210234

# Electronic Meeting, May 9 - 20, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Band n28 25MHz REFSENS

Agenda Item:	8.21.2.1 Addition of bandwidth and Tx/Rx requirements [NR_bands_R17_BWs-Core]

Document for:	Approval

# Introduction

This document presents our views on supporting 25MHz channel bandwidth (CBW) requested for band n28 at RAN plenary meeting #95 [1]. Considering it was previously agreed to support 30MHz CBW for both uplink (UL) and downlink (DL) operation, it is proposed here to specify band n28 25MHz CBW REFSENS with UL 25MHz CBW configuration. REFSENS is evaluated using Power Amplifier (PA) measurement data. We also propose to update Table 5.3.5-1 footnote 7 to ensure the 25MHz CBW can be supported by legacy dual-duplexer architectures.

# Discussion

Since 30MHz CBW operation is supported for both UL and DL, it is proposed that band n28 25MHz CBW operation also be supported for UL and DL. For 25MHz CBW REFSENS UL RB configuration, we propose to adopt Lcrb=25RB (for SCS 15kHz) with RB positioned closest to the DL band.

## Capturing band n28 UL 25MHz/ DL 25MHz in Table 5.3.5-1

We advance proposals 1 and 2 below to align the introduction of band n28 25MHz CBW with the previous agreements for n28 30MHz CBW.

Proposal 1: For band n28 25MHz CBW, adopt the Table 5.3.5-1 (subclause 5.3.5) changes highlighted in yellow in Table 1.

Table 1: Proposed changes to Table 5.3.5-1 to capture band n28 asymmetric UL/DL 20/25 and 20/30MHz CBW.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n28 | 15 | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25MHz and the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: Void. NOTE 10: These UE channel bandwidths are applicable to sidelink operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For n28 25MHz CBW REFSENS, adopt Lcrb=25 and 10 for SCS 15 and 30 kHz as highlighted in yellow in Table 2 for the UL RB allocation configuration of table 7.3.2-3 (sub-clause 7.3.2).

Table 2: Proposed changes to Table 7.3.2-3 to capture the band n28 25MHz REFSENS UL RB configuration

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |

## n28 25MHz REFSENS

### Power Amplifier Calibration and REFSENS Assumptions

PA Tx noise in Rx band measurement conditions:

- PA is calibrated to meet -30dBc ACLR at PC3 1dB MPR with the 20 MHz channel bandwidth (CBW), SCS 15 kHz, DFT-S-OFDM QPSK 100RB0 waveform;

- Power Class 3 (PC3) operation and 4dB post-PA loss;

- Local Oscillator (LO) leakage: -28dBc and IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5: -70dBc;

- Duplexer rejection: 50dB Tx-Rx, 45dB Tx-Antenna; and,

- n28 UL 20MHz RB configuration: according to Table 3, i.e., Lcrb=25RB at SCS 15kHz, with UL RB positioned closest to the DL band.

### Measurement Results and REFSENS

For UL RB allocation Lcrb=25RB, the UL 25MHz Tx noise level measured in Rx band for DL 25MHz is -33.7dBm. This leads to an MSD after MRC combining of 8.2dB. We therefore propose the following:

Proposal 3: Adopt n28 25MHz CBW REFSENS levels highlighted in yellow in Table 3 for Table 7.3.2-1a (sub-clause 7.3.2).

Table 3: n28 25MHz REFSENS proposals for Table 7.3.2-1a.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 | -83.0 | -78.5 |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 | -83.1 | -78.6 |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This contribution proposes modifications of Table 5.3.5-1 footnote 7 to ensure the 25MHz CBW can be supported by legacy dual-duplexer architectures and brings n28 25MHz CBW REFSENS proposals based on PA Tx noise measurements.

Proposal 1: For band n28 25MHz CBW, adopt the Table 5.3.5-1 (subclause 5.3.5) changes highlighted in yellow in Table 1.

Table 1: Proposed changes to Table 5.3.5-1 to capture band n28 asymmetric UL/DL 20/25 and 20/30MHz CBW.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n28 | 15 | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25MHz and the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: Void. NOTE 10: These UE channel bandwidths are applicable to sidelink operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For n28 25MHz CBW REFSENS, adopt Lcrb=25 and 10 for SCS 15 and 30 kHz as highlighted in yellow in Table 2 for the UL RB allocation configuration of table 7.3.2-3 (sub-clause 7.3.2).

Table 2: Proposed changes to Table 7.3.2-3 to capture the band n28 25MHz REFSENS UL RB configuration

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Adopt n28 25MHz CBW REFSENS levels highlighted in yellow in Table 3 for Table 7.3.2-1a (sub-clause 7.3.2).

Table 3: n28 25MHz REFSENS proposals for Table 7.3.2-1a.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 | -83.0 | -78.5 |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 | -83.1 | -78.6 |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	RP-220160, Revised Basket WID on adding channel bandwidth support to existing NR bands, 3GPP TSG RAN meeting #95e, Ericsson.