# 3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2210265

Electronic Meeting, May 09 – May 20, 2022

Agenda item:			9.4.4, 9.4.5

Source:	Moderator (vivo)

Title:	Email discussion summary for [103-e][130] NR_RF_FR2_enh2_Part_3

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

Topic #1:	DC location

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207660 | Qualcomm Incorporated | Offset and DL CA handling for DC location Proposal 1: Offset range allows for offset that is ½ aggregated BW for defined CA BW class Proposal 2: Inform Ran2 that when UE is configured for one UL CC but more than one DL CC, UE needs to be able to signal the DC location on any DL CC. Proposal 3: Limit the CC groups so that each group contains only one UL CC. Proposal 4: The fallback case should follow the same CC grouping as the higher level CA configuration where the DC location was signalled for DL CCs. Proposal 5: For the UL CC fallback case, the second LO is not assumed to be present anymore. |
| R4-2207679 | Apple | Further views on DC location for intra-band UL CA Observation 1: For intra-band DL contiguous CA where UL carrier leakage is within the DL spectrum range and outside the UL carrier channel BW, DC location reporting may still benefit the network. Observation 2: For intra-band non-contiguous UL CA where UL carrier leakage is within the frequency gap between the two carriers, there would be no benefit to report DC location. Observation 3: The bit width designed for Rel-15 and Rel-16 DC location reporting should be sufficient for the expected offset range. Proposal: Single CC UL is also covered by Rel-17 DC reporting mechanism if there is no technical concern or at least DL only CA is covered. |
| R4-2208385 | Nokia, Nokia Shanghai Bell | Handling of multiple DC locations for intra-band CA Proposal 1: Inform RAN2 of a fact that CC groups do not always consist of multiple CCs, but it may consist of single CC. Proposal 2: For a case of multiple DC locations, where at least one CC group consists of single CC, it is up to RAN2 to decide if still default DC location method + offset method is applied to or Rel-15 method is applied to the CC group with single CC within Rel-17 UL DC reporting mechanism framework. Proposal 3: Adopt +/- 12 GHz or larger(if n263 is taken into account) as an offset range Proposal 4: Send an LS to RAN2 to inform of the fact that UEs may not have to report all the permutations with DC positions. |
| R4-2208387 | Nokia, Nokia Shanghai Bell | [Draft] Reply on LS Reply LS on DC location for >2CC |
| R4-2208609 | vivo | Discussion on remaining issue on DC location Observation 1: Considering the limitation of SEM in the gap, ± 20MHz may preclude many possible CC configurations in FR1. Observation 2: For FR2, the shared LO for both UL and DL can not shift in DL-only spectrum due to the restriction of RF architecture. Proposal 1: For FR1, considering the maximum gap condition, the offset range can be ± 75 MHz to avoid potential restriction on UE implementation. Proposal 2: For FR2, the offset range should be ± 675 MHz and the CC(s) located in DL-only spectrum will not be used to calculate the default DC. Proposal 3: RAN4 only inform a reasonable offset range to RAN2 and no need to further discuss the detail of signaling bit which is depend on RAN2’s decision. Proposal 4: The R17 DC location reporting can also cover 1CC case and the “frequency component” is not needed for this case in FR1. |
| R4-2208610 | vivo | LS on DC location for intra-band CA |
| R4-2209166 | Huawei, HiSilicon | Further study on DC location reporting Proposal: Reuse the 12 bit length of the Rel-16 signalling for Rel-17 DC location report offset range, e.g. Offset range::= INTEGER (-2047, 2047). |
| R4-2209424 | OPPO | R17 FR2 DC reporting 2.1 DC reporting offset Observation 1: To avoid narrow band interferences generated inside UE, e.g. spurs, +/-20MHz is needed in implementation. Observation 2: To allow UE leverage relaxed IBE requirements than difficult OOB requirements when DC is in between CCs, large DC offset might be needed for some UE. Observation 3: Large DC offset could degrade UE performance due to relaxed filter rejections in this case which makes UE normally put DC around the middle. Observation 4: DC offset is larger for FR2 comparing to FR1. Observation 5: At least 15bits is needed if want to cover the largest frequency separation 600MHz in FR1, and 2400MHz in FR2. Proposal 1: Define 15bits SCS based DC offset for both FR1 and FR2. Proposal 2: Clarify that DC offset granularity is the smallest SCS in the configured or activated CCs/BWPs even the SCS of CC which include DC is different from smallest SCS. 2.2 Single CC handling within CA Observation 6: The single CC (non-CA) DC is reported in Rel-15 based on BWP configured. Observation 7: The single CC handling within CA in the exception sheet is about DC reporting when SCCs are deactivated and only PCC working. Proposal 2: Clarify that when SCCs are removed from CA, Rel-15 single CC reporting should be followed; when SCCs are deactivated from CA, Rel-17 solution should still be applied based on the CA configured. 2.3 Handling of DL only CC Observation 8: DC location in DL only CCs is covered by current Rel-17 solution where UE takes both UL and DL CCs into account in DC reporting. Proposal 3: If needed, make necessary clarifications in 101-2 to cover the DC location falling into DL only CCs. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 Offset range

Issue 1-1-1: For contiguous CA, what is the limiting factor of upper bound of the offset range?

- Proposals

  - Option 1: “Aggregated channel bandwidth”

    - FR1: 400MHz , FR2: 1600MHz

  - Option 2: Others

- Recommended WF

  - TBA

Issue 1-1-2: For non-contiguous CA, what is the limiting factor of upper bound of the offset range?

- Proposals

  - Option 1: “Maximum allowed frequency separation”

    - FR1: [600] MHz, FR2: 2400MHz;

  - Option 2:  Based on option 1, further restrict:

    - FR1: 200MHz, which is the maximum bandwidth for 1LO architecture in MPR requirements;

    - FR2: 1400MHz. It means LO can’t within DL-only CCs

  - Option 3: Others

- Recommended WF

  - TBA

Issue 1-1-3: For non-contiguous CA, the situation exists that UE needs to pull the LO from GAP to CCs which requires a fairly large LO offset. e.g. Close to (upper bound/2) in previous issue.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA

Issue 1-1-4: How much signalling bits are needed.

- Proposals

  - Option 1: 15bits for FR1 & FR2;

    - About +/-250MHz for 15KHz SCS and +/-1000MHz for 60KHz SCS

  - Option 2: Provide specific offset range to RAN2, and leave the detailed bits to RAN2 decision

    - E.g., 600 MHz for FR1 and 2400MHz for FR2

  - Option 3: Reuse 12bits for FR1 & FR2;

  - Option 4: Others

- Recommended WF

  - TBA

Discussion in 1st round GTW:

| OFFSET bits | SCS (KHz) | Largest offset (MHz) | Can covered largest FS (MHz) | Largest FS in FR1 spec | Largest FS in FR2-1 spec | [Largest CA bandwidth for FR2-2] |
| --- | --- | --- | --- | --- | --- | --- |
| 15 bits | 15 | 491 | 983 + 2xCBW | FR1= 600MHz | NA | NA |
|  | 60 | 1966 | 3932 + 2xCBW |  | FR2-1= 2400MHz |  |
|  | 120 | 3932 | 7894 + 2xCBW | NA |  | FR2-2 = [2000MHz] |

Note: the largest CA bandwidth for FR2-2 come from WF R4-2202365

- Recommended WF

  - Option 1

Discussions:

Oppo: Option 1 can be used for signaling, which can cover legacy frequency range based on SCS for FR1 and FR2.

Nokia: Basically our preference is Option 3 to minimize the signaling overhead but we can accept option 1 in case that RAN4 asks RAN2 to minimize the signaling not to report all the DC location. In the field network cannot use the DC locations located in the other operator frequency range.

Qulacomm: Where is the bits contained in the RAN2 spec? We are fine with both Option 1 and 2. Option 1 provides the big range. We prefer Option 2.

Apple: Since Rel-17 DC scheme is based on first determining the default location. The final DC location is closed to default DC location bsed on the certain criterion. Our preference is Option 3. We can accept Option 1.

ZTE: It looks like that we are discussing how to define the signaling, which is the RAN2 job. We can only need to provide the frequency range and granularity, which are enough.

Vivo: We can fully understand some companies provide range only and leave bits to RAN2 decision. It is possible that we can have rough range. Even we have some different understanding of many details about the final results how many bits that we should use. Option 2 is with rough range which is agreeable to us.

OPPO: Using this table in our calculation is easier for company to understand how many bits are needed. 15 bits are enough.

Huawei: we share the similar comment as ZTE. The signaling design is RAN2 job. The specific bit number can be removed from the first bullet.

Nokia: we would like to go through all the issues.

OPPO: the granularity is in unit of 15KHz or 60KHz.

Qualcomm: the number of bits would cause confusion in RAN2. Number of bits is not relevant.

Apple: do we need put +/-250MHz?

OPPO: we suggest to use the largest number in RAN4.

Tentative Agreement:

- For Offset range:

  - +/-300MHz for FR1, and +/-1200MHz for FR2

Status summary

Tentative agreement in 1st round GTW reached, pending on final agreement. Tentative agreements for 1st round: Reply an offset range as following in the LS to RAN2: - For Offset range: - +/-300MHz for FR1, and +/-1200MHz for FR2 Recommendations for 2nd round: Confirm this tentative agreement.

Sub-topic 1-2 Single CC reporting

Issue 1-2-1: What is the applicability of single CC reporting in Rel-17 DC reporting scheme?

- Proposals

  - Option 1: Applicable for non-CA case in UL (i.e. UL SCCs not exist or removed by network, only DL CA is configured).

  - Option 2: Applicable in case UL CA are configured, but only one CC is activated. Not applicable for non-CA case in UL, as described in Option 1.

  - Option 3: Applicable for both cases:

    - non-CA case in UL (i.e. UL SCCs not exist or removed by network, only DL CA is configured)

    - UL CA are configured, but only one CC is activated.

  - Option 4: Others

- Recommended WF

  - TBA

Discussion in 1st round GTW:

- Recommended WF

  - Option 3

Discussion:

Nokia: the situation is not changed according to UE configuration.

Agreement: agree on Option 3.

Status summary

Agreement reached in 1st round GTW. Agreements for 1st round: Agreement: agree on Option 3. Recommendations for 2nd round: None.

Issue 1-2-2: Will Rel-15 single CC reporting be still applicable for Rel-17 multiple DC location reporting?

- Proposals

  - Option 1: Only Rel-17 reporting scheme can be used throughout different UL CC groups.

  - Option 2: Rel-15 reporting can also be used in case the UL CC group consists of only one CC.

  - Option 3: Others

- Recommended WF

  - TBA

Discussion in 1st round GTW:

- Recommended WF

  - Option 1

Discussions:

Nokia: We would like to leave RAN2 to discuss Option 2. There is no permutation of CCs. UE just reports DC within CCs.

OPPO: OK with Option 1. To clarify, what we are discussing is for single band intra-band uplink CA. In the future, there will be 3 bands for one band there will be intra-band CA. here we are just discussing the single band.

Huawei: we believe only intra-band is under discussion. We need to make it clear that the agreement is for intra-band.

Vivo: the preivous agreeme applies for single band. The reporting scheme varies across bands and for one band the scheme is the same.

Nokia: we did not intent to have multiple bands.

Nokia: in Rel-16 the reporting is per band combination.

Qualcomm: for example, FR1 1 band + FR2 many bands and CCs, then UE uses Rel-16 scheme for FR1 band and use the Rel-17 schemes on FR2.

OPPO: We agree with Qualocmm on the situation. We are OK with agreement. If FR1+FR2 band combination, there will be a bit complicated.

Agreement:

- Only Rel-17 reporting scheme can be used throughout different UL CC groups on the same band

Status summary

Agreement reached in 1st round GTW. Agreements for 1st round: Agreement: - Only Rel-17 reporting scheme can be used throughout different UL CC groups on the same band Recommendations for 2nd round: None.

Sub-topic 1-3 Others

Issue 1-3-1: Whether LO can locate in DL-only spectrum?

- Proposals

  - Option 1: Yes.

  - Option 2: No

  - Option 3: Others

- Recommended WF

  - TBA

Discussion in 1st round GTW:

- Recommended WF

  - TX LO cannot locate in the DL only spectrum

    - The DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3.

Discussions:

OPPO: we are OK with the recommendation. The DL only spectrum is only RAN4 terms. We care more about DL only CC.

Nokia: We do not see the necessity. If DC is not located in UL spectrum, UE does not need to report.

Qualcomm: DL only spectrum is not suitable for DC location. DL only spectrum is specified in 38.101. We can use the term to communication with RAN2.

Vivo: In our understanding, this issue intends to calrify the calculation of DC. The DL only spectrum and DL CC should not be involved in calculation.

Agreement: TX LO cannot locate in the DL only spectrum

- The DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3.

Status summary

Agreement reached in 1st round GTW. Agreements for 1st round: Agreement: TX LO cannot locate in the DL only spectrum - The DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3. Recommendations for 2nd round: None.

Issue 1-3-2: About single CC in one CC group

- Proposals

  - Proposal 1: Considering current spec restriction (1LO for FR2, maximum 2CCs UL for FR1), limit the CC groups so that each group contains only one UL CC; (Qualcomm: R4-2207660)

  - Proposal 2: Clarify CC groups do not always consist of multiple CCs, but it may consist of single CC (Nokia: R4-2208385).

- Recommended WF

  - TBA

Discussion in 1st round GTW:

- Recommended WF

  - Proposal 2 is acceptable, but more discussion is needed for proposal 1.

Discussions:

OPPO: OK with proposal 2. For proposal 1, it is OK for Rel-17 since there are only at most two CCs. But since Rel-17 reporting applies for more CCs, the restriction is not reasonable any more.

Vivo: for proposal 1, it clarifies the configuration. Does it intent to provide information to RAN2 to help signaling design? If so, we are OK. We do not want to have restriction.

Qualcomm: the proposal 1 comes from the discussion with RAN2 colleague. There is only one CC within a group. It just helps RAN2.

Agreement:

- Clarify CC groups do not always consist of multiple CCs, but it may consist of single CC

- To help RAN2 design in Rel-17, RAN4 provides the following information

  - Considering current spec restriction (1LO for FR2, maximum 2CCs UL for FR1), limit the CC groups so that each group contains only one UL CC

Status summary

Agreement reached in 1st round GTW. Agreements for 1st round: Agreement: - Clarify CC groups do not always consist of multiple CCs, but it may consist of single CC - To help RAN2 design in Rel-17, RAN4 provides the following information - Considering current spec restriction (1LO for FR2, maximum 2CCs UL for FR1), limit the CC groups so that each group contains only one UL CC Recommendations for 2nd round: None.

Issue 1-3-3: About DL&UL fallback behaviour and CC groups. (Qualcomm: R4-2207660)

The question what happens in fallback case here. If one DL CC is deactivated or de-configured, then there is no change in CC grouping since the reason for the second LO was two UL CCs. If the second UL CC is deactivated/deconfigured, the second LO is not needed anymore.

- Proposals

  - Proposal 1: The fallback case should follow the same CC grouping as the higher level CA configuration where the DC location was signalled for DL CCs.

  - Proposal 2: For the UL CC fallback case, the second LO is not assumed to be present anymore.

- Recommended WF

  - TBA

Discussion in 1st round GTW:

- Recommended WF

  - Need more discussion

Discussions:

Qualcomm: A+(B+C), what if C is deacitvated, then (A)+(B) or the group needs to be re-communicated? In this case, there will be only one LO.

Apple: the fall back is equal to deacitvation of CC? They may be different. The activation means multiple CC is de-/activated but the configuration is not changed. In the case of activation/de-activtion, we think he DC location has been covered by Rel-17 default scheme. The confusion part would be two DC lcoations are within the different groups. If one group is deactivated, there is no need to have DC for that group. There should not be too much confusion for this case.

OPPO: for proposal 1, it may not be always true. Each CC group is associated with DC location. The default DC lcoation framework applies to each CC group. UE uses the default DC to repot offset. In the previous LS to RAN2, default DC is mandated to configure.

Huawei: are these two proposals relevant to reporting? Deactivation and deconfiguration are different. We should better understand Rel-17 scheme can work and no issue.

Qualcomm: the fall back is the only case that something is configured. RAN2 tries to save something.

Status summary

Views are still diverse and no agreements so far. There are different understandings of the behavior. Agreements for 1st round: None. Recommendations for 2nd round: Continue discussion and see if certain agreements can be reached and if certain information can be sent to RAN2.

Issue 1-3-4: Do we need to inform RAN2 UEs may not have to report all the permutations with DC positions. (Nokia: R4-2208385).

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA

Discussion in 1st round GTW:

- Recommended WF

  - Need more discussion

Discussions:

Nokia: network must be allowed to inform UE that some DC falling in a certain frequency range will not be used. We can save some overhead that UE does not need to report such information.

OPPO: generally we are OK with the statement that UE won’t need to report all the DC location. But for which location not being reported, it is up to UE. For the situation Nokia commented that some DC locations are within other operators, we understand but it is difficult for UE to know such information.

Qualcomm: it is for signaling simplification. To Nokia, we need to see the details when and where and how to provide information to RAN2. In principle we are OK but it is difficult in reality. We need clear rules.

Apple: the original intent of DC reporting is to get help from gNB. gNB can do DC removal processing to improve the uplink signal quality. If DC location is outside uplink bandwidth of UE, it does not help interms of improving the channel Qualcomm. In this case, UE either can not report or report 3000/3001 means out-of-carrier or unknown. Whether to report DC should not be mandatory. If DC falls in the gap for non-contiguos, gNB needs do nothing.

Huawei: We prefer option2. RAN2 kicks off the signalign design. We can wait for RAN2 LS to see if they need help from RAN4.

Nokia: To Apple, Apple assumption is that UE knows where the operator spectrum in the beginning. But UE may not have such information. If DC location falls outside of frequency range of operators, UE can report 3000/3001 but UE may not know where the frequency is in the beginning. Network needs tell UE where the spectrum is. It should be taken into account in RAN2 signaling design. We do not need to force RAN2 to take it, but we should inform RAN2 that some DCs fall into other operators’ spectrum does not need to report. The final decision is up to RAN2.

Status summary

Views are still diverse and no agreements so far. Majority companies can accept some kind of clarification, but the details are still unclear. Agreements for 1st round: None. Recommendations for 2nd round: Continue discussion and see if certain agreements can be reached and send to RAN2.

Companies views’ collection for 1st round

Open issues

Sub topic 1-1 Offset range

| Issues | Company & Comments: |
| --- | --- |
| Issue 1-1-1: | Company A: |
|  | vivo：option 1 can represent the worst case. Nevertheless, contiguous CA may not require such large offset if a suitable “frequency component” is reported. |
|  | OPPO: Issue 1-1-1 and 1-1-2 can be combined together, the offset range need to cover both contiguous and non-contiguous CA. And when in contiguous CA, now the max aggregated CBW is Option 1, i.e. 400MHz FR1, and 1600MHz FR2. |
|  | Qualcomm: Support option 1 |
|  | Nokia(HU): Option 2. We need to agree with a basic understanding that how the reported DC location can be utilized by a network. Limitation due to Intra band contiguous CA or non-contiguous CA does not always become the limiting factor since all the UEs are not always configured with all the CCs available in the network or outermost CCs in the network. Suppose that 3GHz spectrum is available in a network while specification may allow 100 MHz x 8 CA in it and a UE1 may be configured with 8 CCs in the middle of the spectrum. If the UE1’s DC falls outside of the 8CCs, still this report may be meaningful since some other UEs may be using the spectrum outside of the configured 8CCs for the UE1. In this sense, in an extreme case, a spectrum in a different operating band by the same operator outside the band being used by a UE can be also needed to be considered while to make the situation simpler, we are ok to limit the discussion on the maximum pass bandwidth among the specified operating bands. Hence, 600=1200/2 MHz for FR1 and 2=4/2 GHz for FR2-1(if FR2-2 is considered, it’s 12=24/2 GHz). On the other hand, if we go with these large offset values, we need to tell RAN2 that network does not always need all the DC information, e.g., DCs falling another operators’ spectrum are not needed, to leave RAN2 to consider reducing a signaling overhead. |
|  | Apple: It is unclear why the DC can be so far away from the default location. If there would be such cases, it seems like the default DC approach is not quite useful. |
|  | vivo2: as for FR2-2 CA that Nokia mentioned, there is an agreement in previous meeting: Agreement: Use the following as the starting point for further discussions - Consider n x 400 MHz, n= [2, 3, 4, 5] and m x 100 MHz, m=[ 2..8] as the supported channel BW options for CA operation in unlicensed band for total bandwidths up to 2000 MHz. It seem the FR2-2 CA bandwidth will be no larger than 2000MHz in R17 |
| Issue 1-1-2: | vivo: we prefer option 2. For FR1, if the gap larger than aggregated bandwidth, MPR is undefined, and for FR2, only the DL CC(s) which share LO with UL CC will affect the DC location. |
|  | OPPO: Option 1, i.e. 600MHz FR1, and 2400MHz FR2. And it can cover the range of contiguous CA in Issue 1-1-1. |
|  | Qualcomm: Both ok but agree with Oppo that for max we will need one number for each FR. |
|  | Nokia(HU): The same comments in Issue 1-1-1 |
|  | Apple: 100MHz for FR1 and 400MHz for FR2 (single CC maximum channel BW) |
| Issue 1-1-3: | vivo: option 1, this is the main reason why larger offset range is necessary. |
|  | OPPO: It is possible in theory, but some other implementation issues will be caused when the DC offset is quite large because it will require the analogue filter to be extended largely then cause the UE performance degraded. From the spirit of not excluding UE implementations, it would be ok to consider this situation. Therefore, Option 1. |
|  | Qualcomm: This is possible. Option 1. |
|  | Nokia(HU): it would be great if the moderator could share what he or she would like to achieve with the question. Is this mentioning a case that Qualcomm has raised for a few meetings? |
|  | Apple: It is unclear what it means “UE needs to pull the LO from GAP to CCs which requires a fairly large LO offset”? |
|  | Huawei: Similar question as Nokia. Seems unnecessary to confirm an implementation choice but without any spec impacts. (if our understanding correct?) |
| Issue 1-1-4: | Option 1 or option 2 is OK for us. If we can not make any consensus on specific offset range, maybe the signalling bit would be a good one to try. For option 3, we are afraid 12bit is not enough. |
|  | OPPO: Option 1, 15 bits SCS based ranges for FR1 and FR2 will be enough to cover the largest frequency separation 600MHz in FR1, and 2400MHz in FR2 as below. OFFSET bits; SCS (KHz); Largest offset (MHz); Can covered largest FS (MHz); Largest FS in FR1 spec; Largest FS in FR2 spec. 15 bits; 15; 491; 983 + 2xCBW; FR1= 600MHz; NA. 60; 1966; 3932 + 2xCBW; FR2= 2400MHz. 120; 3932; 7894 + 2xCBW; NA. |
|  | Qualcomm: Option 2. Number of bits in signaling interface is not ran4 UE RF issue at all and ran4 should use its limited time to discuss this nor it should be any reason to make choices for RAN4 design or requirements. |
|  | Nokia(HU): Whichever option is selected(specifically, if go with Option 1), RAN4 shall share the fact that network does not need all the DC locations from all the frequency component permutations. If this information was not shared with RAN2, then, we should select the smallest offset range. Otherwise, the amount of signaling would become too much. |
|  | Apple: We think 12 bits is sufficient for both FR1 and FR2. |

Sub topic 1-2 Single CC reporting

| Issues | Company & Comments: |
| --- | --- |
| Issue 1-2-1: | Company A: |
|  | vivo: we prefer option 3. In our understanding, if UE support R17 reporting mechanism, it’s depend on NW which mechanism will be used and no need further restriction |
|  | OPPO: Option 3. But the single CC sometimes is misleading, and it doesn’t mean 1CC UL+1CC DL, and it should mean at least in UL or DL there is CA configured. - In previous LS to RAN2, it has clarified that whether UE will take UL only or both UL and DL CCs into account in determining the default DC location is up to UE indication. And in FR2 the DL only CA will also be considered in UL IBE requirements. Therefore, non-CA in UL but CA in DL is applicable. - It should also be clear that when NW configured a SCC no matter this SCC is activated or not this UE will be working in CA mode. It can fallback to non-CA only when NW de-configure SCC. Therefore, it should also applicable when UL CA configured but not activated. |
|  | Nokia(HU): Option 3. For Option 1, it depends on what the reported frequency component is, e.g., DL, UL, configured, or activated. If frequency component is DL configured CCs, this is not single CC case anymore. If it is UL CC, this is a case that the number of frequency component is one. In this case, the issue is the same as that of Issue 1-2-2. For option 2, again, it depends on reported capability for frequency component. If the UE reports affecting factor is configured CC, if only one of the CCs is activated or not does not matter. Hence, it can be considered as CA case. If it reports activated CC as an affecting factor, the issue is the same as that of Issue 1-2-2, i.e., the number of frequency component is one. |
|  | Apple: Option 3 |
|  | Huawei: Option 3. |
| Issue 1-2-2: | Company A: |
|  | vivo: in the last meeting, we have agreement as follow: For a UL CA configuration in one band combination, there is only one mechanism will be used for DC location(s) reporting. So option 1 seems more align with previous agreement. |
|  | OPPO: Option 1. Only 1 DC reporting scheme should be applied in a intra-band CA configuration. Mixed DC reporting schemes will cause unnecessary complexity since it is much simpler to get all the DC information from single signaling IE than checking Rel-15 and Rel-17 different IEs. And there is no problem if Rel-17 scheme is used in the scenario of group A with several CCs and group B with 1 CC case. For UE whose default DC depends on the activated CC/BWP, the CC grouping might also change from time to time. Mixed schemes will also cause difficulty for NW to read the DC locations. |
|  | Nokia(HU): Option 3. First of all, we did not say that “UL” CC group since a UE may report that frequency component is configured DL CC or activated DL CC. What we proposed is incorporation of a part of Rel-15 mechanism into Rel-17, but not exactly the same scheme of Rel-15 to be used in Rel17 scheme. If we agree that a DC location to fall outside the configured or activated frequency component should be reported, then, strictly speaking, Rel-15 scheme must not work as it is since it can only report a DC location within the carrier. What we want to tell RAN2 is that CC group may consist of single CC and if default DC + offset still applies to the single CC, then, the default must be the center of the outer edges of the frequency component. |
|  | Apple: Option 1 |
|  | Huawei: First we would like to emphasis that we should not couple Rel-15 scheme with Rel-17 scheme, because Rel-15 scheme for single CC is mandatory and Rel-17 for intra-band CA is optional. As for the above example, if that is a case for intra-band UL CA, then we believe the agreement from last meeting (copied by VIVO) has already covered it, i.e. Option 1. |

Sub topic 1-3 Others

| Issues | Company & Comments: |
| --- | --- |
| Issue 1-3-1: | Company A: |
|  | vivo: option 2. As we comment above, only the DL CC(s) within the UL and DL shared spectrum will affect the DC location. A reference architecture for intra-band non-contiguous CA is as follow which is also used in R16 DL-only discussion If DL-only CC share same LO with UL CC, the DL-only spectrum can not extend on one-side which is not align with the description in current spec. |
|  | OPPO: Option 1, Yes. And for FR2 this DC location should be clear. For FR1 UE can simply report 3300 or 3301. |
|  | Qualcomm: TX LO can not be on the DL only spectrum but to ensure, the DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3. TX LO can be on spectrum occupied by DL only CC. Option 2 with the understanding above. |
|  | Nokia(HU): Option 3. We think that when we use DL-only spectrum, clarification is needed. If it means for example, a UE is configured with one UL CC and two DL CCs and one of the DL CC is the DL-only spectrum for “the said” UE or the spectrum is never used for UL for any UEs under the network. If the DL-only spectrum here means is the latter one, at least network would not need DC location information since the network cannot utilize that information at all. And if it exists as vivo’ paper mentioned, the power is quite low, isn’t it? Does UE vendors need an exception for that case to meet some existing requirements? |
|  | Apple: What is the concern we intend to address here? The DC/LO is outside of the UL carrier but within the DL CA range? Our understanding is that such scenario should already be covered by the Rel-17 method. It is up to gNB whether the DC/LO would be processed if it is outside of UL channel BW. |
|  | Huawei: The same with QC. Seems the “DL-only spectrum” is misused here? |
|  | vivo2: To Nokia and Apple: For FR2 we agree that DL CC also impact the default DC, so we define the frequency component as follows: Frequency component = Calculated relative to either 1) UL or 2) DL frequencies of the frequency component or 3) edge most frequencies among any DL and UL frequency components It seems we have different understanding for DL-only spectrum and the DL-only CC in our comment is the DL CC(s) locate in Fsd which is described in TS 38.101-2 clause 5.3A.4 as Qualcomm mentioned. Our concern here is these CC(s) will not impact the DC location so should be preclude from the calculation of default DC, and this information should let RAN2 know. |
| Issue 1-3-2: | Company A: |
|  | vivo: both proposal are ok for us, and for proposal 1, this clarification only applies to R17. |
|  | OPPO: For Proposal 2, it is ok. For Proposal 1, no. there is no need to do this limitation since Rel-17 solution will be used for more than 2UL CC cases. In the LS to RAN2, it is clear that UE will report the default DC location and its related CCs, then it will be clear for NW which DC mapping to which CCs. What we can say is each group contains only one DC instead of one UL CC. |
|  | Qualcomm: To simplify and there is no need, ran4 spec up to rel-17 only has one CC per group for 2 UL LO case. |
|  | Nokia(HU): For proposal 1, if DL only spectrum can have a DC as vivo’s paper mentioned, then, this proposal 1 cannot hold true since the DL spectrum cannot have any UL CC on it, though we may not need to take care of the case raised by vivo if the power of the LO only for DL only is quite low. We support Proposal 2. |
|  | Apple: Proposal 2 is more general and future proof. Proposal 1 states the existing configuration constraint and practical UE implementation for FR1 and FR2. Proposal 1 is a subset of proposal 2. As long as the Rel-17 DC location method can handle single CC and multiple CCs as a group, there should not be concern with the proposals. |
|  | Huawei: Option 2 is OK. |
| Issue 1-3-3: | Company A: |
|  | vivo: OK with the clarification. |
|  | OPPO: It is confusing about “fallback case”, it may mean de-configure, or deactivate. About Proposal1: For UE whose DC is dependent on the configured CC/BWP will not change the DC location and also grouping if NW only de-activate the CC/BWP. But for UE whose DC is dependent on the activated CC/BWP may change the DC location and also grouping if NW deactivate or de-configure the CC/BWP. Therefore, Proposal 1 may not be correct in some cases. And the relation of CC group and DC are reported by UE in the DC reporting, there is no need to do this restriction. About Proposal 2: As commented in issue 1-3-2, there is no need to do this limitation, since the Rel-17 solutions will not only cover the 2UL CC case but also more than 2CC cases. And the DC location is reported by UE, if there is no 2nd DC exists then UE simply don’t report it. This is straight forward, and we don’t see the benefits of this limitation. |
|  | Qualcomm: To further clarify the intent, if SCell that is part of the second group is deactivated, then the second LO is not present anymore? This is the intention of the proposal 2. Depends on outcome of the issue 1-3-2, right now ran4 requirements exists only for 2 LO case when there is max 2 UL CCs. |
|  | Nokia(HU): This is related to the Issue 1-3-1. If the agreement of 1-3-1 is option 1, the following part cannot hold true “. If the second UL CC is deactivated/deconfigured, the second LO is not needed anymore” |
|  | Apple: For single DC location, this issue would not be a concern as the default location can be determined by either configured/activated CC/BWP for UL/DL. The clarification is mainly for two DC locations where if the UL containing one of the DC is deactivated, it should be clear enough that the DC associated with the deactivated CC should not exist. |
|  | Huawei: We would like to better understand these two Proposals. For Proposal 1, why the introducing of Rel-17 DC reporting scheme will limit the fallback behavior? For Proposal 2, our understanding is that whether a CC is deactivated or de-configured is determined by gNB. The CC index of each CC group is also known to gNB by UE reporting. Then is there any potential issue for not having such clarification? |
| Issue 1-3-4: | Company A: |
|  | vivo: prefer option 1, maybe RAN2 can define a dedicated signal to address such situation just like 3300/3301 in previous release. |
|  | OPPO: Option 2, no. The DC reporting is decided by UE, and it is difficult for UE to report DC location based on which spectrum belongs to which operator beforehand and sometimes UE doesn’t know that since the spectrum of each operator may change. |
|  | Nokia(HU): Option 1. This is also somehow related to Apple’s paper. Apple is saying that the DC information by a UE to fall into a carrier(s) which is not used by the UE should be reported if the carrier is used by the same network. We are saying that if the DC by a UE to fall into a carrier(s) which is not used by the UE should NOT be reported if the carrier is NOT used by the same network. It’s just not useful. |
|  | Apple: Our understanding is that DC reporting is not mandatory. Some UE may choose not to report DC at all as they do not need to rely on gNB to remove DC to improve EVM performance. As from emission requirement point of view, if the carrier leakage is outside the UL channel BW, UE needs to take care of the in-band emission or out-of-band emission by itself. |
|  | Huawei: Option 2. |
|  | Nokia: We think that our understanding with OPPO and Apple is very close. To make the discussion more specific, assume that n78(3300-3800MHz) and an operator has two blocks, 100MHz and 200 MHz which are 3300 - 3400MHz and 3600 - 3800MHz. A UE is using CA with 100 MHz@3300-3400MHz and 100MHz@3700-3800MHz under operator “A” network. The UE may want to report (based on UE’s decision) DC(s) to fall into 3400 -3600 MHz. This information is surely not helpful for operator A’s network at all. The UE may also want to report (based on UE’s decision) DC(s) to fall into 3700-3800MHz which is not used by the UE. While this information can be useful for operator “A” network since some other UEs using operators’ spectrum in 3700-3800MHz as UL. For both cases, network needs to tell the UE that their spectrum is 3300-3400MHz and 3600-3800MHz or their spectrum is NOT within 3400-3600MHz, or any other means. Since UE can know where their DC(s) for each permutation falls, the UE can refrain from reporting DC(s) which is NOT useful. It’s up to RAN2 to incorporate this aspect, but at least we must tell RAN2 this information. We did spend much time on offset frequency range and discussed the number of bits. That signaling bit discussion was important but this is even more important since we can avoid signlling which is not useful at all. That should be really avoided. It doesn’t make sense that we increase the number of signalling overhead per permutation AND allow UE to report meaningless information with the increase signalling bits. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Moderator’s note: This section has been moved to Clause 1.2 under respective issues and sub-topics.

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: FR2 CA BW classes

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207680 | Apple | Further views on new FR2 CA BW classes Observation 1: Option 2b had overlooked the supported maximum number of 100MHz cell which should be constrained at 9 as originally proposed in [7]. Observation 2: (BCS) in the fallback group column is irrelevant to CA BW classes definition and must have been a typo which should be removed. Observation 3: If an odd multiple of 100MHz spectrum is extended from the existing deployment (such as 100MHz, 300MHz, etc.), all the added new cells can be 200MHz but with edge cell configured at 100MHz only. Observation 4: For Option 2c, there is no constraint on the maximum number of 100MHz cell which would seemingly defeat the purpose of urging the new network cell migration from 100 MHz to 200 MHz. Observation 5: Option 2c has a potential fallback issue if the UE total aggregated BW capability is less than the upper bound of each CA BW class. Observation 6: To support 1600MHz network based on (8x100MHz + 4x200MHz) configuration, UE would be required to support up to 2400MHz total aggregated channel to avoid the fallback issue which could be a huge design stretch to only support a 1600MHz network. Observation 7: For Option 2c, one possible mitigation to avoid the fallback issue without UE supporting more than 1600MHz aggregated BW is to retain the CA BW classes R, S, T, and U in order for UE to signal CA BW class R12 together with CA BW class U. Observation 8: Option 3 has the merit of needing only 4 new CA BW classes as compared to 11 for modified Option 2b and Option 2c. Observation 9: Option 3 has the same fallback issue as with Option 2c which however may not be resolvable. Observation 10: Option 3 can only support very limited numbers of fallback configurations. Observation 11: The modified Option 2b is the most preferred way forward for the new FR2 CA BW classes definition to support up to 1600MHz aggregated BW with full backward compatibility. In this contribution, we share our views on the 3 down-selected options of the new CA BW classes definition and propose the modified Option 2b shown in the table below as the way forward. NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. V2; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 2; 5. V3; 300 MHz ≤ BWChannel_CA ≤ 600 MHz; 3. V4; 400 MHz ≤ BWChannel_CA ≤ 800 MHz; 4. V5; 500 MHz ≤ BWChannel_CA ≤ 900 MHz; 5. V6; 600 MHz ≤ BWChannel_CA ≤ 1000 MHz; 6. V7; 700 MHz ≤ BWChannel_CA ≤ 1100 MHz; 7. V8; 800 MHz ≤ BWChannel_CA ≤ 1200 MHz; 8. V9; 900 MHz ≤ BWChannel_CA ≤ 1300 MHz; 9. V10; 1100 MHz ≤ BWChannel_CA ≤ 1400 MHz; 10. V11; 1300 MHz ≤ BWChannel_CA ≤ 1500 MHz; 11. V12; 1500 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. |
| R4-2208314 | MediaTek Beijing Inc. | Discussion on FR2 mixed-CC CA BW class Proposal1: No matter which option is selected, below statements shall be applied by certain manner: - The interlacing CC bandwidth is not allowed. - Limit the maximum aggregated bandwidth to 1600MHz. - 50MHz channel bandwidth is not supported. Observation1: The main difference between 2b and 2c is “whether to leave a more generic framework or not”. Observation2: The main difference between 2c and 3 is “whether 100MHz x 9 to 100MHz x 12 are allowed or not”. Observation3: In our understanding, there is operator demand on fallback to “100MHz x 9 to 100MHz x 12”. Proposal2: Define Option 2b or 2c because of operator’s demand on fallback flexibility. |
| R4-2208486 | Samsung | Discussion on CA BW class of mixed 100MHz and 200MHz CCs Observation 1: Option 2b could not avoid the necessity of defining new FBGs when there is future demand of more than 4x200MHz deployment mixed with 100MHz CCs, and option 3 changes fall back rules. Observation 2: RAN4 agreement of no interlacing CC bandwidths may conflicts with current RAN2 requirement. Proposal 1: RAN4 to specify new CA BW class of mixed 100Hz and 200MHz CCs based on option 2c in Alternative 1 by implementing agreements of no interlacing CC bandwidth and restriction of 1600MHz upper limit. Proposal 2: It is proposed to send the RAN4 agreements of no interlacing of CC BW for new FBG5 to RAN2 with LS. Proposal 3: RAN4 to implement the 1600MHz maximum aggregated bandwidth for Rel-17 UE from following options - option 1: removing R9 ~ R12 which is beyond 1600MHz from option 2c - option 2: adding a note for R9 ~ R12 to limit the maximum aggregated BW as 1600MHz - option 3: limit the maximum aggregated BW as 1600MHz in Table 5.5A.1-1 of TS 38.101-2 |
| R4-2208752 | Ericsson, Verizon | Draft LS to RAN2 on FR2 bandwidth classes covering up to 2400 MHz aggregated bandwidth with mixed carrier bandwidths Proposal 1: adopt Option 2c without interlaced 100 MHz and 200 MHz channel bandwidths, the existing fallback rule already applied in the field should be maintained - the BW classes R-U no longer needed, and the D-F would be obsolete (hence FBG2 obsolete). This means that the new FBG 5 is four new classes in practice - inform RAN2 accordingly Proposal 2: for a supported band combination of CA BW class with more than eight CCs, the associated featureSetCombination is used to limit the CC bandwidths to an aggregated BW of 1600 MHz in one or more featureSets |
| R4-2208753 | Ericsson, Verizon | FR2 CA BW classes up to 2400 MHz aggregated BW with mixed channel bandwidths Table 5.3A.4-1: CA bandwidth classes NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. A; BWChannel ≤ 400 MHz; 1; 1,2,3,4,5. B; 400 MHz < BWChannel_CA ≤ 800 MHz; 2; 1. C; 800 MHz < BWChannel_CA ≤ 1200 MHz; 3. D; 200 MHz < BWChannel_CA ≤ 400 MHz; 2; 2. E; 400 MHz < BWChannel_CA ≤ 600 MHz; 3. F; 600 MHz < BWChannel_CA ≤ 800 MHz; 4. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. O; 100 MHz ≤ BWChannel_CA ≤ 200 MHz; 2; 4. P; 150 MHz ≤ BWChannel_CA ≤ 300 MHz; 3. Q; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 4. R2; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 2; 5. R3; 300 MHz ≤ BWChannel_CA ≤ 600 MHz; 3. R4; 400 MHz ≤ BWChannel_CA ≤ 800 MHz; 4. R5; 500 MHz ≤ BWChannel_CA ≤ 1000 MHz; 5. R6; 600 MHz ≤ BWChannel_CA ≤ 1200 MHz; 6. R7; 700 MHz ≤ BWChannel_CA ≤ 1400 MHz; 7. R8; 800 MHz ≤ BWChannel_CA ≤ 1600 MHz; 8. R9; 900 MHz ≤ BWChannel_CA ≤ 1800 MHz; 9. R10; 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz; 10. R11; 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz; 11. R12; 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz; 12. NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.. |
| R4-2208864 | Xiaomi | Discussion on FR2 new CA BW classes Proposal 1: Limiting the combination up to 8x100MHz CCs and up to 4x200MHz CCs for CA operation for mix of 100 and 200 MHz CCs. Proposal 2: introduce new CA BW classes in an existing FBG as Option 3b/3c: Table 2-3 Option 3b NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. V; 900 MHz ≤ BWChannel_CA ≤ 1300 MHz; 9. W; 1000 MHz ≤ BWChannel_CA ≤ 1400 MHz; 10. X; 1100 MHz ≤ BWChannel_CA ≤ 1500 MHz; 11. Y; 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. Table 2-4 Option 3c NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. V3; 400 MHz ≤ BWChannel_CA ≤ 500 MHz; 3. V4; 500 MHz ≤ BWChannel_CA ≤ 700 MHz; 4. V5; 600 MHz ≤ BWChannel_CA ≤ 900 MHz; 5. V6; 700MHz ≤ BWChannel_CA ≤ 1000 MHz; 6. V7; 800 MHz ≤ BWChannel_CA ≤ 1100 MHz; 7. V8; 900 MHz ≤ BWChannel_CA ≤ 1200 MHz; 8. V9; 900 MHz ≤ BWChannel_CA ≤ 1300 MHz; 9. V10; 1000 MHz ≤ BWChannel_CA ≤ 1400 MHz; 10. V11; 1100 MHz ≤ BWChannel_CA ≤ 1500 MHz; 11. V12; 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. |
| R4-2208865 | Xiaomi | LS to introduce new FR2 CA BW Classes |
| R4-2208866 | Xiaomi | Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classes Table 5.3A.4-1: CA bandwidth classes NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. A; BWChannel ≤ 400 MHz; 1; 1,2,3,4. B; 400 MHz < BWChannel_CA ≤ 800 MHz; 2; 1. C; 800 MHz < BWChannel_CA ≤ 1200 MHz; 3. Z; 1200 MHz < BWChannel_CA ≤ 1600 MHz; 4. D; 200 MHz < BWChannel_CA ≤ 400 MHz; 2; 2. E; 400 MHz < BWChannel_CA ≤ 600 MHz; 3. F; 600 MHz < BWChannel_CA ≤ 800 MHz; 4. R; 800 MHz < BWChannel_CA ≤ 1000 MHz; 5. S; 1000 MHz < BWChannel_CA ≤ 1200 MHz; 6. T; 1200 MHz < BWChannel_CA ≤ 1400 MHz; 7. U; 1400 MHz < BWChannel_CA ≤ 1600 MHz; 8. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. V3; 900 MHz ≤ BWChannel_CA ≤ 1300 MHz; 9. W3; 1000 MHz ≤ BWChannel_CA ≤ 1400 MHz; 10. X3; 1100 MHz ≤ BWChannel_CA ≤ 1500 MHz; 11. Y3; 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. O; 100 MHz ≤ BWChannel_CA ≤ 200 MHz; 2; 4. P; 150 MHz ≤ BWChannel_CA ≤ 300 MHz; 3. Q; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 4. NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A, unless otherwise specified. NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: The minimum supported component carrier bandwidth is 100 MHz and the maximum supported bandwidth is 200 MHz. The component carriers shall be arranged so all 100 MHz carriers are contiguous and all 200 MHz carriers are contiguous.. |
| R4-2209617 | ZTE Corporation | Discussion on FR2 CA BW class with aggregated BW up to 1600MHz Observation 1: To support CA BW class with aggregated BW up to 1600MHz, a tentative agreement to introduce a new class in FBG#1 with 400MHz CC’s has already been reached in the previous WF [2]. Proposal 1: To introduce a new CA BW class for aggregated BW 1200 MHz < BWChannel_CA ≤ 1600 MHz with 4CCs in FBG#1. CA BW Class; Aggregated Channel BW; Number of CC; Fallback Group. 1200 MHz < BWChannel_CA ≤ 1600 MHz; 4; 1. Observation 2: With regard to the new CA BW class supporting mix 200MHz CCs and 100MHz CCs, Option 2b/2c has more duplicated lower order classes than Option 3. However, the fallback behaviour in Option 2b/2c is the same as the legacy FBG which is better than Option 3. Proposal 2: It is suggested to introduce new CA BW class for FR2 by Option 3x or Option 3y. Option 3x: NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. V (Note 3); 900 MHz ≤ BWChannel_CA ≤ 1000 MHz; 9. W (Note 3); 1000 MHz ≤ BWChannel_CA ≤ 1200 MHz; 10. X (Note 3); 1100 MHz ≤ BWChannel_CA ≤ 1400 MHz; 11. Y (Note 3); 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A, unless otherwise specified. NOTE 2: (no change) It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: The minimum supported component carrier bandwidth is 100 MHz and the maximum supported bandwidth is 200 MHz. The component carriers shall be arranged so all 100 MHz carriers are contiguous and all 200 MHz carriers are contiguous.. In order to make the fallback more flexible up to 8 x 100MHz CCs and up to 4 x 200MHz CCs like Option 2B, we can further optimize Option 3x to Option 3y as follows, which enlarges the upper limit of aggregated BW. Option 3y: NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. V (Note 3); 900 MHz ≤ BWChannel_CA ≤ 1300 MHz; 9. W (Note 3); 1000 MHz ≤ BWChannel_CA ≤ 1400 MHz; 10. X (Note 3); 1100 MHz ≤ BWChannel_CA ≤ 1500 MHz; 11. Y (Note 3); 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A, unless otherwise specified. NOTE 2: (no change) It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: The minimum supported component carrier bandwidth is 100 MHz and the maximum supported bandwidth is 200 MHz. The component carriers shall be arranged so all 100 MHz carriers are contiguous and all 200 MHz carriers are contiguous.. Proposal 3: It is suggested to approve the following CA BW classes for FR2. Table 5.3A.4-1: CA bandwidth classes NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. A; BWChannel ≤ 400 MHz; 1; 1,2,3,4. B; 400 MHz < BWChannel_CA ≤ 800 MHz; 2; 1. C; 800 MHz < BWChannel_CA ≤ 1200 MHz; 3. Z; 1200 MHz < BWChannel_CA ≤ 1600 MHz; 4. D; 200 MHz < BWChannel_CA ≤ 400 MHz; 2; 2. E; 400 MHz < BWChannel_CA ≤ 600 MHz; 3. F; 600 MHz < BWChannel_CA ≤ 800 MHz; 4. R; 800 MHz < BWChannel_CA ≤ 1000 MHz; 5. S; 1000 MHz < BWChannel_CA ≤ 1200 MHz; 6. T; 1200 MHz < BWChannel_CA ≤ 1400 MHz; 7. U; 1400 MHz < BWChannel_CA ≤ 1600 MHz; 8. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. V3; 900 MHz ≤ BWChannel_CA ≤ 1300 MHz; 9. W3; 1000 MHz ≤ BWChannel_CA ≤ 1400 MHz; 10. X3; 1100 MHz ≤ BWChannel_CA ≤ 1500 MHz; 11. Y3; 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz; 12. O; 100 MHz ≤ BWChannel_CA ≤ 200 MHz; 2; 4. P; 150 MHz ≤ BWChannel_CA ≤ 300 MHz; 3. Q; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 4. NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A, unless otherwise specified.. NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: The minimum supported component carrier bandwidth is 100 MHz and the maximum supported bandwidth is 200 MHz. The component carriers shall be arranged so all 100 MHz carriers are contiguous and all 200 MHz carriers are contiguous.. |

Open issues summary

Sub-topic 2-1 New FR2 CA BW classes and Fallback behavior

Issue 2-1-1: How to select the baseline of new CA BW classes and fall back behaviour?

- Proposals

  - (Modified) Option 2b (Apple):

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| V2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| V3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| V4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| V5 | 500 MHz ≤ BWChannel_CA ≤ 900 MHz | 5 |  |
| V6 | 600 MHz ≤ BWChannel_CA ≤ 1000 MHz | 6 |  |
| V7 | 700 MHz ≤ BWChannel_CA ≤ 1100 MHz | 7 |  |
| V8 | 800 MHz ≤ BWChannel_CA ≤ 1200 MHz | 8 |  |
| V9 | 900 MHz ≤ BWChannel_CA ≤ 1300 MHz | 9 |  |
| V10 | 1100 MHz ≤ BWChannel_CA ≤ 1400 MHz | 10 |  |
| V11 | 1300 MHz ≤ BWChannel_CA ≤ 1500 MHz | 11 |  |
| V12 | 1500 MHz ≤ BWChannel_CA ≤ 1600 MHz | 12 |  |

  - (Modified) Option 2c: (Ericsson’s CR)  non-interlaced 100 MHz and 200 MHz.

Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. |  |  |  |

  - (Modified) Option 3 (Xiaomi’s CR): Define new FBG2 classes V, W, X and Y with associated note 3 as presented in table below.

- Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| Z | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| V3 | 900 MHz ≤ BWChannel_CA ≤ 1300 MHz | 9 |  |
| W3 | 1000 MHz ≤ BWChannel_CA ≤ 1400 MHz | 10 |  |
| X3 | 1100 MHz ≤ BWChannel_CA ≤ 1500 MHz | 11 |  |
| Y3 | 1200 MHz ≤ BWChannel_CA ≤ 1600 MHz | 12 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A, unless otherwise specified. NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: The minimum supported component carrier bandwidth is 100 MHz and the maximum supported bandwidth is 200 MHz. The component carriers shall be arranged so all 100 MHz carriers are contiguous and all 200 MHz carriers are contiguous. |  |  |  |

- Recommended WF

  - TBA

Discussion in 1st round GTW:

Agreement:

  - Take option 2c as baseline

    - For CA BW class R9, R10, R11, R12 of FBG5, the maximum aggregated BW is limited to 1600MHz in Rel-17.

      - FFS on whether and how to capture it in Rel-17 RAN4 specification, and whether to capture it in Rel-17 RAN2 specification

    - Option 2c:

  - Further refinement is needed:

    - Further discussion on proposal 4 in Issue 2-1-2.

    - Considering other issues

Issue 2-1-2: (new issue) How to refine the option 2c?

- Proposals

  - Proposal 1(Xiaomi, ZTE, Apple, ):

    - Retaining R/S/T/U in FBG#2.

Ericsson: we disagree with proposal 1. Considering signaling, we can use proposal 4.

Apple: the reason to keep RSTU is to limit the signaling overhead. Even if adding option 4, network does not know more than 4 200MHz. 1600Mhz is not the worst case. We see the signalign would be complicated.

Samsung: for proposal 1, we agree with Ericsson. Operator demand is to deploy hybrid 100 and 200 CCs. For proposal 4, we think the new IE won’t resolve the problem. 1600MHz max aggregated bandwidth is agreed to be the maximum one. We need proposal 2. Is capability of proposal 4 is mandatory or optional.The new IE is the RAN2 issue.

Qualcomm: RSTU is not necessary which is covered by 2c. To Samsung, the new IE should not be mandatory. But is not the corner case. It is aligned with the actual hardward capability. The current structure does not allow UE to report in that way.

Huawei: for proposal 4, we are against that. The current signaling is enough. The signaling is nothing to do with per band capability.

Verizon: Agree with Samsung and Qualcomm. We do not think RSTU is needed. We support Qualcomm proposal 3 and 4. They try to benefit for UE to search for combos. It is optional feature and useful.

Ericsson: We disagree with Apple that the signaling issue is due to 1600Mhz limitation as such. It is the issue if UE can support 1600MHz and can support more 200Mhz carriers. For the original problem, 2400MHz can be covered by 2c and issue is for fallback issue when UE supports more 200Mhz carriers. Regarding additional IE, the multiple feature set is already problematic for FR1.

Xiaomi: we support proposal 1. It does not mean all the UE needs to support the combiantions. RSTU should be kept in the FBG#2. If removing RSTU, it means FBG#2 is abandoned. The new UE supporting FBG#2 only supports FBG#5. There would be problem for legacy network.

ZTE: Support #1. RSTU are the previous agreement. It is better to keep FBG#2 for UE vendor not to use FBG#5. As the preiouvs agreement supporting 4 CC in FBG#1 should be considered. 400Mhz should also be considered in FBG#1.

Apple: To Ericsson, if we go back to option 2c. What information does UE need to report to network to inform the true capability, ie.., how many CCs to support 100Mhz and how many CCs to support 200Mhz?

Huawei: regarding proposal #4 we do not see the need to introduce the new signaling. The multiple feature set problems has been addressed by RAN2.

Samsung: the UE does not have two signals. UE is only to support 1600MHz for R12. UE does not need to support the 1600Mhz for all the fallbacks. We propose to first agree

Ericsson: to Xiaomi, regarding fall back group 2, it is too obselet. RSTU is not introduced in the RRC specification, it is not published. To Apple, we agree from signlaing perpest that only R12 is reported. UE has to either indicate the fall back capability or it can be done by introducing more feature sets. We were to indicate the envolope of capability of 1600MHz, then UE won’t need to report the multiple feature sets to represent multiple fallback modes. To Huawei, RAN2 has introduced the additional filtering mechanism. Network can indicate that UE can only need to report upto, e.g., 8 carriers. But it does not solve the problem here.

Qualcomm: to Apple, what is the information that UE needs to report? R12, UE needs include just one feature set. With the IE, it says 1600Mhz. To Samsung, if you want to bear maximum, it can solve the legacy problem and we have other simple solution. We would like to have future-proof solution. To Huawei, actually it is not only RAN2 problem. UE can support up to X bandwidth and Y CC number. Today signaling does not indicate such envelope.

  - Proposal 2 (Samsung, MTK):

    - For CA BW class R9, R10, R11, R12 of FBG5, the maximum aggregated BW is limited to 1600MHz in Rel-17.

  - Proposal 3(Qualcomm): Add a clarification in the notes that ‘explicit indication of support of a BW class in FBG5 does not imply that all possible variants of the lower order BW class must be supported. Only those variants of the lower order BW class that obey fallback rules as described in 38.306 must be supported’

  - Proposal 4(Qualcomm): Add an IE for the UE to explicitly indicate ‘max Agg. BW capability’ of the UE for any BW class in this FBG. This type of IE, if signaled, would also allow a flexible UE to inform the network that it can support any combination of 100M and 200M non-interlaced channels inside the indicated max. agg. BW, provided the number of CCs is not greater than those associated with the explicitly supported BW class.

  - Proposal 5(Apple):

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| V2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| V3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| V4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| V5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| V6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| V7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| V8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| V9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| V10 | 1100 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| V11 | 1300 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| V12 | 1500 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. |  |  |  |

  - Proposal 6 (Nokia): Updating NOTE 2 as follows:

    - NOTE 2:	It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group that results in a release of at least one component carrier. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.

- Recommended WF

  - Need more discussion

Status summary

Option 2c is agreed to be baseline. However, there are still many different proposals for refinements, and agreements are still not reached. Agreements for 1st round: Agreement: - Take option 2c as baseline - For CA BW class R9, R10, R11, R12 of FBG5, the maximum aggregated BW is limited to 1600MHz in Rel-17. - FFS on whether and how to capture it in Rel-17 RAN4 specification, and whether to capture it in Rel-17 RAN2 specification - Further refinement is needed: - Further discussion on proposal 4 in Issue 2-1-2. - Considering other issues Recommendations for 2nd round: Further discuss the proposals and see if an agreement of CR can be reached.

Companies views’ collection for 1st round

Open issues

Sub topic 2-1 New FR2 CA BW classes and Fallback behavior

| Issues | Company & Comments: |
| --- | --- |
| Issue 2-1-1: How to select the baseline of new CA BW classes and fall back behaviour? | Company A: |
|  | Qualcomm: On how to select: In our view, fallback characteristics and ability to streamline other FBG into one are important criteria from a functional perspective. Based on these 2 criteria option 2c appears best suited. Based on concerns expressed by various companies, we propose the following two refinements to 2c: - Add a clarification in the notes that ‘explicit indication of support of a BW class in FBG5 does not imply that all possible variants of the lower order BW class must be supported. Only those variants of the lower order BW class that obey fallback rules as described in 38.306 must be supported’ - Add an IE for the UE to explicitly indicate ‘max Agg. BW capability’ of the UE for any BW class in this FBG. This type of IE, if signaled, would also allow a flexible UE to inform the network that it can support any combination of 100M and 200M non-interlaced channels inside the indicated max. agg. BW, provided the number of CCs is not greater than those associated with the explicitly supported BW class. To further explain #2, consider a UE that can support up to 12 CC and up to 1600 MHz of contiguous BW as independent limitations. This type of UE would naturally indicate support for R12 in 2c. This max. agg. BW IE (if signaled) would convey to the network that the UE can also support 1600MHz with 11, 10, 9 and 8 CCs. Without this IE, the UE will have to burn additional band combinations to separately signal R11, R10, R9 etc in configurations that are more capable than those that are fallbacks from the original R12. |
|  | Verizon: We shared the same views from both Samsung and Qualcomm and believed that the Option 2c appears best suited for criterial fallback characteristics and ability to streamline. This proposal can also meet the Mediatek proposal 1. In R4-2208752 proposal, it provides a way to limit the Option 2c to 1600 MHz aggregated BW for Rel-17. The option can also be straightforward to remove the restriction in a later release without any signalling impact and enable 2400MHz aggregated bandwidth without further updates again as what Samsung proposed. For the Qualcomm refinement 1 to the Option 2c, it provides additional clarification of practice operations, and we share the exact same understanding for this option too. And, we are open to including this clarification into the specification. For the Qualcomm refinement 2 for Option 2c, the IE may be a new concept for companies and IE self should be an optional. We would like to see more companies’ views for consensus. |
|  | MediaTek: We are also fine to select “(Modified) Option 2c” as baseline. About the 3 points raised by couple of companies in prior meetings: - “The interlacing CC bandwidth is not allowed.” → The concept is implemented as “NOTE 1”. - “Limit the maximum aggregated bandwidth to 1600MHz.” → We realize it can be implemented by “featureSetCombination” as Proposal2 in R4-2208752. Moreover, we also would like to add “band combinations are limited to 1600 MHz aggregated channel bandwidth for Rel-17” as “NOTE3”. We think it make situation clearer, and the restriction is straightforward to be removed in a later release without signalling impact while need. - 50MHz channel bandwidth is not supported. → The “(Modified) Option2c” already implies this. |
|  | Xiaomi: The purpose of mixing 100MHz and 200mhz CCs is to solve the problem of needing too much CCs for 100MHz to 1600MHz CCs, so no matter introducing the new classes in FBG3 or new FBG need still keep the original purpose, not mix all request new classes in one FBGs. Actually, our option 3c and Apple’s modified option 2b can meet operator’s request and current fallback rules. Of course, if most companies support (modified) option 2c, as a compromise, we can accept option 2c with the precondition that keeping FBG2 could further evolve independently by retaining R, S, T, U in FBG2. For the Qualcomm refinement 2 for Option 2c, it is a good proposal to limit the cases of mixing 100MHz and 200mhz CCs, i.e., a UE indicates class R12 and max Agg. BW 1400MHz, it means the UE only support 10*100+2*200 aggregated channel bandwidth. But we have some concerns for the new IE: - Whether the new IE max Agg. BW applies to all FBGs or only to the FBG for mixing classes. - I think the IE max Agg. BW should be indicated per band combination, whether it will apply to all BCSs or just to some special BCSs. |
|  | ZTE: We support modified option 3 since it not only keeps the UE with legacy FBG#2 and FBG#3 work unchangeably but also has the benefits that the least new added BW classes are needed. We don’t think it’s appropriate to reverse the previous agreements of introducing the BW classes R/S/T/U in FBG#2. We are not sure if all the UE vendors will follow the roadmap that using mix 100MHz and 200MHz CCs in FBG#5. So from our point of view, if all other companies support option 2c, we can also compromise to modified option 2c except that we share the same view as Xiaomi to keep the classes R/S/T/U remain in FBG#2. Furthermore, if modified option 2c is chosen, we suggest to rename the classes R2/R3/…/R12 to V2/V3/…/V12. By keeping the classes R/S/T/U in FBG#2, the UE vendors which do not support mix 100MHz and 200MHz CCs can continue to stay in FBG#2. Regarding to QC’s #2 refinement, we think it is good to minimize the number of permutations supporting in each BC class if only smaller aggregated channel BW is supported. One question is that what is the range of the ‘max Agg. BW capability’? Any value between 1600 and 2400MHz? or even below than 1600MHz? Are there any lower order fallback classes not supported if the new IE is introduced? The last point we would like to emphasize is that we still suggest to support 4CCs in FBG#1 with the maximum aggregated BW up to 1600MHz which is the tentative agreement in the previous meeting. We now even support 1600MHz with so many carriers of mix 100 and 200MHz, the support of contiguous 400MHz CC is obvious. If in future, the maximum aggregated BW is extended to 2400MHz, the number of CCs should also extend to 5. |
|  | Samsung: Generally we share similar view with Qualcomm, Verizon and MediaTek on how to select the options, i.e. option 2c should be as baseline, and some refinement is needed in our view. The (Modified) Option 2c is not aligned with the agreement of last meeting, i.e. Limit the maximum aggregated bandwidth to 1600MHz. We understand the feature sets approach in R4-2208752 allows UE reporting maximum aggregated BW as 1600MHz even UE reporting CA BW class R12, however, it is RAN2 mechanism and UE can always reporting so even with legacy CA BW class. In RAN4 specification, to reflect the WID objective and previous meeting agreement, we agree with MediaTek that a note is needed for the 1600MHz restriction in Rel-17. About “The interlacing CC bandwidth is not allowed” which we also agree, we observed that it is a RAN4 restriction but RAN2 has no such restriction, it is suggested to share with RAN2 as early as possible. About the refinement 1 proposed by Qualcomm, we share the same understanding and hence support it. About the refinement 2 proposed by Qualcomm, it is newly proposed in 1st round comment and there is no sufficient discussion before. Our initial thinking is that it is RAN2 issue together with the feature set approach. Not sure if the new IE is redundant with the feature set approach, especially for FR2 where there is only one BCS compared with FR1 where there are many. |
|  | Apple: Our preference is Option 2b for the following reasons: - It fulfils the original intent of mixing FBG3 and FBG2 to support aggregated BW up to 1600 MHz. - It provides full flexibility in terms of fallback combinations with maximum of 8 100MHz CCs and 4 200MHz CCs. - For aggregated BW wider than 800 MHz, only 200MHz cell would be added. - Manageable number of CA configurations For Option 2c, though it offers even more flexibility in view of mixing 100MHz and 200MHz CCs as compared to Option 2b and may be preferred for including the full functionality of FBG2, there are also a few issues associated with Option 2c which should be carefully reviewed and considered: 1. Option 2c has a potential fallback issue if the UE total aggregated BW capability is less than the upper bound of each CA BW class. In such cases, UE would have to use FeatureSet to indicate its maximum aggregated BW capability. For example, if UE’s capability is at 1600MHz and would like to support R12, the following FeatureSet would need to be signaled: R12: 8x100+4x200 R11: 6x100+5x200 R10: 4x100+6x200 R9: 2x100 + 7x200 R8: 8x200 The signaling complexity would aggravate if UE’s total maximum aggregated BW is further reduced. For 1400MHz and 1200MHz aggregated BW limits, the following FeatureSet would need to be signaled. 1400 MHz: R12: 10x100+2x200 R11: 8x100+3x200 R10: 6x100+4x200 R9: 4x100+5x200 R8: 2x100+6x200 R7: 7x200 1200 MHz: R12: 12x100 R11: 10x100+1x200 R10: 8x100+2x200 R9: 6x100+3x200 R8: 4x100+4x200 R7: 2x100+5x200 R6: 6x200 The application of FeatureSet in our view simply defeats the purpose of CA BW classes fallback rule where UE only needs to signal the highest BW class for each fallback group. On the other hand, the UE implementation may have different maximum aggregated BW limit for single-band and FR1+FR2 CA/DC and FR2 inter-band CA. So the FeatureSet mapping could be quite messy for different combinations. 2. In the case that UE’s limit is at 1200 MHz, it would be unclear whether UE should signal R12 or R11, or even lower order BW class as its capability. If UE signals R12 which would mean the only configuration is 12x100. However, there may not be 12x100MHz network deployment at all, as network may choose to use 200MHz cell for beyond 8x100MHz. On the other hand, since UE does not have any knowledge on what the network's capability is, if UE chose to signal R12 just for the sake for not missing any potential configurations, the FeatureSet signaling overhead would be maximized. Anyway, if Option 2c is preferred by the majority companies, one way to mitigate the fallback signaling issue in FBG5 is to keep the existing classes R,S,T,U in FBG2 and for UE to signal only R12 in FBG5 (using FeatureSet with 8x100+4x200 for limit at 1600MHz) and class U (8x200). This way network would understand UE’s capability to support up to 12 CCs, and at least 8 out of 12 CCs can be 200 MHz. As a result, all the fallback classes in FBG5 does not need to be further signaled. (R4-2207680 (Observation 7)). But RAN4 companies need to agree first that when only R12 upper limit at 1600MHz is indicated, the upper limit for lower BW classes is also implied to be limited at 1600 MHz so that there is no need to explicitly signal the FeatureSet for R11, R10, R9, and R8. Or a new IE associated with maximum aggregated BW only as proposed by Qualcomm may be considered. Last but not least, if we would consider Option 2c, in addition to retaining R, S, T, U in FBG2, we think some constraint on the lower bound of certain BW classes also needs to be added to ensure no more than 9 100MHz cells would be specified and deployed. We also want to make sure from proponent companies that there would not the possibility in future to still use the existing 100MHz cells together with 200MHz cells to support up to 2400 MHz. In that case, we would have to increase the CC number in FBG5 to more than 12 which would totally defeat the merit of Option 2c. To sum up, if Option 2c is the preference of majority companies, we would like to propose the following revisions for Option 2c: NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. A; BWChannel ≤ 400 MHz; 1; 1,2,3,4,5. B; 400 MHz < BWChannel_CA ≤ 800 MHz; 2; 1. C; 800 MHz < BWChannel_CA ≤ 1200 MHz; 3. D; 200 MHz < BWChannel_CA ≤ 400 MHz; 2; 2. E; 400 MHz < BWChannel_CA ≤ 600 MHz; 3. F; 600 MHz < BWChannel_CA ≤ 800 MHz; 4. R; 800 MHz < BWChannel_CA ≤ 1000 MHz; 5. S; 1000 MHz < BWChannel_CA ≤ 1200 MHz; 6. T; 1200 MHz < BWChannel_CA ≤ 1400 MHz; 7. U; 1400 MHz < BWChannel_CA ≤ 1600 MHz; 8. G; 100 MHz < BWChannel_CA ≤ 200 MHz; 2; 3. H; 200 MHz < BWChannel_CA ≤ 300 MHz; 3. I; 300 MHz < BWChannel_CA ≤ 400 MHz; 4. J; 400 MHz < BWChannel_CA ≤ 500 MHz; 5. K; 500 MHz < BWChannel_CA ≤ 600 MHz; 6. L; 600 MHz < BWChannel_CA ≤ 700 MHz; 7. M; 700 MHz < BWChannel_CA ≤ 800 MHz; 8. O; 100 MHz ≤ BWChannel_CA ≤ 200 MHz; 2; 4. P; 150 MHz ≤ BWChannel_CA ≤ 300 MHz; 3. Q; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 4. V2; 200 MHz ≤ BWChannel_CA ≤ 400 MHz; 2; 5. V3; 300 MHz ≤ BWChannel_CA ≤ 600 MHz; 3. V4; 400 MHz ≤ BWChannel_CA ≤ 800 MHz; 4. V5; 500 MHz ≤ BWChannel_CA ≤ 1000 MHz; 5. V6; 600 MHz ≤ BWChannel_CA ≤ 1200 MHz; 6. V7; 700 MHz ≤ BWChannel_CA ≤ 1400 MHz; 7. V8; 800 MHz ≤ BWChannel_CA ≤ 1600 MHz; 8. V9; 900 MHz ≤ BWChannel_CA ≤ 1800 MHz; 9. V10; 1100 MHz ≤ BWChannel_CA ≤ 2000 MHz; 10. V11; 1300 MHz ≤ BWChannel_CA ≤ 2200 MHz; 11. V12; 1500 MHz ≤ BWChannel_CA ≤ 2400 MHz; 12. NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.. |
|  | Nokia: To complete the work in this meeting as targeted, we support using option 2c as a baseline to achieve a compromise. To ensure UE supporting, e.g., 1200 MHz aggregated channel bandwidth will not be requested to fallback to lower order CA bandwidth class configuration within a fallback group with a larger aggregated channel bandwidth, we suggest updating NOTE 2 as follows: NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group that results in a release of at least one component carrier. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. |
|  | Ericsson: (Modified) Option 2c. Other options, except Option 2b, break the fallback rules. The original problem was support of 1600 MHz aggregated bandwidth by 8 x 100 + 4 x 200 MHz, up to four 200 MHz carriers and legacy UEs could be supported. Using Option 2c, this can be supported by R12 with one featureSetCombination (FSC) and one featureSet (FS) entry indicating 8 x 100 + 4 x 200 MHz in the featureSetDownlinkPerCC (FSDpCC). If the UE does not include any fallbacks in its capability (then with the same capabilities), the network can release any SCell. Now the ‘problem’ (opportunity) is that the UE may support up to an aggregate 1600 MHz bandwidth with more than four 200 MHz CCs, which requires multiple FS in the FSC of the top-level BC included or inclusion of fallback BC as shown in the examples by Apple. This signaling can indeed become excessive if multiple FS are needed. To this end -- we are open to specify a new parameter/IE “max aggregated BW” as proposed by Qualcomm This envelope parameter/IE should be general and not only apply for FBG5, could be per band, per BC or FS, for example. To Qualcomm: we do not see the need for a clarification of the notes, fallback is as defined in 38.306 and 38.331. The new IE is a good proposal that can reduce signaling in general! To MediaTek: we would like to consider further how to best capture the agreed restriction to 1600 MHz for Rel-17, this does not necessarily have to be captured in the 38.101-2. To Apple: we do not understand Observation 5. It is agreed that indication of multiple FS, if a problem in practice. can less to excessive capability signaling. To alleviate this, we are supportive of introducing a new envelope IE proposed by Qualcomm. We are not supportive of the idea of retaining R-U for this purpose. To Nokia: fallback should be as defined in 38.306 and 38.331, we see no need to modify the note. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208866 (Xiaomi) | Ericsson: not agreed. |
|  | Company B |
| R4-2208753 （Ericsson） | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Moderator’s note: This section has been moved to Clause 2.2 under respective issues and sub-topics.

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2207660 |  | Offset and DL CA handling for DC location | Qualcomm Incorporated | Noted |  |
| R4-2207679 |  | Further views on DC location for intra-band UL CA | Apple | Noted |  |
| R4-2208385 |  | Handling of multiple DC locations for intra-band CA | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2208387 |  | [Draft] Reply on LS Reply LS on DC location for >2CC | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2208609 |  | Discussion on remaining issue on DC location | vivo | Noted |  |
| R4-2208610 |  | LS on DC location for intra-band CA | vivo | Revised | To be used for 2nd round discussion |
| R4-2209166 |  | Further study on DC location reporting | Huawei, HiSilicon | Noted |  |
| R4-2209424 |  | R17 FR2 DC reporting | OPPO | Noted |  |
| R4-2207680 |  | Further views on new FR2 CA BW classes | Apple | Noted |  |
| R4-2208314 |  | Discussion on FR2 mixed-CC CA BW class | MediaTek Beijing Inc. | Noted |  |
| R4-2208486 |  | Discussion on CA BW class of mixed 100MHz and 200MHz CCs | Samsung | Noted |  |
| R4-2208752 |  | Draft LS to RAN2 on FR2 bandwidth classes covering up to 2400 MHz aggregated bandwidth with mixed carrier bandwidths | Ericsson, Verizon | Noted |  |
| R4-2208753 |  | FR2 CA BW classes up to 2400 MHz aggregated BW with mixed channel bandwidths | Ericsson, Verizon | Revised | Title may also need revision (“up to 2400 MHz” may be removed). To be used for 2nd round discussion |
| R4-2208864 |  | Discussion on FR2 new CA BW classes | Xiaomi | Noted |  |
| R4-2208865 |  | LS to introduce new FR2 CA BW Classes | Xiaomi | Revised | To be used for 2nd round discussion |
| R4-2208866 |  | Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classes | Xiaomi | Noted |  |
| R4-2209617 |  | Discussion on FR2 CA BW class with aggregated BW up to 1600MHz | ZTE Corporation | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Ville Vintola | vvintola@qti.qualcomm.com |
| Xiaomi | Juan Zhang | zhangjuan8@xiaomi.com |
| Apple | James Wang | fucheng_wang@apple.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)