3GPP TSG-RAN WG4 Meeting # 103-e												R4-2210267

Electronic Meeting, May 09 – 20, 2022

Agenda item:			9.15.3

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [102-e][132] NR_ext_to_71GHz_Part_2

Document for:	Information

# Introduction

Scope is UE RX and TX requirements for 60 GHz.

# Topic #1: UE TX and RX

Main technical topic overview.

## Companies’ contributions summary TX

| T-doc number | title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2207696 | On remaining Tx issues for band n263 | Apple | Apple considers these items concluded - Min peak EIRP and spherical coverage percentile for band n263 (Tables 6.2.1.3-1, 6.2.1.3-3) can be finalized based on the agreements reached during RAN4 #102 - Max output power for band n263 (Table 6.2.1.3-2) can be based on agreements in R4-2114989 - Default NS_200 applicability can be extended to band n263 (Table 6.2.3.1-2) - Pumax tolerance for band n263 can be reused from FR2-1 (Table 6.2.4-1) - The existing Tx ON/OFF time mask requirements can be applied for both FR2-1 and FR2-2, based on lack of consensus to converge on faster periods applicable only to FR2-2 (Clause 6.3.3.1) - Occupied channel bandwidth for FR2-2 (Table 6.5.1-1) can be defined based on agreements in R4-2120062 - SEM for CWs 800, 1600, 2000 MHz (Table 6.5.2.1-1) can be defined based on agreement in R4-2120062 - ACLR for FR2-2 (Table 6.5.2.3-2) can be defined based on agreement in R4-2202367 For discussion this meeting - UE multi-band relaxation factors (left as TBD in the draft CR) in Table 6.2.1.3-4 - Minimum output power for PC3 set to TBD, with measurement BW set per agreement on spectrum utilization from RAN4 #102 (Table 6.3.1.2-1) - Tx OFF power for PC3 set to TBD, with measurement BW set per agreement on spectrum utilization from RAN4 #102 (Table 6.3.2-2) - Placeholder for spurious emission band UE co-existence requirements (Table 6.5.3.1-1) MPR and A-MPR should be defined based on the available data in the RAN4 #103 meeting. Unfortunately, our own simulation results are not available by the time of submitting this contribution. Regarding CA within FR2-2 and UL MIMO, we have not found agreements on the related requirements. Thus, in the interest of making progress, we recommend focusing the Rel-17 requirements for band n263 to cover just single carrier operation without UL MIMO. - Add the following note related to CA: "carrier aggregation within FR2 is defined only within FR2-1 in this release of the specification"; this note can be removed/updated if an agreement is reached on CA within FR2-2 during the RAN4 #103 meeting - Add the following note related to UL MIMO: "UL MIMO for FR2 is defined only for FR2-1 in this release of the specification"; this note can be removed/updated if an agreement is reached on UL MIMO within FR2-2 during the RAN4 #103 meeting Our understanding is that once operator interest in CA and UL MIMO within FR2-2 increases, then it should be staightforward for RAN to approve a work item to enable these features for FR2-2 and, consequently, to include the various CA and UL MIMO configurations with band n263 to the specification via the basket work item approach. |
| R4-2208049 | UE Tx requirements for FR2-2 - remaining issues | Intel Corporation | Observation 1: Given the current power class framework, notes can be added to the maximum output power limits tables to include any additional regulatory information. Though more involved, alternatively we can revise the power class framework for FR2-2 to ensure all relevant content is included. Observation 2: Power classes should be cognizant of relevant regional regulations. To ensure this is the case, we can further discuss two approaches: defining a dedicated power class considering regional limits, or NS value implementation. Proposal 1: To ensure all relevant regional limits are captured in the specifications, RAN4 should consider the following approaches: defining a dedicated power class or NS implementation. ON/ON transient period Observation 3: - Option 1: No gNB scheduling optimizations for ON/ON transient period - Using 5 µS ON/ON transient period leads to high throughput reduction due to corruption of the PUSCH data symbols. Up to 50% and 12% throughput loss can be expected for bundling size 2 and 8, respectively. - An improved ON/ON transient period faster than 5 µS is required to support at least full MCS for 16 QAM modulation. - Option 2: Optimized gNB scheduling for ON/ON transient period - Using 5 µS ON/ON transient period leads to high throughput loss even with optimized gNB scheduling without corrupted symbols on UE side. Up to 25% and 6% throughput loss can be expected for bundling size 2 and 8, respectively. An improved ON/ON transient period faster than 5 µS allows better throughput performance with almost 20%, 10% and 5% improvement for scenarios with bundling size 2, 4 and 8, respectively. Proposal 2: Introduce 2 µS improved ON/ON transient period as optional UE capabilities for 480 and 960 kHz SCS. Other requirements Observation 4: Multi-band support including FR2-2 presents additional challenges given the higher frequency, broader frequency range and additional design complexity. Therefore, the multi-band relaxation factors (∆MBP,n and ∆MBS,n) of band n263 should be larger than those defined for FR2-1 bands. Observation 5: Based on achieved progress in this meeting, RAN4 should agree on the remaining issues to be addressed in Rel-18 during the maintenance stage. |
| R4-2208619 | Discussion on remaining Tx RF requirements for FR2-2 | vivo | Proposal Power Class (UE type); Min peak EIRP (dBm); Max TRP (dBm); Max EIRP (dBm); Spherical Coverage. PC1 (FWA); 30.6; 27; 43; 19.1 dBm min EIRP at 85%-tile CDF; 11.5 dB Gain drop, 85%ile. PC2 （Vehicular）; 22.7; 27; 43; 7.6 dBm min EIRP at 60%-tile CDF; 15.1 dB Gain drop, 60%ile. PC3 (Handheld); 14.1; 27; 43; 2.3 dBm min EIRP at 50%-tile CDF; 11.8 dB Gain drop, 50%ile. Proposed OOB emission Channel bandwidth / NRACLR / Measurement bandwidth. 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. 15 dB; 15 dB; 15 dB; 15 dB; 15 dB. 95.16; 380.28; 760.32; 1520.64; 1797.12. +100 / -100; +400 / -400; +800 / -800; +1600 / -1600; +2000 / -2000. Proposed SEM table 3 per -5/-13 method |
| R4-2208647 | Discussion on open specification items for Tx RF requirements | LG Electronics Finland | - TRP Proposal 1: Define following UE maximum output power limits for n263. - PC1 - Max TRP = 25dBm & Max EIRP = 55dBm for fixed outdoor installations with ≥ 30 dB transmit directivity - Max TRP = 25dBm & Max EIRP = 40dBm in other cases - PC2 - Max TRP = 25dBm & Max EIRP = 40dBm - PC3 - Max TRP = 25dBm & Max EIRP = 40dBm - MPR Proposal 2: Apply FR2-2 PC3 MPR to PC2. Proposal 3: For FR2-2 PC3 MPR, - For CBW of 100MHz and 400MHz - Reuse FR2-1 PC3 MPR for FR2-2 PC3 MPR - For CBW of 800MHz, 1600MHz and 2000MHz - Consider 3dB(Y1), 4dB(Y2) and 4dB(Y3) as MPR delta (in Table 2.3) respectively. - For Edge RB allocations, in case of Pi/2 BPSK and QPSK in DFT-s-OFDM, consider 4dB, 5dB and 5dB respectively - MPR for CA Proposal 4: Apply FR2-2 PC3 MPR CA to PC2. - Minimum Pout Proposal 5: Consider -13dBm as starting point for discussing minimum Pout for FR2-2 PC2/PC3. - Minimum Pout for CA Proposal 6: Consider -13dBm as starting point for discussing CA minimum Pout for FR2-2 PC2/PC3. - Carrier leakage & in-band emission (I/Q image) Proposal 7: Define carrier leakage and I/Q image as shown in Table 2.6 for FR2-2 PC2 and PC3. - Carrier leakage & in-band emission (I/Q image) for CA Proposal 8: Define carrier leakage and I/Q image for CA as shown in Table 2.6 for FR2-2 PC2 and PC3. - Beam correspondence Proposal 9: If beam correspondence requirements for FR2-2 PC3 are introduced, then reuse the FR2-1 PC3 beam correspondence procedure for FR2-2 PC3. For beam correspondence tolerance, further discussion may be necessary. Proposal 10: For FR2-2 PC2 beam correspondence use the same approach as for FR2-1 PC2 beam correspondence |
| R4-2208759 | On n263 associated band specific Tx requirements | Huawei, HiSilicon | Proposal 1: Capture RF requirements for band n263 in new sub-clauses with suffix F. Proposal 2: For band n263 handheld UE, the maximum peak EIRP is 43dBm, the Maximum TRP is 23dBm. Proposal 3: For band n263 UE of PC1 and PC2, existing FR2-1 requirements for maximum peak EIRP and maximum TRP could be reused. Proposal 4: For band n263 the multi-band relaxation is 1.0dB for both MBP,n (dB) and MBS,n (dB). Proposal 5: The Pumax tolerance of band n263 could reuse existing requirements for FR2-1 operating bands. Proposal 6: The minimum output power of band n263 could reuse existing requirements for FR2-1 operating bands. The measurement bandwidth is selected as the maximum required MBW among all applicable SCS, i.e. the highlighted ones in Table 2.4-2. Proposal 7: The transmit OFF power of band n263 could reuse existing requirements for FR2-1 operating bands. The measurement bandwidth is the same as minimum output power. Proposal 8: For unlicensed band n263, the transient period locates in both ON slot and OFF slots. Proposal 9: Only specify the PRACH time mask requirements for 120kHz SCS. Proposal 10: The ACLR of band n263 is 15dB. The measurement bandwidth is the same as minimum output power. Proposal 11: Waive the test of ACLR for band n263. Proposal 12: For band n263, the FR2-1 Tx requirements that are not band specific could be reused. |
| R4-2209322 | Multi-band relaxation requirement for band n263 | Apple | Observation 1: Band n263 antenna elements are expected to be highly susceptible to distortions from asymmetries in the module, preventing effective beamforming. Observation 2: mm-Wave modules are required to use a high aspect ratio in order to avoid excessive growth in product thickness. This creates significant limitations for multi-band designs. Observation 3: Large gain swings due to the lensing from electrically thick layers, which is expected to be even more pronounced in co-located configurations with FR2-1 antennas. Observation 4: The impairments coming from the lensing and cavity effect cause pattern distortion resulting in element gain degradation, which needs to be considered for the derivation of MBR. Proposal 1: Introduce the multi-band relaxation requirement for band n263 as provided in Table 1. Table 1: UE multi-band relaxation factors for power class 3 Band; MBP,n (dB); MBS,n (dB). n257; 0.73; 0.73. n258; 0.6; 0.7. n259; 0.5; 0.4. n260; 0.51; 0.41. n261; 0.52,4; 0.74. n262; 0.7; 0.7. n263; 1.0; 1.0. Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257. |
| R4-2209507 | on the UE TX requirement | Xiaomi | Proposal 1: It is proposed PC3 50%ile spherical coverage drop as 12 dB. Proposal 2: For faster capability of ON/ON transient period for 480 and 960 kHz SCS, it is proposed to further discuss in Rel-18. Proposal 3: Put the max PSD requirement in the NS value for EU regulation. |
| R4-2209716 | UE features for NR in 52.6 GHz - 71 GHz | Nokia, Nokia Shanghai Bell | Proposal 1: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required. Proposal 2: For optional ON-ON transient time, only one value among 1 us or 2 us is specified. |
| R4-2210166 revised to R4-2210433 | 60 GHz UE TX | Qualcomm Incorporated | Proposal: Use the NRB table values for FR2-2 transmission bandwidth requirements, with [] removed from 156. Proposal: Use the table in 38.101-2 to specify the FR2-2 TX OFF power. Note the value is -35 dBm in the measurement bandwidth. Proposal: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown. Proposal: Use the same FR2-1 absolute power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions. Proposal: Use the same FR2-1 relative power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions. Proposal: Use the same FR2-1 aggregate power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions. Proposal: Use the FR2-1 EVM percentages for n263. Proposal: Scale the minimum EVM power level from 400 MHz for noise BW. 3 dB for 800 MHz, 6 dB for 1600 MHz, 7 dB for 2000 MHz. Proposal: RAN4 to further discuss how to capture PTRS processing in EVM requirement. Proposal: Use the values in the table above for n263 PC1 relative carrier leakage requirements. Proposal: Use the values in the table above for n263 PC2 relative carrier leakage requirements. Proposal: Use the values in the table above for n263 PC3 relative carrier leakage requirements. Inband emissions proposals: - Use the same general in-band emissions limit as FR2-1 - Modify the carrier leakage power limits, using the limits shown in the previous section tables for PC1-3 - Modify the IQ image values, adding 10 dB to the carrier leakage power limits. This is the same method used in FR2-1. Proposal: Use the modified SEM table for the FR2-2 general SEM requirement. Proposal: For n263 use 2x the CCBW as the spurious domain, as in FR2-1. Proposal: For n263 spurious emissions use the same limits as FR2-1 Proposal: All n263 UEs shall support beamCorrespondenceWithoutUL-BeamSweeping. PC1 SEM Observation: Only 100 MHz is SEM-limited, with some backoff required for higher modulation orders. PC1 EVM Observation: Backoff for EVM is modulation order dependent, with no sensitivity to SCS or CCBW. PC3 SEM Observation: Only 100 MHz is SEM-limited, with some backoff required for higher modulation orders. This is similar to PC1 SEM. PC3 EVM Observation: Backoff for EVM is primarily modulation order dependent. There is also some dependence on CCBW as 400 and 800M QPSK require more backoff than 100M. We can’t determine yet if there is a dependence on SCS for QPSK. Proposal: The transient period from FR2-1 is based on the capability of the UE to configure the transmitter and receiver. The same capability will exist in FR2-2. Use the same 5usec for FR2-2. Proposal: UE requires 200 nsec for beam direction only switching for all SCS Observation: Our understanding is the UE can perform a beam direction change, or a power control change, or both during this 200 nsec time. |
| R4-2208226 | Discussion on FFT sizes for 52 6-71GHz | CATT | Proposal 1: To adopt the FFT sizes for FR2-2 in the following table 2-1. Table 2-1: FFT sizes for FR2-2 SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. FFT size; FFT size; FFT size; FFT size; FFT size. 120; 1024; 4096; N/A; N/A; N/A. 480; N/A; 1024; 2048; 4096; N/A. 960; N/A; 512; 1024; 2048; 4096. |
| R4-2208754 | Upper limit on configured maximum power for 57-71 GHz | Ericsson | Proposal 1: the upper limit of the power class for a UE form factor should not be determined by regulations of a particular region (like Europe) but be indicated by an NS value indicating the maximum output power applicable along with gain, directivity and unwanted emissions limits as required by local regulation. |
| R4-2209717 revised to R4-2210350 | On UE Tx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Proposal 1: Confirm 156 RBs for 2GHz CBW with 960 kHz SCS. Proposal 2: Use a UE beam direction switching time of 59 ns. Proposal 3: Based on MPR simulations, adopt the same MPR for FR2-2 as defined for FR2-1 for PC3 UEs. |
| Draft CRs |  |  |  |
| R4-2207697 | Draft CR to 38.101-2 on band n263 Tx aspects | Apple |  |
| R4-2208760 | Draft CR for n263 RF Tx requirements | Huawei, HiSilicon |  |
| R4-2208781 | draft CR on open specification for PC2 in FR2-2 | LG Electronics Finland |  |
| R4-2209510 | draftCR on the UE TX requirement for band n263 | Xiaomi |  |

## Companies’ contributions summary RX

| T-doc number | title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2208620 | Discussion on remaining Rx RF requirements for FR2-2 | vivo | This contribution further discusses UE Rx RF requirements for FR2-2. The REFSENS and ACS requirements are proposed in this contribution. |
| R4-2208761 | On n263 associated band specific Rx requirements | Huawei, HiSilicon | Proposal 1: Capture RF requirements for band n263 in new sub-clauses with suffix F. Proposal 2: The ACS of band n263 is 21dB. The measurement bandwidth is the same as minimum output power. Proposal 3: The IBB of band n263 is specified for all supported bandwidths. Proposal 4: The IBB requirement of band n263 is the same as ACS requirement Proposal 5: For band n263, the FR2-1 Rx requirements that are not band specific could be reused. |
| R4-2209323 | Rx requirement for band n263 | Apple | Proposal 1: Define ACS requirement for band n263 as provided in Table 2, Table 3, and Table 4. Proposal 2: Define In-band Blocking requirement for band n263 as provided in Table 5. |
| R4-2209508 | on the UE RX requirement | Xiaomi | Proposal 1: It is proposed the above REFSENS and EIS requirement corresponding to the table 1 above. Proposal 2: It is proposed the above table 2 for other RX requirement. |
| R4-2209718 | On UE Rx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Proposal 1: Remaining FR2-2 Rx requirements shall adopt FR2-1 requirements. |
| Draft CRs |  |  |  |
| R4-2208762 | Draft CR for n263 RF Rx requirements | Huawei, HiSilicon |  |
| R4-2209511 | draftCR on the UE RX requirement for band n263 | Xiaomi |  |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Transmit

#### Output RF spectrum emissions

##### SEM

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Proposed SEM table 3 per -5/-13 method in 0166 section 10

Table 6.5.2.1-1: General NR spectrum emission mask for frequency range 2.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | -5 | -5 | -5 | 1 MHz |
| 40-80 | -13 | -13 | -13 | -13 | -5 | -5 | -5 | 1 MHz |
| 80-100 | -13 | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 100-160 |  | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 160-200 |  | -13 | -13 | -13 | -13 | -13 | -5 | 1 MHz |
| 200-400 |  |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | -13 | -13 | -13 | 1 MHz |
| 800-1600 |  |  |  |  | -13 | -13 | -13 | 1 MHz |
| 1600-3200 |  |  |  |  |  | -13 | -13 | 1 MHz |
| 3200-4000 |  |  |  |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |  |  |  |

Discussion:

Apple: during RAN4#101 there is a table agreed as starting point. People proposed to follow -5/-13 manner. What is the reason for the additional change?

Qualcomm: I believe Apple referred to initial table with -5 and -13. The breaking point was not correct in the previous table. This table follows the same methodology.

Nokia: looking at 100MHz, -13dB range should be extended.

Huawei: we have no concern on power level. Could we revisit the channel bandwidth? Considering SEM is with TRP requirement, the measurement time is long. Could we reduce the measurement time, e.g., by change measurement bandwidth to 10MHz and increasing power level by 10dB.

ZTE: whether to increase the measurement bandwidths reply on the regulation. I wonder if there is any regulation to allow 10MHz measurement.

Agreement: agree on the following table for SEM.

Table 6.5.2.1-1: General NR spectrum emission mask for frequency range 2.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | -5 | -5 | -5 | 1 MHz |
| 40-80 | -13 | -13 | -13 | -13 | -5 | -5 | -5 | 1 MHz |
| 80-100 | -13 | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 100-160 |  | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 160-200 |  | -13 | -13 | -13 | -13 | -13 | -5 | 1 MHz |
| 200-400 |  |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | -13 | -13 | -13 | 1 MHz |
| 800-1600 |  |  |  |  | -13 | -13 | -13 | 1 MHz |
| 1600-3200 |  |  |  |  |  | -13 | -13 | 1 MHz |
| 3200-4000 |  |  |  |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |  |  |  |

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| QCOM | Agree WF, proposal 1 |
| Xiaomi | Agree with the WF. |
| Skyworks | It would be good to adopt the equation-based approach used in FR1: Spectrum emission limit (dBm) / Channel bandwidth. ΔfOOB (MHz); 50, 100, 200, 400, 800, 1800, 2000 MHz; Measurement bandwidth. 0-0.1*CBW; -5; 1 MHz. 0.1*CBW to 2*CBW or -0.1*CBW to -2*CBW; -13; 1 MHz. |
| Nokia, Nokia Shanghai Bell | We are ok with both the GtW outcome or the Skyworks proposal. |
| LGE | We are OK with GTW outcome and also the Skyworks proposal above |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: No further discussion. GTW agreement was made May 10 |

##### Spurious emissions

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: For n263 use 2x the CCBW as the spurious domain, as in FR2-1.

- Table 6.5.3-1: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 | 1600 | 3200 | 4000 |

  - Proposal 2: For n263 spurious emissions use the same limits as FR2-1

Table 6.5.3-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 1 MHz |

- Recommended WF

  - Agree both proposal 1 and proposal 2

Discussion

Agreement: agree both proposal 1 and proposal 2.

| Company | Comments |
| --- | --- |
| QCOM | Agree WF, proposals 1 and 2 |
| Xiaomi | Agree with the WF. |
| MediaTek | We do not have a 200MHz channel bandwidth, that should be removed. Moderator: The table is combined for all of FR2. that is why 50 and 200 MHz are included |
| LGE | We agree the WF |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: No further discussion. GTW agreement was made May 10 |

##### Spurious coexistence

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: Placeholder for spurious emission band UE co-existence requirements (Table 6.5.3.1-1)

  - Option 2: something else

- Recommended WF

  - Further discuss in round 1

Discussions:

Nokia: would it be acceptable to have the same level as for FR2-1, i.e., -2 and -5dB.

Qualcomm: we want to check it.

Agreement: Baseline is to reuse the same level as for FR2-1. Further checking is needed in this meeting.

| Company | Comments |
| --- | --- |
| QCOM | We would like to get views from more companies on whether we need to define coex and how we arrive at bands and levels |
| Skyworks | One aspect that needs to be settled is how to treat harmonic interference or ecven harmonic mixing of FR2-1 into FR2-2 and whether this needs to be considered for coex and MSD or if beamforming is enough to ignore the issue (harmonics are beamformed but not necessarily in the direction of the fundamental) |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss in WF on 60 GHz UE Requirements |

#### Transmit signal quality

##### EVM

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Use the FR2-1 EVM percentages for n263.

  - Proposal 2: Scale the minimum EVM power level from 400 MHz for noise BW. 3 dB for 800 MHz, 6 dB for 1600 MHz, 7 dB for 2000 MHz.

  - Proposal 3: RAN4 to further discuss how to capture PTRS processing in EVM requirement.

- Recommended WF

  - Agree Proposals 1,2 and 3.

Discussion:

Apple: We would like to share our view. The phase noise is too large. We need consider all the applicable gNB Tx and UE Rx impairment.

Qualcomm: to proposal 2, the proposal is to agree the scaling. We need to work on what the minimum levels are. We agree that phase noise is important for higher modulation orders.

Nokia: we agree that 64QAM is optional for UE.

Agreement:

- EVM requirement:

  - Pi/2 BPSK: 30%

  - QPSK: 17.5%

  - FFS for 16QAM and 64QAM

- Scale the minimum EVM power level from 400 MHz for noise BW

  - FFS on the values.

- FFS whether PTRS is configured or not

| Company | Comments |
| --- | --- |
| QCOM | Agree with the WF. For proposal 3 companies should consider the details of the EVM processing and how to add the PTRS processing part in the specification. |
| Skyworks | Agree with proposal 1 and 2 |
| Nokia, Nokia Shanghai Bell | After the GtW discussion, we are ok with enabling PT-RS for UE EVM test and re-using the same EVM-% values as in FR2-1. |
| LGE | We are OK with Nokia comment above. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss the FFS items in WF on 60 GHz UE Requirements |

##### Carrier leakage, image, and inband emissions

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Define carrier leakage and I/Q image as shown in Table 2.6 for FR2-2 PC2 and PC3.

| FR2-2 PC3 | IQ Image | dB | -25 | Output power > 10 dBm |
| --- | --- | --- | --- | --- |
|  |  |  | -20 | Output power ≤ 10 dBm |
|  | Carrier leakage | dBc | -25 | Output power > 0 dBm |
|  |  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |
| FR2-2 PC2 | IQ Image | dB | -25 | Output power > 19 dBm |
|  |  |  | -20 | Output power ≤ 19 dBm |
|  | Carrier leakage | dBc | -25 | Output power > 9 dBm |
|  |  |  | -20 | -13 dBm ≤ Output power ≤ 9 dBm |

  - Proposal 2: Define carrier leakage and I/Q image for CA as shown in Table 2.6 for FR2-2 PC2 and PC3.

  - Proposal 3: Use values in table (R4-2210166) for PC1-PC3 relative carrier leakage requirement.

  - Proposal 4: Inband emissions proposal from (R4-2210166)

- Recommended WF

  - WF1: Agree on -25/-20 limits as in FR2-1 for PC1, PC2, and PC3

  - WF2: PC2 and PC3 Agree output power levels from proposal 1

  - WF3:  PC1 further discuss output power levels for the -25 range and the -20 range

Agreement:

- Agree on -25/-20 limits as in FR2-1 for PC1, PC2, and PC3

- For PC2 and PC3, Agree output power levels from proposal 1

- PC1 further discuss output power levels for the -25 range and the -20 range

| Company | Comments |
| --- | --- |
| QCOM | Agree with the three WFs: WF1, WF2, and WF3 |
| Skyworks | 20dBc image level does not seem compatible with 64QAM 8% EVM requirement and may already be a large contributor for 16QAM. Should there be different levels vs modulation or at least some note? |
| Nokia, Nokia Shanghai Bell | We are supportive towards the Skyworks suggestion to add a note to ensure that IQ-image does not block gains from supporting (optional) 64QAM. |
| LGE | The dBc values are reused from FR2-1. With large IQ-impairments (not removed in EVM test) the device will not pass the EVM requirements. LO is removed from EVM measurement, but image is not ? “The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM, the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.” |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss PC1 output power levels for the -25 -20 range in WF on 60 GHz UE Requirements |

#### UE Maximum output power

##### Power class upper limit and regions

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: To ensure all relevant regional limits are captured in the specifications, RAN4 should consider the following approaches: defining a dedicated power class or NS implementation. (Intel 8049)

  - Option 2: the upper limit of the power class for a UE form factor should not be determined by regulations of a particular region (like Europe) but be indicated by an NS value indicating the maximum output power applicable along with gain, directivity and unwanted emissions limits as required by local regulation.

- Recommended WF

  - TBA

Discussion:

Apple: We agree to introduce the NS value.

Intel: we are fine with this. Is the idea that it only applies to FR2-2?

Ericsson: yes. It includes the parameters in the region. We also have spurious emission. The NS indicates all the requirements applicable for FR2-2 bands.

Xiaomi: we agree with NS value. For option 2, during the discussion directivity should not be included.

Ericsson: Directivity is regional regulation. It applies when only the regulation is applicable.

Xiaomi: how the requirements can be included in 3GPP.

Ericsson: it may not be specified in 3GPP.

Intel: no performance requirements need be defined for regulation.

Apple: do you have the concrete the NS proposal.

Ericsson: 43 is for Europe.

Intel: what is the difference from FR2-1.

Ericsson: For FR2-1 we have the fixed upper limit. For the lower limit, the same applies for all the frequency ranges.

Xiaomi: we can keep the framework of power class and use NS to capture the additional requirements.

Ericsson: For FR2-2 power classes, the upper limit of power should be indicated by NS value.

Agreement:

- For the FR2-2 existing power classes, the upper limit of the power class for a UE form factor should be indicated by an NS value indicating the maximum output power applicable along with other requirements applicable in the local regulation

  - No additional performance verification is needed in 3GPP for the other requirements, which are not specified in 3GPP and applicable in the local regulation.

| Company | Comments |
| --- | --- |
| Xiaomi | It seems option 1 and option 2 are partially overlapped. We agree to use NS value to specify the regional requirement. |
| Skyworks | NS and A-MPR may be the best approach |
| LGE | We are OK with GTW agreement = UE is informed on presence of certain regulative requirements with NS, but we do not see a need for A-MPR (=new requirement to be verified). |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: GTW agreement May 10. No further discussion. |

##### Max PSD and NS for EU

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Put the max PSD requirement in the NS value for EU regulation.

  - Option 2:

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Xiaomi | Agree with proposal 1 as we proposed in our draftCR as R4-2209510. The regional requirement should be captured in the specification by NS values. |
| Skyworks | Option 1 is consistent with the NR-U approach but will require A-MPR to be derived |
| Nokia, Nokia Shanghai Bell | We support proposal 1. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Confirm Proposal 1 in WF on 60 GHz UE Requirements |

##### TRP and Max EIRP

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

|  | PC1 |  | PC2 |  | PC3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max TRP | Max EIRP | Max TRP | Max EIRP | Max TRP | Max EIRP |
| Proposal 1 | 27 | 43 | 27 | 43 | 27 | 43 |
| Proposal 2 | 25 | 55 | 25 | 40 | 25 | 40 |

Discussions:

Intel: we do not have discuss, since we agree on the previous NS value. The definitions of 43dBm and 40dBm are different. 40dBm is referred to max average EIRP.

Nokia: we need some values if no NS value.

Agreement: The regulation about TRP and maximum EIPR needs be captured by using NS value.

- Recommended WF

  - Further discuss proposals

| Company | Comments |
| --- | --- |
| Company A |  |
| Company B |  |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: In WF on 60 GHz UE Requirements, Record the agreement and also Further discuss values |

##### MPR

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals related to method

  - Proposal 1: Use MPR delta due to excess BW method as described in LGE (8647 proposal 3)

- Proposals PC3 MPR for 100MHz and 400 MHz

  - Proposal  2: Re-use FR2-1 PC3 MPR for FR2-2

- Proposals PC3 MPR for 800, 1600, 2000 MHz

  - Proposal  3: Consider 3dB(Y1), 4dB(Y2) and 4dB(Y3) as MPR delta (in Table 2.3) respectively. For Edge RB allocations, in case of Pi/2 BPSK and QPSK in DFT-s-OFDM, consider 4dB, 5dB and 5dB respectively

- Proposals PC2

  - Proposal 4: Apply CA FR2-2 PC3 MPR to PC2

- Recommended WF

  - Further discuss in round 1

Discussion:

Nokia: we propose the MPR data recently. For PC3, we can use the same definition for FR2-1. We can reuse values for 100MHz and extend the 400 values for other channel bandwidths.

Qualcomm: we do agreement SU, EVM.. those limitation factors for MPR. We provide some initial evaluations. We need more discussion.

LGE: we take into account the bandwidth. The larger bandwidth leads to some complexity. We are surprised to see the lower MPR values from Qualcomm and Nokia. MPR is driven by EVM and in-band emission requirements. We should agree on them in a package.

| Company | Comments |
| --- | --- |
| QCOM | Proposals 1 ,2, and 3: What about 480 and 960 kHz SCS which were not in FR2-1? Not sure we understand the thinking about the new SCS. Proposal 4: Premature. We don’t have CA results yet. |
| Nokia | We support proposal 2. Based on simulation results no further excess MPR is needed compared to FR2-1. The same values for inner/edge MPR should be defined for FR2-2 as used in FR2-1. MPR defined for 400 MHz ChBW in FR2-1 shall be extended to apply to 400 MHz and wider ChBWs in FR2-2. |
| LGE | Support reuse of PC3 MPR also for PC2. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: In WF on 60 GHz UE Requirements further discuss the proposals |

##### A-MPR

Sub-topic description: This section was intended to cover the EU mask, but no proposals were made this meeting so we will have to discuss in future meeting.

##### Configured transmitted power

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

  - Proposal 1: The Pumax tolerance of band n263 could reuse existing requirements for FR2-1 operating bands.

- Table 6.2.4-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n259, n260, n261, n262 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3.1 |  |  |

- Recommended WF

  - Further discuss in round 1

Discussion:

Ericsson: it means the actual MPR in the test would significantly increase. We should be aware of the implementation.

| Company | Comments |
| --- | --- |
| QCOM | Further discussion needed to determine if re-use is appropriate. Can the proponents describe why re-use is technically appropriate? |
| Company B |  |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss the proposal in WF on 60 GHz UE Requirements |

#### Output power dynamics

##### Minimum output power

Sub-topic description: How to handle min output power in the spec

Open issues and candidate options before e-meeting:

|  | Min pwr | Meas BW |
| --- | --- | --- |
| Prop 1 | TBD | from SU |
| Prop 2 | -13 dBm ‘starting point’ for PC2 and PC3 |  |
| Prop 3 | PC1 +4; PC2 PC3: -13 | SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 95.16; 380.28; N/A; N/A; N/A. 480; N/A; 380.64; 760.8; 1521.12; N/A. 960; N/A; 381.12; 761.28; 1521.6; [1798.08]. |

- Recommended WF

  - Further discuss min output power for PC1, PC2, and PC3 and the measurement BWs

| Company | Comments |
| --- | --- |
| QCOM | We would like to further discuss these values |
| Skyworks | This should derive directly from SU agreements |
| LGE | We propose to reuse the FR2-1 values. However, as shown in earlier system simulations the need for low UE output power levels is low. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss in WF on 60 GHz UE Requirements |

##### TX off power

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

| Proposal 1 | TBD with measurement BW per SU agreement |
| --- | --- |
| Proposal 2 | 100 and 400 MHz: -35 dBm with measurement BW same as min power 800 – 2000 MHz: FFS |
| Proposal 3 | Operating band; 50 MHz; 100 MHz; 200 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. n257, n258, n259, n260, n261, n262; -35; -35; -35; -35; N/A; N/A; N/A. 47.58MHz; 95.16 MHz; 190.20 MHz; 380.28 MHz; N/A; N/A; N/A. n263; N/A; -35; N/A; -35; -35; -35; -35. N/A; 95.16 MHz; N/A; 380.28 MHz; 760.32 MHz; 1520.64 MHz; 1797.12 MHz. |

Discussions:

Huawei: we are fine with the power level, but we need discuss the measurement bandwidth.

Nokia: agree to align the measurement channel bandwidth between Tx off power and SU.

ZTE: the SU is still under discussion. The measurement bandwidth should be in []

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| QCOM | We prefer proposal 3. |
| Nokia | We agree with proposal 3 but in case SU changes the measurement bandwidth needs to be aligned with the maximum transmission bandwidth configuration. |
| MediaTek | We do not have a 200MHz CBW. Why is that there? Moderator: The table applies to FR2, so FR2-1 is included. |
| LGE | We are OK with proposal 3. |

5

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: In WF on 60 GHz UE Requirements Confirm proposal 3 with the modification using BW from SU. |

##### Power control and tolerances

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1 : Use same as FR2-1 for absolute, relative, and aggregate power tolerances agreeing that more exceptions may be allowed for FR2-2

- Recommended WF

  - Further discuss

Discussions:

Nokia: what is the motivation to further relaxation?

Qualcomm: we expect variability of control. There would be uncertainty.

Ericsson: the current requirement for FR2-1 is not testable. 1dB step requirements can help to get the meaningful test.

| Company | Comments |
| --- | --- |
| QCOM | We are OK with proposal 1, and the number of exceptions should be FFS. |
| Nokia | We are OK with re-using same power tolerances, but we do not see a need for additional exceptions. |
| LGE | We are OK with proposal 1. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss proposal 1 in WF on 60 GHz UE Requirements. |

##### PRACH time mask

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: Only specify the PRACH ON power measurement period requirements for 120kHz SCS.

  - Option 2: PRACH ON power measurement period table should be updated for 480 and 960 SCS as in (0166)

- Recommended WF

  - further discuss

| Company | Comments |
| --- | --- |
| QCOM | Option 2. |
| Nokia B | Option 2. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Confirm Option 2 in WF on 60 GHz UE Requirements. |

##### General ON/OFF time mask for n263

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: For unlicensed band n263, the transient period locates in both ON slot and OFF slots.

  - Option 2:  Define General ON/OFF time mask as in FR2-1

- Recommended WF

  - Companies discuss Options 1 and 2 during round 1

| Company | Comments |
| --- | --- |
| Nokia | We support option 2. There is no need to change the principles of how and where transient periods are placed compared to FR2-1. |
| Company B |  |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss two options in WF on 60 GHz UE Requirements |

#### Features

##### Multi-band relaxation

Sub-topic description: Handling MBR in the spec

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: UE multi-band relaxation factors (left as TBD in the draft CR) in Table 6.2.1.3-4

  - Option 2: For band n263 the multi-band relaxation is 1.0dB for both MBP,n (dB) and MBS,n (dB).

  - Option 3: something else

- Recommended WF

  - discuss during round 1

| Company | Comments |
| --- | --- |
| QCOM | Our understanding is multi-band operation is not being defined in rel17 timeframe, so there is no need to define multi-band relaxation. |
| MediaTek | If UEs are allowed to have multiple band hardware design including n263, MBR is required even if no inter-band CA operation. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss in WF on 60 GHz UE Requirements |

##### CA

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: Regarding CA within FR2-2 and UL MIMO, we have not found agreements on the related requirements.  Thus, in the interest of making progress, we recommend focusing the Rel-17 requirements for band n263 to cover just single carrier operation without UL MIMO. Include Apple notes on CA and UL MIMO from R4-2207696

  - Option 2:

- Recommended WF

  - discuss during round 1

| Company | Comments |
| --- | --- |
| Nokia | It is necessary to define at least downlink CA requirements. We think this is feasible following similar delta in requirements as in FR2-1 single carrier and CA receiver requirements. |
| Company B |  |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss in WF on 60 GHz UE Requirements. |

##### UL MIMO

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: Uplink MIMO not in rel17.

  - Option 2: Uplink MIMO in rel17

- Recommended WF

  - discuss during round 1

| Company | Comments |
| --- | --- |
| QCOM | We are ok with option 1, not having UL MIMO defined in rel17 timeframe. |
| Company B |  |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss in WF on 60 GHz UE Requirements |

#### Beam correspondence

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: If beam correspondence requirements for FR2-2 PC3 are introduced, then reuse the FR2-1 PC3 beam correspondence procedure for FR2-2 PC3. For beam correspondence tolerance, further discussion may be necessary.

  - Option 2: All FR2-2 UEs declare beam correspondence

- Recommended WF

  - discuss during round 1

| Company | Comments |
| --- | --- |
| QCOM | Agree proposal 1 |
| MediaTek | Agree proposal 1 |
| Nokia | It is unclear what declaring beam correspondence means here. In our view all FR2-2 UEs should meet beam correspondence requirements without need for UL beam sweeping |
| LGE | Agree proposal 1 |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Confirm proposal 1 in WF on 60 GHz UE Requirements with some clarification. |

#### Time-related

##### ON/ON transient periods for 480 and 960 SCS

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: Introduce 2 µS improved ON/ON transient period as optional UE capabilities for 480 and 960 kHz SCS.

  - Option 2: For faster capability of ON/ON transient period for 480 and 960 kHz SCS, it is proposed to further discuss in Rel-18.

  - Option 3: Same as FR2-1

  - Option 4: For optional ON-ON transient time, only one value among 1 us or 2 us is specified.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| QCOM | Option 3. Our view the bundling is a corner case therefore the gains in a real system are not significant. |
| Nokia | We are ok with option 1 and option 4. There is no follow-up WI to discuss in rel-18. |
| MediaTek | Option 3 |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss options 1,3,4 in WF on 60 GHz UE Requirements |

##### Beam direction switching time

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: 200 nsec

  - Option 2: Use a UE beam direction switching time of 59 ns.

- Recommended WF

  - further discuss

| Company | Comments |
| --- | --- |
| QCOM | Option 1. As we have shared in many meetings prior the UE implementation timeline can do 200 nsec. |
| Nokia | Support option 2. |
| MediaTek | Option 1 |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss options 1 and 2 in WF on 60 GHz UE Requirements. |

#### FFT sizes (R4-2208226)

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: To adopt the FFT sizes for FR2-2 in the following table 2-1.

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

- Recommended WF

  - discuss during round 1

| Company | Comments |
| --- | --- |
| QCOM | We don’t understand the proposal. Would like to ask the proposal what spec needs to have this table added. |
| Nokia | This relates to EVM window length defined in annex F.5. 2048 is sufficient for 960 kHz / 2000 MHz. |
| CATT | As Nokia commented, this is related to the FFT size and EVM window length for the EVM measurement. It’s also discussed in [311]. If UE experts think it ok, maybe we can discuss and conclude it in [311]. We can also share the thinking here. When the largest CBW was discussed, our company proposed 2GHz CBW because we think 2048 FFT can be ok for the Fs, then Tc is not need to be changed with 2048 FFT size. When we considered more on the filter design, we found that 4096 FFT may bring some benefit for the digital filter design if 156 RB or less RB is the SU agreement and BB digital filter needs to be used. However, when think more on the spec impact, we’re ok with 2048, and Tc will not need to be changed. How to implement t in the design actually can have some flexibility. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss option 1 in WF on 60 GHz UE Requirements |

### Receive

#### ACS

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

|  |  | Case 1 |  | Case 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | ACS dB | Pwr | Pint | Pwr | Pint |
| Proposal 1 | 21 | REFSENS + 14 | REFSENS + 33.5 | -44.5 dBm | -25 dBm |
| Proposal 2 | Same as proposal 1 but only for 100 MHz and 400 MHz. FFS for wider BWs. |  |  |  |  |

- Recommended WF

  - WF1: Agree proposal 2

  - WF2: Further discuss ACS for 800, 1600, and 2000 MHz

| Company | Comments |
| --- | --- |
| QCOM | Agree with WF1 and WF2 |
| Nokia | Proposal 1 is preferred. Network conditions for proper co-existence do not change with channel bandwidth. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Two comments intersect on defining 400MHz and 100 MHz. Confirm 100/400 ACS parameters. Continue discussing higher CCBWs |

#### IBB

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: The IBB of band n263 is specified for all supported bandwidths.

  - Proposal 2: The IBB requirement of band n263 is the same as ACS requirement.

  - Proposal 3: Single CC Similar to FR2-1 with changed interference level (9508)

  - Proposal 4: intra contig DL CA similar to FR2-1 with interference level changed (9508)

  - Proposal 5: Define In-band Blocking requirement for band n263 as provided in Table 5. (R4-2209323)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | We are ok with proposal 1, 2, 3 and 4. Proposal 5 has some issues, e.g. channel bandwidths include 200 MHz which is not defined for FR2-2 and requirements do not extend to wider than 400 MHz channel bandwidths. |
| Company B |  |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss the proposals in WF on 60 GHz UE Requirements |

#### RX maximum input level

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: For single CC same as FR2-1 (FR2-1 spec show for reference.)

  - Proposal 2: For intra contig DL CA same as FR2-1(FR2-1 spec show for reference.)

- Recommended WF

  - Further discuss proposals

| Company | Comments |
| --- | --- |
| QCOM | We need to discuss further in the context of wider BWs in FR2-2 |
| Nokia | We support both proposals |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss in WF on 60 GHz UE Requirements |

#### RX spurious emissions

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1: Same as FR2-1 (9508) (FR2-1 spurious shown for reference)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Ok with option 1 |
| QCOM | RAN4 should align with EN draft spec which is based on ERC Rec 74-01 |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss in WF on 60 GHz UE Requirements |

### Spec structure and suffix F for n263

Sub-topic description: Description here

Open issues and candidate options before e-meeting:

- Proposals

  - Option 1:  Use suffix F

  - Option 2: Don’t use

- Recommended WF

  - discuss during round 1

| Company | Comments |
| --- | --- |
| QCOM | Option 2. Adding a suffix for 60 GHz doesn’t make any sense to us. The existing spec suffix structure works. |
| Xiaomi | Option 2. |
| Nokia | Support option 2. |

| Issue name | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: Agree option 2. No further discussion |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest focusing on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207697 | Company A: Company B: |
| R4-2208760 | Company A: Company B: |
| R4-2208781 | Company A: Company B: |
| R4-2209510 | Company A: Company B: |
| R4-2208762 | Company A: Company B: |
| R4-2209511 | Company A: Company B: |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2207697 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2208760 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2208781 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2209510 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2208762 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2209511 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on 60 GHz UE Requirements | Qualcomm Incorporated |  |

Existing tdocs

| T-doc number | title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2207696 | On remaining Tx issues for band n263 | Apple | Noted |  |
| R4-2208049 | UE Tx requirements for FR2-2 - remaining issues | Intel Corporation | Noted |  |
| R4-2208619 | Discussion on remaining Tx RF requirements for FR2-2 | vivo | Noted |  |
| R4-2208647 | Discussion on open specification items for Tx RF requirements | LG Electronics Finland | Noted |  |
| R4-2208759 | On n263 associated band specific Tx requirements | Huawei, HiSilicon | Noted |  |
| R4-2209507 | on the UE TX requirement | Xiaomi | Noted |  |
| R4-2210166 | 60 GHz UE TX | Qualcomm Incorporated | Revised to R4-2210433 and Noted |  |
| R4-2208226 | Discussion on FFT sizes for 52 6-71GHz | CATT | Noted |  |
| R4-2208754 | Upper limit on configured maximum power for 57-71 GHz | Ericsson | Noted |  |
| R4-2209322 | Multi-band relaxation requirement for band n263 | Apple | Noted |  |
| R4-2209716 | UE features for NR in 52.6 GHz - 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2209717 | On UE Tx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Revised to R4-2210350 and Noted |  |
| Draft CRs |  |  |  |  |
| R4-2207697 | Draft CR to 38.101-2 on band n263 Tx aspects | Apple |  |  |
| R4-2208760 | Draft CR for n263 RF Tx requirements | Huawei, HiSilicon |  |  |
| R4-2208781 | draft CR on open specification for PC2 in FR2-2 | LG Electronics Finland |  |  |
| R4-2209510 | draftCR on the UE TX requirement for band n263 | Xiaomi |  |  |
|  | RX |  |  |  |
| R4-2208620 | Discussion on remaining Rx RF requirements for FR2-2 | vivo | Noted |  |
| R4-2208761 | On n263 associated band specific Rx requirements | Huawei, HiSilicon | Noted |  |
| R4-2209323 | Rx requirement for band n263 | Apple | Noted |  |
| R4-2209508 | on the UE RX requirement | Xiaomi | Noted |  |
| R4-2209718 | On UE Rx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Noted |  |
| Draft CRs |  |  |  |  |
| R4-2208762 | Draft CR for n263 RF Rx requirements | Huawei, HiSilicon |  |  |
| R4-2209511 | draftCR on the UE RX requirement for band n263 | Xiaomi |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on to/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm Inc | Phil Coan | pcoan@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e., Company A (XX, XX)

# Agreements to date

For reference the moderator has listed the UE requirement agreements as of the beginning of this RAN4 meeting.

| Spec clause | description | agreement | comment |
| --- | --- | --- | --- |
|  | PC3 UE min peak EIRP | Agreement: PC3 UE min peak EIRP is 14.1dBm (R4-2206536) |  |
|  | PC1 min peak EIRP | Agreement: PC1 min peak EIRP is 30.6 dBm. (R4-2206536) |  |
|  | PC2 min peak EIRP | Agreement: PC2 min peak EIRP is 22.7 dBm. (R4-2206536) |  |
|  | PC3 spherical coverage 50%ile drop | Agreement: 11.8 dB drop (R4-2206536) |  |
|  | PC1 %ile for spatial coverage | Agreement: Agree on 85%ile for PC1 spherical coverage. (R4-2206536) |  |
|  | PC1 drop for spatial coverage | Agreement: Agree on 11.5dB for PC1 drop for spatial coverage. (R4-2206536) |  |
|  | PC2 %ile for spatial coverage | Agreement: agree 60%ile for PC2 spatial coverage. (R4-2206536) |  |
|  | PC2 drop for spatial coverage | Agreement: agree on 15.1dB for PC2 drop for spatial coverage. (R4-2206536) |  |
|  | PC3 REFSENS | Agreement: PC3 REFSENS is -72 dBm for n263 400 MHz. (R4-2206536) |  |
|  | PC1 REFSENS | Agreement: -79 dBm in 400 MHz (R4-2206536) |  |
|  | PC2 REFSENS | Agreement: PC2 REFSENS is -80.3 dBm for n263 400 MHz. (R4-2206536) |  |
|  | EIS all power classes | Agreement: use the spherical coverage drops from each power class to determine the EIS (R4-2206536) |  |
|  | TRP | Agreement: RAN4 agrees to ensure EN max EIRP, TRP, max PSD requirements are covered by the RAN4 spec. FFS how to accomplish that in the RAN4 spec. (R4-2206536) | Discussion continues |
|  | UE OBW | Specify 99% OBW requirements in CCBW (R4-2120062) |  |
|  | 64 QAM | Find reference 64QAM optional UL and DL |  |
|  | SCS support | Find reference RAN1 decision on optional 480 and 960 SCS |  |
|  | UE ACLR | Agreement: agree on 15dB ACLR (R4-2206536) |  |
|  | UE spectral utilization | (R4-2206536) Agreement: agree the spectral utilization in the table below. Main session chairman has put an additional agreement in the meeting report that the statement captured in 16 Spectrum Utilization is the working assumption and to be confirmed in the future meeting. | Discussion continues |
|  | EVM | Agreement : PTRS is to be used in EVM processing for greater than 16-QAM . FFS whether 16-QAM and lower EVM will use PTRS. (R4-2120062) | Discussion continues |
|  | ON/ON transient periods for 480 and 960 SCS | R4-2202366 Use the same 5usec for FR2-2. FFS on introduction of a single value among {1, 2, 3} µS improved ON/ON transient period as the optional UE capabilities for 480 and 960 kHz SCS. | Discussion continues |
|  | SEM | -5/-13 table method agreed (find reference). Need exact table agreed | Discussion continues |

SU table working assumption

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

# Table of open specification items

For reference the moderator has listed the UE requirement open items as of the beginning of this RAN4 meeting

| 38.101-2 clause | description | comments |
| --- | --- | --- |
| 6.2.1.1 – 6.2.1.3 | TRP | PC1, PC2, PC3 |
| 6.2.2.1 – 6.2.2.3 | MPR | PC1, PC2, PC3 |
| 6.2.3 | A-MPR |  |
| 6.2A.1 | max Pout CA |  |
| 6.2A.2 | MPR CA | PC1, PC2, PC3 |
| 6.2A.3 | A-MPR CA |  |
| 6.3.1 | minimum Pout | PC1, PC2, PC3 |
| 6.3.2 | TX OFF power |  |
| 6.3.3.4 | PRACH time mask |  |
| 6.3.4. | absolute, relative, aggregate power tolerance |  |
| 6.3A.1 | min power for CA | PC1, PC2, PC3 |
| 6.3A.2 | OFF power for CA |  |
| 6.3A.4 | absolute, relative, aggregate power tolerance |  |
| 6.4 | EVM, carrier leakage, inband emissions | PC1, PC2, PC3 |
| 6.4A | CA EVM, carrier leakage, inband emissions | PC1, PC2, PC3 |
| 6.5.2 | SEM |  |
| 6.5.3 | spurious, coex, addition spurious |  |
| 6.5A.1.1 | OBW for intra contig CA |  |
| 6.5A.2.1.1 | SEM for intra contig CA |  |
| 6.5A.2.3.1 | ACLR for intra contig CA |  |
| 6.5A.3 | intra contig CA spurious, coex, additional spurious |  |
| 6.6 | beam correspondence | PC1, PC2, PC3 |
| 6.6A | beam correspondence for intra contig CA |  |
| 7.3A | REFSENS and EIS for intra contig CA |  |
| 7.4 | maximum input level |  |
| 7.4A.1 | max input level for intra contig CA |  |
| 7.5 | ACS |  |
| 7.5A | ACS for intra contig DL CA |  |
| 7.6 | blocking |  |
| 7.6A | blocking for intra contig DL CA |  |
| 7.9 | RX spurious emissions |  |