# 3GPP TSG-RAN WG4 Meeting # 103-e                                                         R4-22xxxxx

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:			4.1.4, 9.5.4

Source:	Moderator (ZTE Corporation)

Title:	Email discussion summary for [103-e][303] NR_EMC

Document for:	Information

Introduction

For the RAN4 [103-e] [303] NR_EMC, the main topics are about NR UE/BS EMC and NR repeaters EMC. Therefore, the discussions will separate into four parts:

Topic #1: Agenda item 4.1.4: NR UE/BS EMC

Topic #2: Agenda item 9.5.4: NR Repeaters EMC

Topic #1: NR UE/BS EMC (AI: 4.1.4)

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207894 | Ericsson | Cat F. draft CR for Rel-16 TS38.175. Reason for change: These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater (R4-2117585). This CR proposes similar changes in TS 38.175. Summary of change: -Added a statement in the scope to clarify that technical specifications related to the antenna port are not considered. -Add instead the level of 10 V/m and delete the references to EU EMC regulations. -Remove the note claiming that the EUT should be fully discharged between each ESD exposure. It is very difficult to ensure that EUT is “fully discharged”, especially if there are plastic parts on it. -Remove a statement on application of transients on an impedance that shall be 50 ohm -Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |
| R4-2210044 | Ericsson | Mirror draft CR(Rel-17) to R4-2207894 |
| R4-2207895 | Ericsson | Cat F. draft CR for Rel-15 TS36.113. Reason for change: These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater (R4-2117585). This CR proposes similar changes in TS 36.113 Summary of change: -Introduce use of reverberation chamber for RF electromagnetic field testing. -Remove the note claiming that the EUT should be fully discharged between each ESD exposure. It is very difficult to ensure that EUT is “fully discharged”, especially if there are plastic parts on it. -Remove a statement on application of transients on an impedance that shall be 50 ohm -Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |
| R4-2207896 | Ericsson | Mirror draft CR(Rel-16) to R4-2207895 |
| R4-2210045 | Ericsson | Mirror draft CR(Rel-17) to R4-2207895 |
| R4-2207897 | Ericsson | Cat F. draft CR for Rel-15 TS37.113. Reason for change: These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater (R4-2117585). This CR proposes similar changes in TS 37.113 Summary of change: -Remove the note claiming that the EUT should be fully discharged between each ESD exposure. It is very difficult to ensure that EUT is “fully discharged”, especially if there are plastic parts on it. -Remove a statement on application of transients on an impedance that shall be 50 ohm -Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |
| R4-2207898 | Ericsson | Mirror draft CR(Rel-16) to R4-2207897 |
| R4-2210046 | Ericsson | Mirror draft CR(Rel-17) to R4-2207897 |
| R4-2207899 | Ericsson | Cat F. draft CR for Rel-15 TS37.114. Reason for change: These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater (R4-2117585). This CR proposes similar changes in TS 37.114 Summary of change: -Added a statement in the scope to clarify that technical specifications related to the antenna port are not considered. -Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |
| R4-2207900 | Ericsson | Mirror draft CR(Rel-16) to R4-2207899 |
| R4-2210047 | Ericsson | Mirror draft CR(Rel-16) to R4-2207899 |
| R4-2207901 | Ericsson | Cat F. draft CR for Rel-15 TS38.113. Reason for change: These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater (R4-2117585). This CR proposes similar changes in TS 38.113 Summary of change: -Added a statement in the scope to clarify that technical specifications related to the antenna port are not considered. -Add instead the level of 10 V/m and delete the references to EU EMC regulations. -Remove the note claiming that the EUT should be fully discharged between each ESD exposure. It is very difficult to ensure that EUT is “fully discharged”, especially if there are plastic parts on it. -Remove a statement on application of transients on an impedance that shall be 50 ohm -Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |
| R4-2207902 | Ericsson | Mirror draft CR(Rel-16) to R4-2207901 |
| R4-2210048 | Ericsson | Mirror draft CR(Rel-16) to R4-2207901 |
| R4-2208380 | ZTE Corporation | Observation 1: To align with BS TS 38.104, the highest test frequency of radiated spurious emission for band n46 and n96 is 26GHz. Observation 2: The MU values in terms of the calculation model of ETSI TR 100 028-1 for radiated spurious emission with large than 2m big size EUT would be exceed 6dB. Proposal 1: It is recommended that the maximum MU value between 12,75 GHz and 26 GHz with EUT size greater than 1m in BS EMC specification should be 9dB. |
| R4-2208382 | ZTE Corporation | Cat F. draft CR for Rel-15 TS38.114. Reason for change: According to the last WF(R4-2207188), the NR operating band allocation shall be extent, which is the fifth harmonic frequency of the upper frequency edge for band n46 and n96 will exceed 12.75 GHz. Accordingly, the maximum measurement uncertainty (MU) of radiated emission above 12.75 GHz (up to 26GHz) should be defined, which is analyzed in R4-2208380. Summary of change: The highest test frequency of radiated spurious emission for band n46 and n96 is proposed to be 26GHz. The maximum MU value of BS EMC specifications above 12.75GHz (up to 26GHz) is proposed to be 6dB (<=1) and 9dB (>1m). |
| R4-2208384 | ZTE Corporation | Mirror draft CR(Rel-16) to R4-2208382 |
| R4-2208386 | ZTE Corporation | Mirror draft CR(Rel-17) to R4-2208382 |
| R4-2209653 | Huawei, HiSilicon | Cat F. CR for Rel-17 TS34.124 (UTRA UE EMC). Reason for change: Referring to the RAN4#101-bis-e discussion on R4-2201804 (Discussion on the Rel-17 specifications: 25-series and 34-series, Noted), it was concluded in Moderators summary in R4-2202305 that “proposals are agreed and see how to capture and communicate to MCC”: Proposal 1: mark 25-series specifications as “inhibit upgrade”. Proposal 2: mark 34-series specifications as “inhibit upgrade”. There was related follow-up discussion during RAN#95-e meeting. Moderator summary in RP-220906 has concluded to send LS to CT and SA (RP-221015) with the following: - LSout RP-221015 to: SA, CT is approved suggesting - to not upgrade 25-series REL-16 specs to REL-17 - to not upgrade 34-series REL-16 specs TS 34.124, TS 34.926, TS 34.109 to REL-17 - to ask SA to decide about references to the not upgraded specs in other REL-17 specs should be corrected Further referring to the related SA#95e discussion on SP-220358 (LS IN LS from TSG RAN: LS on 25-series and 34-series Specifications Upgrading. Source: RP-221015), the following was concluded: Conclusion: It was decided to upgrade the 25-series and 34-series TSs and the previously exempted SA WG4 specifications after this meeting. All in all, the TS 34.124 specification was decided to be updated to Rel-17. With this, we see the need to correct number of obvious error and technical inconsistencies observed in this UTRA UE EMC specification. It is proposed to correct only the latest version of the TS 34.124 specification, and to skip corrections of the previous releases where similar bugs are also existing. Summary of change: -Scope: UTRA clarification -References: removal of the outdated CISPR 22 reference, being replaced with the CISPR 32 reference -Correction of the Rx exclusion band definition to be band-agnostic. Please note that the current band-specific definition is incorrect for FDD band XII, and for TDD band b. Editorial corrections. |
| R4-2209654 | Huawei, HiSilicon, Samsung | Cat F. draft CR for Rel-15 TS38.124. Reason for change: Referring to the TS 38.124 v 15.2.0, the following was captured for the radiated Rx spurious emissions limits: Despite the above highlighted unresolved [], there were also inconsistencies with the limits defined in the NR UE RF specification TS 38.101-1. Therefore, CR in R4-2008717 was agreed and implemented. Afterwards it was observed, that the radiated Rx spurious emissions limits (for idle mode testing) were unintentionally removed from the specification. In this CR we are fixing this by bringing the Rx spurious emissions limits back, further aligning them with the TS 38.101-1 specification to better reflect SM.329 guidelines (missing consideration of the 5th harmonic, missing 12.75-26 GHz range). Summary of change: -3.1: Traffic mode definition introduced, aligned with the definition in 34.124 and 36.124. -3.3: missing abbreviations added -4.2: text consistency correction -8.1, 9.1: clarification on the idle mode definition -8.2.4: addition of the missing Rx spur limits, aligned with the TS 38.101-1, clause 7.9 |
| R4-2209655 | Huawei, HiSilicon, Samsung | Mirror draft CR(Rel-16) to R4-2209654 |
| R4-2209656 | Huawei, HiSilicon, Samsung | Mirror draft CR(Rel-16) to R4-2209654 |
| R4-2209657 | Huawei, HiSilicon | Cat F. draft CR for Rel-8 TS36.124. Reason for change: Recently, new E-UTRA band 103 was introduced in Rel-17. It was observed, that it was not implemented to the TS 36.124 specification. At the same time,it was observed that Rx exclusion zone was not defined for bands 17. In order to reduce the workload related to the new bands introduction, this CR is introducing a band-agnostic way to define the Rx exclusion band. Similar correction was already introduced in other EMC specifications to redure the workload when new bands are introduced. Summary of change: -Introduction of a band-agnostic Rx exclusion band definition. |
| R4-2209658 | Huawei, HiSilicon | Mirror draft CR(Rel-9) to R4-2209657 |
| R4-2209659 | Huawei, HiSilicon | Mirror draft CR(Rel-10) to R4-2209657 |
| R4-2209660 | Huawei, HiSilicon | Mirror draft CR(Rel-11) to R4-2209657 |
| R4-2209661 | Huawei, HiSilicon | Mirror draft CR(Rel-12) to R4-2209657 |
| R4-2209662 | Huawei, HiSilicon | Mirror draft CR(Rel-13) to R4-2209657 |
| R4-2209663 | Huawei, HiSilicon | Mirror draft CR(Rel-14) to R4-2209657 |
| R4-2209664 | Huawei, HiSilicon | Mirror draft CR(Rel-15) to R4-2209657 |
| R4-2209665 | Huawei, HiSilicon | Mirror draft CR(Rel-16) to R4-2209657 |
| R4-2209667 | Huawei, HiSilicon | Mirror draft CR(Rel-17) to R4-2209657 |
| R4-2209668 | Huawei, HiSilicon | Proposal 1: 6 GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications which are referring to the IEC 61000-4-3. Proposal 2: the following 38-series NR-related EMC specifications require updates due to IEC 61000-4-3:2020 revision: - TS 38.113 - TS 38.114 - TS 38.124 - TS 38.175 Proposal 3: RAN4 EMC modifications related to the IEC 61000-4-3 updates are to be applied from Rel-17 onwards. Related CRs were provided in [3-6]. |
| R4-2209666 | Huawei, HiSilicon | Cat F. CR for Rel-17 TS38.175. Reason for change: Recently, the IEC 61000-4-3 specification has been updated (IEC 61000-4-3:2020), where one of the modifications introduced was the removal of the upper frequency limit of the RI requirements. RAN4 EMC specifications are referring to the 61000-4-3 and reusing the upper frequency limit for the RI testing up to 6 GHz, so far. Therefore based on the IEC 61000-4-3:2020 content, the related 6GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications. Related discussion paper was provided in R4-2209668. Summary of change: Removal of the upper frequency limit of 6 GHz for the Radiated Immunity testing. |
| R4-2209669 | Huawei, HiSilicon | Cat F. CR for Rel-17 TS38.113. Reason for change: Recently, the IEC 61000-4-3 specification has been updated (IEC 61000-4-3:2020), where one of the modifications introduced was the removal of the upper frequency limit of the RI requirements. RAN4 EMC specifications are referring to the 61000-4-3 and reusing the upper frequency limit for the RI testing up to 6 GHz, so far. Therefore based on the IEC 61000-4-3:2020 content, the related 6GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications. Related discussion paper was provided in R4-2209668. Summary of change: Removal of the upper frequency limit of 6 GHz for the Radiated Immunity testing. |
| R4-2209670 | Huawei, HiSilicon | Cat F. CR for Rel-17 TS38.124. Reason for change: Recently, the IEC 61000-4-3 specification has been updated (IEC 61000-4-3:2020), where one of the modifications introduced was the removal of the upper frequency limit of the RI requirements. RAN4 EMC specifications are referring to the 61000-4-3 and reusing the upper frequency limit for the RI testing up to 6 GHz, so far. Therefore based on the IEC 61000-4-3:2020 content, the related 6GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications. Related discussion paper was provided in R4-2209668. Summary of change: Removal of the upper frequency limit of 6 GHz for the Radiated Immunity testing. |
| R4-2209671 | Huawei, HiSilicon | Cat F. CR for Rel-17 TS38.114. Reason for change: Recently, the IEC 61000-4-3 specification has been updated (IEC 61000-4-3:2020), where one of the modifications introduced was the removal of the upper frequency limit of the RI requirements. RAN4 EMC specifications are referring to the 61000-4-3 and reusing the upper frequency limit for the RI testing up to 6 GHz, so far. Therefore based on the IEC 61000-4-3:2020 content, the related 6GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications. Related discussion paper was provided in R4-2209668. Summary of change: Removal of the upper frequency limit of 6 GHz for the Radiated Immunity testing. |

Open issues summary

In last meeting, it was agreed to extend the highest test frequency of NR BS EMC radiated spurious emission for band n46 and n96 to 26 GHz to align with TS38.104. However, the corresponding MU values for the extended frequency range were still FFS.

The agreements can be found in the agreed WF R4-2207188, which are shown below.

  - WF on MU value between 12.75 GHz and 26 GHz for BS EMC

- FFS on the MU value between 12.75GHz and 26GHz for BS EMC. Encourage companies provide analyses in next meeting

Note 1: Considering it is for TEI and missing the MU value larger than 12.75GHz in Rel-15, it is important to define this value as soon as possible. It was already discussed for several meetings without agreement due to lack of analyses from other companies.

Note 2: If there are no further analyses provided by other companies in next meeting, then the value highlighted in yellow as below table would be recommended to be agreed as the MU value between 12.75 GHz and 26 GHz for BS EMC.

| Parameter | Uncertainty for EUT dimension ≤ 1 m (NOTE 1) | Uncertainty for EUT dimension >1 m (NOTE 1) |
| --- | --- | --- |
| Effective radiated RF power between 30 MHz and 180 MHz | 6 dB | 6 dB |
| Effective radiated RF power between 180 MHz and 4 GHz | 4 dB | 6 dB |
| Effective radiated RF power between 4 GHz and 12,75 GHz | 6 dB | 9 dB (NOTE 2) |
| Effective radiated RF power between 12,75 GHz and 26 GHz | 6 dB | 9 dB (NOTE 2) |
| Field strength between 30 MHz and 12,75 GHz | 6 dB | 6 dB |
| Field strength between 12,75 GHz and 26 GHz | 6 dB | 6 dB |
| NOTE 1: These MU values estimates and are not based on the MU budget calculations. For more background on MU derivation analyses refer to CISPR 16-4-2 [31] and ETSI TR 100 028-1 [32]. NOTE 2: This value may be reduced to 6 dB when further information on the potential radiation characteristic of the EUT is available. |  |  |

Sub-topic 1: MU value for the effective radiated RF power measurements

Issue 1-1: Can we agree with the MU values as captured in for the last approved WF R4-2207188 for the frequency range of 12,75 GHz and 26 GHz (highlighted in yellow as above)?

- Proposals

  - Tentative agreements:Yes

- Recommended WF

  - In terms of the note1 and note 2 in the WF R4-2207188, considering there are no further analyses provided by other companies in this meeting. Moderator recommende to agree the MU values for the frequency range of 12,75 GHz and 26 GHz proposed in R4-2207188.

Sub-topic 2: Upper limits for the radiated Immunity testing

Issue 1-2: Is it ok to remove 6 GHz limit for the Radiated Immunity testing from RAN4 EMC specifications which are referring to the IEC 61000-4-3?

- Proposal:

  - Option 1: Yes (as proposed in  R4-2209668)

  - Option 2: No (Reasons)

- Recommended WF

  - TBA

Issue 1-3: If Yes for issue 1-2, which releases should be corrected to compliance to updates?

- Proposal:

  - Option 1: Rel-17 onwards (as proposed in  R4-2209668)

  - Option 2: Others (for example Rel-15/16 onwards)

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 1-1: Can we agree with the MU values as captured in for the last approved WF R4-2207188 for the frequency range of 12,75 GHz and 26 GHz (highlighted in yellow as above)?

| Company | Comments |
| --- | --- |
| Nokia | The MU is acceptable. |
| ZTE | Agree with recommended WF. |
| Huawei | Based on the WF we are supposed to close this topic during this meeting. We are still running internal checks so we ask for the final decision to be postponed till the second round. |
| Ericsson | For erp, how the calculations were done is not clear, need more details, e.g., the uncertainty value of each factor. For field strength, frequency from 18GHz to 26GHz, how MU was decided is not clear. |

Issue 1-2: Is it ok to remove 6 GHz limit for the Radiated Immunity testing from RAN4 EMC specifications which are referring to the IEC 61000-4-3?

| Company | Comments |
| --- | --- |
| Nokia | What are the problems if the text of 6GHz is not removed? |
| ZTE | Althrough the upper frequency limit of radiated immunity is decided by capability of test equipment in general standard IEC 61000-4-3, but we notice that some product standards like CISPR 35, EN301489-1/50/51 and EN300386 are not following right now. Therefore, shall we wait for their updates? |
| Huawei | Option 1 @Nokia: if we don’t remove 6GHz limit, then we are not following IEC 61000-4-3 specification, which RAN4 EMC is based on. Another consequence is that the FR2 RI testing is not covered. @ZTE: RAN4 specification does not define capabilities of the test equipment – RAN4 defined EMC requirements for BS (and other) products. If you want to follow “CISPR 35, EN301489-1/50/51 and EN300386” standards, then why do we need all those RAN4 EMC specs at all? RAN4 is not responsible to maintaining CISPR no ETSI EMC specs – if those communities decide not to follow IEC 61000-4-3, it is their decision. Our reference is IEC 61000-4-3 which we shall respect. Please note that RAN4 has sent an LS to IEC back in Rel-15, asking for their plans to cover RI for FR2. |
| Ericsson | Option 2. @Huawei: Although IEC removed the upper frequency limit, IEC 61000-4-3 also states that, the frequencies or frequency ranges to be selected for testing by product committees may be limited to those where intentional RF emitting devices actually operate. IEC is generic STD, for all electric and electrical devices. For BS, the frequency range needs to be adjusted. IEC says it is limited only by the capability of the test instrumentation. What if an EMC LAB can only test lower than 6 GHz? For FR2 product, it does not mean we need to test FR2, isn’t it considered as exclusion band? |
| Huawei | I agree with your observation on “, the frequencies or frequency ranges to be selected for testing by product committees may be limited to those where intentional RF emitting devices actually operate.”. However we have FR2 products in 3gpp since a long time – so above 6 GHz is definitely a range where “intentional RF emitting devices actually operate”. “IEC says it is limited only by the capability of the test instrumentation. What if an EMC LAB can only test lower than 6 GHz?“: RAN4 defines requirements for BS producrts, not for labs. If lab cannot test it, then you report it in the test report, or go to other lab. This shall not prevent the RN4 standard to be limited to up to 6 GHz. “For FR2 product, it does not mean we need to test FR2, isn’t it considered as exclusion band?” exclusion band does NOT solve FR2 testing. |

Issue 1-3: If Yes for issue 1-2, which releases should be corrected to compliance to updates?

| Company | Comments |
| --- | --- |
| Huawei | Option 1, due to the availability of the related IEC spec update, against the 3gpp release timeline. We can discuss if we could motivate to start from any earlier release, if needed. |
| Company B |  |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

1. Draft CRs to implement the similar changes as TS 38.114 NR Repeater (R4-2117585)

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207894 Mirror CR R4-2210044 | ZTE: How about correct TS38.114 to align with other EMC spec? Huawei: based on ZTE question, TS 38.113 is not aligned either. With this I would like to stress that having one EMC spec for all the network nodes would be much easier to maintain. Any correction of the EMC spec triggers the need to check consistency with all the other RAN4 EMC specs, which is generating workload. But this is a separate issue. On technical side of the CR, we have some concerns to clarify: - Section 9.2.2: Field strength levels (3v/m, 10v/m): Referring to IEC 61000-4-3 selection of the text levels specifies that the product committees shall select the appropriate test level(s). Annex E gives guidance for the product committees on the selection of test levels: levels are to be selected in accordance with the electromagnetic radiation environment to which the EUT can be exposed when finally installed. Furthermore, deployments/location aspects are considered for the selection of the proper levels. Therefore: applicable field strength levels shall NOT be copy pasted among EMC specs (BS, repeater, IAB, etc.) as related deployment aspects may require more study for the selection of the field strength levels for the Immunity testing. We need to capture motivation for the [3 vs 10 V/m] field strength levels selection somewhere. With the removal of the -50 reference we all remove related information. - Section 9.3.2: we feel that instead of deleting that Note, better to reword it to keep some kind of recommendation, e.g. “NOTE: whenever possible, it is recommended to aim to fully discharge the EUT between each ESD exposure.” - Section 9.4.2: you remove much more than just information on the impedance – the test scope seems to be actually removed. What is the motivation to remove the whole sentence? - Section 9.5: this note indicates on the alternative method. We would rather prefer to keep information on that “clamp injection” instead of deleting. Double checking if it is ok to remove the “intrusive method” text. Ericsson: @ZTE & Huawei: The corrections in TS38.114 were already discussed and approved in previous RAN4#101-e, please refer to R4-2117585. - We agree that having one EMC spec for all the network nodes would be much easier to maintain. |
| R4-2207895 Mirror CR R4-2207896 R4-2210045 | ZTE: Same as above. Huawei: see comments to R4-2207894. Company B: |
| R4-2207897 Mirror CR R4-2207898 R4-2210046 | ZTE: Same as above. Huawei: see comments to R4-2207894. Company B: |
| R4-2207899 Mirror CR R4-2207900 R4-2210047 | ZTE: Same as above. Huawei: Scope correction is not correct. There are TAB connectors covered for AAS BS – text on antenna connectors will be confusing. See comments to R4-2207894. Company B: |
| R4-2207901 Mirror CR R4-2207902 R4-2210048 | ZTE: Same as above. Huawei: Scope correction is not correct. There are TAB connectors covered for NR BS, which is based on AAS architecture framework – text on antenna connectors will be confusing. See comments to R4-2207894. Company B: |

2. MU value draft CRs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208382 Mirror CR R4-2208384 R4-2208386 | Huawei: Pending on issue 1-1. Note 6 shall be re-written in band-agnostic manner so that we do not have to update it in future with new bands. ZTE: @Huawei: Note 6 comes from RF spec TS38.104 and it was asked to align with 38.104 since RAN4 102-e meeting. |

3. CRs based on the IEC 61000-4-3 updates

| CR/TP number | Comments collection |
| --- | --- |
| R4-2209666 | Ericsson: We think the upper limit 6 GHz should be kept. Company B: |
| R4-2209669 | Ericsson: See above. Company B: |
| R4-2209670 | Ericsson: See above. Company B: |
| R4-2209671 | Ericsson: See above. Company B: |

4. Miscellaneous CRs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2209653 | Ericsson: It seems OK. Company B: |
| R4-2209654 Mirror CR R4-2209655 R4-2209656 | Ericsson: Regarding Table 8.2.4-2, where is NOTE 1 located? For idle mode, where are the values of limits (-57dBm, -47dBm) from? Company B: |
| R4-2209657 Mirror CR R4-2209658 R4-2209659 R4-2209660 R4-2209661 R4-2209662 R4-2209663 R4-2209664 R4-2209665 R4-2209667 | ZTE: We understand the intention of this CR, however we think there is no need to correct the early Rel, especially for the frozen Rel, since there would no new bands introduce to those early Rel. Also we notice that the similar correction on the exclusion band of 34.124 (R4-2209653) is only for R17 so we suggest the corrections for 36.124 should be R17 onwards. Huawei: @ZTE: the difference with the 34.124 correction is following: when you refer to TS36.104 v8.14.1 (Rel-8) there is Band 17 defined, which is not covered in TS 36.124 v8.2.0 (Rel-8), i.e. Band 17 exclusion zone is missing. Therefore we shall correct TS 36.124 back from Rel-8 onwards. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: Can we agree with the MU values as captured in for the last approved WF R4-2207188 for the frequency range of 12,75 GHz and 26 GHz (highlighted in yellow as above)? | Basically, no objections on MU values as captured in the last the last approved WF R4-2207188 for the frequency range of 12,75 GHz and 26 GHz (highlighted in yellow) although one company need to internal check in 2nd round and one company think more details would be needed. Considering it have been discussed for a long time, only 1 company provide the analysis and there are no inputs from the other companies, in terms of the last approved WF R4-2207188, moderator recommend to agree the MU values for the frequency range of 12,75 GHz and 26 GHz proposed in R4-2207188 and close this topic in this meeting. Tentative agreements: - Approve the MU values highlighted in yellow for the frequency range of 12,75 GHz and 26 GHz in the following table: Parameter; Uncertainty for EUT dimension ≤ 1 m (NOTE 1); Uncertainty for EUT dimension >1 m (NOTE 1). Effective radiated RF power between 30 MHz and 180 MHz; 6 dB; 6 dB. Effective radiated RF power between 180 MHz and 4 GHz; 4 dB; 6 dB. Effective radiated RF power between 4 GHz and 12,75 GHz; 6 dB; 9 dB (NOTE 2). Effective radiated RF power between 12,75 GHz and 26 GHz; 6 dB; 9 dB (NOTE 2). Field strength between 30 MHz and 12,75 GHz; 6 dB; 6 dB. Field strength between 12,75 GHz and 26 GHz; 6 dB; 6 dB. NOTE 1: These MU values estimates and are not based on the MU budget calculations. For more background on MU derivation analyses refer to CISPR 16-4-2 [31] and ETSI TR 100 028-1 [32]. NOTE 2: This value may be reduced to 6 dB when further information on the potential radiation characteristic of the EUT is available.. Candidate options: Recommendations for 2nd round: - Companies check the tentative agreements |
| Issue 1-2: Is it ok to remove 6 GHz limit for the Radiated Immunity testing from RAN4 EMC specifications which are referring to the IEC 61000-4-3? | Comments are diverse. 2 companies disagree to remove the 6GHz, and 1 company think it should postpone till other standards also do the updates. Tentative agreements: - N/A Candidate options: For the Radiated Immunity testing from RAN4 EMC specifications (TS38.175/TS38.113/38.124/38.114). Option 1: remove 6 GHz limit Option 2: Postpone to remove 6 GHz limit till other standards ( i.e. CISPR 35, EN301489-1/50/51 and EN300386 ) also do the updates Option 3. Keep 6GHz limits unchanged. Recommendations for 2nd round: - Further discuss in the WF |
| Issue 1-3: If Yes for issue 1-2, which releases should be corrected to compliance to updates? | It depends on the consensus of issue 1-2. Tentative agreements: -N/A Candidate options: Recommendations for 2nd round: - together with issue 1-2. |

Discussion on 2nd round (if applicable)

Topic #2: NR Repeaters EMC  (AI: 9.5.4)

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208379 | ZTE Corporation | Observation 1: The RF requirements for type 2-O NR Repeater are rated TRP output power and the rated beam EIRP output power. Throughput is no longer used for NR Repeater RF. Observation 2: The EMC performance assessment parameters for Repeater type 1-C and type 2-O should be aligned. Observation 3: NR Repeater does not have base band function. Observation 4: The highlighted row in the table is the perf task that should be filled next step. Proposal 1: Throughput could not be used as performance assessment for NR Repeater EMC. Proposal 2: For EMC repeater performance assessment, consider the following two options: Option 1: Gain; Option 2: Power accuracy; Proposal 3: To adopt work split method for the left clauses of performance/test related. |
| R4-2209140 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 should evaluate the pros and cons for the following candidate performance assessment parameters: - Option 1: power accuracy - Option 2: throughput Proposal 2: For TDD NR repeaters, RAN4 should discuss testability issues taking into account the candidate performance assessment parameters based on the outcome of RF discussions. |

Open issues summary

The core parts for TS38.114 have already completed, test/performance should be started and discussed from Q2.

Performance assessment

According to R4-2207187, the performance assessment parameter is FFS for NR repeater, which is

- Proposal 2-1: Parameters for performance assessment should be aligned between NR type 1-C and type 2-O Repeater EMC, FFS on the performance assessment parameters.

- Proposal 2-2: UL and DL are worked together for the communication link configuration of TDD NR Repeater. But whether or not monitoring their performance together should wait for RF discussion results.

- Proposal 2-3: Switching should be considered as a baseline for testing of TDD NR repeater.

For the remaining contents for the performance/test, to facilitate the completion of the performance/test contents in the specification on time, and to avoid the overlapping work effort among companies, work split approaches for TS38.114 is recommended to be used per the guidance.

Sub-topic 2.1: Performance assessment criteria

Issue 2-1 Could throughput be used as performance assessment for NR Repeater EMC?

- Proposal:

  - Option 1: No (as proposed in R4-2208379)

  - Option 2: Yes (Reasons)

- Recommended WF

  - TBA

Issue 2-2 Which candidate criteria for performance assessment should be adopted for NR Repeater EMC testing?

- Proposal:

  - Option 1: Gain

  - Option 2: Power accuracy

  - Option 3: Throughput

- Recommended WF

  - TBA

Sub-topic 2.2: Work split for the remaining clauses related to test/performance in TS38.114

Moderator recommend TS work split to be discussed in  this meeting, only focus on the work split on the clauses highlighted in yellow in the following table. Companies should fill out their company’s name in the ‘work split’ column if they show interesting at the certain sub-clause(s). Noted that ONLY one company in each subclause is allowed in the final version.

TS38.114 work split for next meeting (i.e. Aug meeting #104 meeting)

| Clauses in TS38.114 | Status | Work Split |
| --- | --- | --- |
| 1 Scope | Completed |  |
| 2 Reference | Completed (completed in R4-2202986) |  |
| 3 Definitions, symbols and abbreviations | Completed (completed in R4-2204358) |  |
| Section 4: Test condition |  |  |
| 4.1 General | Ongoing | ZTE |
| 4.2 Arrangements for establishing a communication work | Ongoing | ZTE |
| 4.3 Narrow band responses on receivers | Completed |  |
| 4.4 Exclusion bands | Ongoing |  |
| 4.5 NR repeaters test configurations | Ongoing |  |
| Section 5: Performance assessment |  |  |
| 5.1 General | Ongoing Ongoing | Huawei |
| 5.2 NR repeater |  |  |
| 5.3 Ancillary equipment | Completed |  |
| Section 6: Performance criteria |  |  |
| 6.1 Performance criteria for continuous phenomena for BS | Ongoing Ongoing | Huawei |
| 6.2 Performance criteria for transient phenomena for BS |  |  |
| 6.3 Performance criteria for continuous phenomena for Ancillary equipment | Completed |  |
| 6.4 Performance criteria for transient phenomena for Ancillary equipment | Completed |  |
| Section 7: Applicability overview |  |  |
| 7.1 Emission | Completed |  |
| 7.2 Immunity | Completed |  |
| Section 8: Emission |  |  |
| 8.1 Test configurations | Ongoing | Nokia |
| 8.2 Radiated emission | Ongoing | Nokia |
| 8.3 Conducted emission DC power input/output port | Completed |  |
| 8.4 Conducted emissions, AC mains power input/output port | Completed |  |
| 8.5 Conducted emissions, telecommunication port | Completed |  |
| 8.6 Harmonic Current emissions (AC mains input port) | Completed |  |
| 8.7 Voltage fluctuations and flicker (AC mains input port) | Completed |  |
| Section 9: Immunity |  |  |
| 9.1 Test configurations | Ongoing | ZTE |
| 9.2 RF electromagnetic field (80MHz - 6000MHz) | Completed |  |
| 9.3 Electrostatic discharge | Completed |  |
| 9.4 Fast transient common mode | Completed |  |
| 9.5 RF common mode (0.15 MHz - 80 MHz) | Completed |  |
| 9.6 Voltage dips and interruptions | Completed |  |
| 9.7 Surges, common and differential mode | Completed |  |

Companies views’ collection for 1st round

Open issues

Issue 2-1 Could throughput be used as performance assessment for NR Repeater EMC?

| Company | Comments |
| --- | --- |
| Nokia | Support Option 1. |
| ZTE | Option1: No NR Repeater does not have baseband function, which means that throughput can not be obtained. |
| Huawei | Option 1 seems well justified. Besides, there are no demod requirements defined for the NR repeater. |

Issue 2-2 Which candidate criteria for performance assessment should be adopted for NR Repeater EMC testing?

| Company | Comments |
| --- | --- |
| Nokia | Option 3 is not applicable according to Issue 2-1. No strong preference because gain and power accuracy are inter-related. Pros and cons of gain and power accuracy should be further discussed and assessed. |
| ZTE | Option1: Gain Considering power accuracy is for EIRP testing, which is not the objective of EMC testing and also gain is adopted in TS36.113 and other EMC spec like EN301489-50, therefore, we prefer to option 1. |
| Huawei | It seems justified to follow the framework of the EUTRA repeater in TS 36.113, which is using Gain as the metric. In case other more suitable metrics are possible, we would be also open to discuss them more. |
| Ericsson | Among the three options, except for feasibility, we should discuss and have an agreement on which is the key performance of NR TDD repeater. |
| Nokia2 | What is the definition of gain? Does gain mean antenna gain or amplifier gain? Can you distinguish the two? In TS 36.113, there is no corresponding Type 2-O repeater. Is gain suitable for 2-O? Could the proponents provide technical advantages for adopting gain instead of power accuracy. Simply it is used in 36.113 is one but not sufficient. Are there more advantages? EIRP is a simple and easy to measure metric. |

CRs/TPs comments collection

N/A.

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1 Could throughput be used as performance assessment for NR Repeater EMC? | No objections Tentative agreements: - Throughput cannot be used as performance assessment for NR Repeater EMC Candidate options: Recommendations for 2nd round: - Captured into the WF |
| Issue 2-2 Which candidate criteria for performance assessment should be adopted for NR Repeater EMC testing? | 1 company prefer to select gain while another 1 company prefer to select Power accuracy, and the other companies think more studies would be needed Tentative agreements: -N/A Candidate options: - Option 1: Gain - Option 2: Power accuracy - Option 3: Other criteria not preclude. Recommendations for 2nd round: - Discuss the assigned WF |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Moderator recommend TS work split to be discussed in  this meeting, only focus on the work split on the clauses highlighted in yellow in the following table. Companies should fill out their company’s name in the ‘work split’ column if they show interesting at the certain sub-clause(s). Noted that ONLY one company in each subclause is allowed in the final version.

Continue to the work split in 2nd round.

TS38.114 work split for next meeting (i.e. Aug meeting #104 meeting)

| Clauses in TS38.114 | Status | Work Split |
| --- | --- | --- |
| 1 Scope | Completed |  |
| 2 Reference | Completed (completed in R4-2202986) |  |
| 3 Definitions, symbols and abbreviations | Completed (completed in R4-2204358) |  |
| Section 4: Test condition |  |  |
| 4.1 General | Ongoing | ZTE |
| 4.2 Arrangements for establishing a communication work | Ongoing | ZTE |
| 4.3 Narrow band responses on receivers | Completed |  |
| 4.4 Exclusion bands | Ongoing |  |
| 4.5 NR repeaters test configurations | Ongoing |  |
| Section 5: Performance assessment |  |  |
| 5.1 General | Ongoing Ongoing | Huawei |
| 5.2 NR repeater |  |  |
| 5.3 Ancillary equipment | Completed |  |
| Section 6: Performance criteria |  |  |
| 6.1 Performance criteria for continuous phenomena for BS | Ongoing Ongoing | Huawei |
| 6.2 Performance criteria for transient phenomena for BS |  |  |
| 6.3 Performance criteria for continuous phenomena for Ancillary equipment | Completed |  |
| 6.4 Performance criteria for transient phenomena for Ancillary equipment | Completed |  |
| Section 7: Applicability overview |  |  |
| 7.1 Emission | Completed |  |
| 7.2 Immunity | Completed |  |
| Section 8: Emission |  |  |
| 8.1 Test configurations | Ongoing | Nokia |
| 8.2 Radiated emission | Ongoing | Nokia |
| 8.3 Conducted emission DC power input/output port | Completed |  |
| 8.4 Conducted emissions, AC mains power input/output port | Completed |  |
| 8.5 Conducted emissions, telecommunication port | Completed |  |
| 8.6 Harmonic Current emissions (AC mains input port) | Completed |  |
| 8.7 Voltage fluctuations and flicker (AC mains input port) | Completed |  |
| Section 9: Immunity |  |  |
| 9.1 Test configurations | Ongoing | ZTE |
| 9.2 RF electromagnetic field (80MHz - 6000MHz) | Completed |  |
| 9.3 Electrostatic discharge | Completed |  |
| 9.4 Fast transient common mode | Completed |  |
| 9.5 RF common mode (0.15 MHz - 80 MHz) | Completed |  |
| 9.6 Voltage dips and interruptions | Completed |  |
| 9.7 Surges, common and differential mode | Completed |  |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on 6 GHz limit for the Radiated Immunity testing | Huawei |  |
|  | WF on the criteria for performance assessment for NR Repeater EMC testing | ZTE |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-210xxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2207894 |  | TS 38.175: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson | To be revised |  |
| R4-2210044 |  | TS 38.175: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2207895 |  | TS 36.113: Corrections in clause 9 Immunity | Ericsson | To be revised |  |
| R4-2207896 |  | TS 36.113: Corrections in clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2210045 |  | TS 36.113: Corrections in clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2207897 |  | TS 37.113: Corrections in clause 9 Immunity | Ericsson | To be revised |  |
| R4-2207898 |  | TS 37.113: Corrections in clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2210046 |  | TS 37.113: Corrections in clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2207899 |  | TS 37.114: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson | To be revised |  |
| R4-2207900 |  | TS 37.114: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2210047 |  | TS 37.114: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2207901 |  | TS 38.113: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson | To be revised |  |
| R4-2207902 |  | TS 38.113: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2210048 |  | TS 38.113: Corrections in clause 1 Scope and clause 9 Immunity | Ericsson |  | Mirror CR |
| R4-2208380 |  | Discussion on highest frequency and measurement uncertainty for NR BS radiated emission test with big size EUT | ZTE Corporation | noted |  |
| R4-2208382 |  | Draft CR to TS 38.113 Radiated emission measurement uncertainty (R15) | ZTE Corporation | Return to |  |
| R4-2208384 |  | Draft CR to TS 38.113 Radiated emission measurement uncertainty (R16) | ZTE Corporation |  | Mirror CR |
| R4-2208386 |  | Draft CR to TS 38.113 Radiated emission measurement uncertainty (R17) | ZTE Corporation |  | Mirror CR |
| R4-2209653 |  | CR to TS 34.124: corrections of the UTRA UE EMC specification, Rel-17 | Huawei, HiSilicon | Agreeable |  |
| R4-2209654 |  | Draft CR to TS 38.124: addition of the missing Rx spurious emissions limits for idle mode testing, Rel-15 | Huawei, HiSilicon, Samsung | Return to |  |
| R4-2209655 |  | Draft CR to TS 38.124: addition of the missing Rx spurious emissions limits for idle mode testing, Rel-16 | Huawei, HiSilicon, Samsung |  | Mirror CR |
| R4-2209656 |  | Draft CR to TS 38.124: addition of the missing Rx spurious emissions limits for idle mode testing, Rel-17 | Huawei, HiSilicon, Samsung |  | Mirror CR |
| R4-2209657 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-8 | Huawei, HiSilicon | Return to | Moderator note: To MCC (ask for guidance) : Whether or not it is allowed to submit CRs for the earlier releases (such as R8) which frozen for many years? |
| R4-2209658 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-9 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209659 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-10 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209660 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-11 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209661 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-12 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209662 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-13 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209663 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-14 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209664 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-15 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209665 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-16 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209667 |  | Draft CR to TS 36.124: correction of the Rx spurious exclusion band (band-agnostic), Rel-17 | Huawei, HiSilicon |  | Mirror CR |
| R4-2209666 |  | CR to TS 38.175: updates reflecting modifications in IEC 61000-4-3:2020 for the upper frequency range of the RI test | Huawei, HiSilicon | Return to |  |
| R4-2209668 |  | Further analysis of the updated IEC 61000-4-3:2020 specification: upper frequency range for radiated immunity requirements | Huawei, HiSilicon | noted |  |
| R4-2209669 |  | CR to TS 38.113: updates reflecting modifications in IEC 61000-4-3:2020 for the upper frequency range of the RI test | Huawei, HiSilicon | Return to |  |
| R4-2209670 |  | CR to TS 38.124: updates reflecting modifications in IEC 61000-4-3:2020 for the upper frequency range of the RI test, Rel-17 | Huawei, HiSilicon | Return to |  |
| R4-2209671 |  | CR to TS 38.114: updates reflecting modifications in IEC 61000-4-3:2020 for the upper frequency range of the RI test, Rel-17 | Huawei, HiSilicon | Return to |  |
| R4-2208379 |  | Discussion on performance assessment and work split of NR Repeater EMC | ZTE Corporation | noted |  |
| R4-2209140 |  | On TDD NR repeater EMC testing | Nokia, Nokia Shanghai Bell | noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents