3GPP TSG-RAN WG4 Meeting #103-e	R4-2210329

# Electronic Meeting, 9th – 20th May, 2022

Agenda item:			9.12.8.2

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [103-e][323] NR_NTN_Demod_Part2

Document for:	Information

Introduction

The summary covers the contributions submitted under the following agendas

- 9.12.8.2 - Satellite Access Node demodulation requirements

  - 9.12.8.2.1 - PUSCH requirements

  - 9.12.8.2.2 - PUCCH requirements

  - 9.12.8.2.3 - PRACH requirements

Topic #1: Satellite Access Node demodulation requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2208015 | Discussion on PUSCH requirement for SAN demodulation | Ericssion | Observation 1: The remaining timing error could be very small in MsgA of 2-step RACH regarding UE pre-compensation precision. Observation 2: Only very limited PRBs could achieve targe SNR based on link budget. Proposal 1: It is better to define the SAN PUSCH requirement for a few of PRB allocation. Observation 3: 2Rx would be better for SAN demodulation requirement than 1Rx regarding to less additional loss caused by possible depolarization. Proposal 2: Define SAN PUSCH demodulation requirement for 1Tx2Rx and 1Tx1Rx. Only take LOS channel for 1Tx1Rx requirements. Observation 4: The decision of MCS depends on the simulation with agreed channel model, but the proper channel model should be investigated at the first. |
| R4-2208016 | Discussion on PUCCH requirement for SAN demodulation | Ericssion | Proposal 1: Follow the agreement on SAN PUSCH demodulation discussion. • If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths. • If full bandwidth can be used for UL, only requirements for minimum and maximum bandwidths could be introduced. The Rel-15 FR1 PUCCH applicability rule on bandwidth can be reused. Proposal 2: Define SAN PUCCH requirements for 1Tx2Rx and 1Tx1Rx. Only take LOS channel model for 1Tx1Rx requirements. Proposal 3: The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part. Proposal 4: If the same BS manufactory declaration in Rel-15 would be applied for SAN, PUCCH format 3/4 requirements should be defined for both with and without additional DM-RS to avoid no requirement situation. |
| R4-2208017 | Discussion on PRACH requirement for SAN demodulation | Ericssion | Proposal 1: Use the same NLOS multi-path fading channel model as PUSCH. Proposal 2: Define SAN PRACH demodulation requirements for: Option 1: preamble format 1 and all short formats. Option 2: If down selection can be agreed, then consider format 0, A2, B4 and C2. Proposal 3: Define SAN PRACH demodulation requirements for 1Tx2Rx. Proposal 4: Define SAN PRACH demodulation requirements for 1.25kHz, 15kHz and 30kHz SCS. Proposal 5: Define SAN PRACH demodulation requirements with 400Hz frequency offset. Proposal 6: Use same time error tolerance of AWGN as Rel-15 FR1 for 15kHz and 30kHz SCS. Use the same method to calculate time error tolerance of multi-path channel as Rel-15. Proposal 7: Use the same Ncs as Rel-15 normal PRACH requirement. |
| R4-2208085 | View on NTN SAN demodulation requirement | Samsung | Proposal 1: No 2-step RACH case is considered for SAN requirement Proposal 2: RAN4 define SAN requirement with 5MHz/20MHz for 15KHz SCS, and 5MHz/20MHz for 30KHz SCS. Proposal 3: RAN4 define SAN requirement with 2 Rx only Proposal 4: RAN4 define SAN requirement with CP-OFDM only Proposal 5: RAN4 define SAN PRACH requirement with format 0, FFS on format 1 and format 2 |
| R4-2208878 | Discussion on PUSCH demodulation requirements for NTN | Nokia, Nokia Shanghai Bell | Observation 1 : A few PRBs allocation is the most commonly used in the considered NTN applications. Observation 2 : Considering different CBW with the same PRB allocation have similar results. Proposal 1 : Consider 4 PRBs allocation for all SCS. Observation 3 : The most common SAN antenna arrays is 2 Rx. Observation 4 : Doubling Rx antennas at SAN Rx provide a predictable receive diversity gain. Proposal 2 : Consider UE 1 Tx and SAN 1Rx, SAN 2 Rx. Proposal 3: Use parameters in Table 1 for NTN PUSCH demodulation requirements. Observation 5: The degradation with 0.1 ppm residual frequency error is negligible with MCS4 for all configuration even with SAN 1 Rx. Observation 6: Results with SAN 1 Rx, especially with NTN TDL-A, require a high SNR even with MCS4 that could not be available for some cases. Important gain is achieved with SAN 2 Rx receive diversity especially in NLOS NTN-TDL-A. Observation 7: For each channel profile, the 1% BLER and 70% throughput with MCS4 and same PRB allocation do not differ significantly between the different SCS/CBW. |
| R4-2209877 | Discussion on satellite NTN demod PUSCH | Huawei, HiSilicon | Proposal 1: For NTN PUSCH, define requirements for 2-step RA type requirement. Proposal 2: Only define 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz for NTN PUSCH performance requirements. Proposal 3: Also consider SAN 4Rx and 8Rx for NTN PUSCH requirements definition. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration. |
| R4-2209878 | Simulation results on satellite NTN demod PUSCH | Huawei, HiSilicon | Simulation results for PUSCH |
| R4-2209879 | Discussion on satellite NTN demod PUCCH | Huawei, HiSilicon | For NTN PUCCH performance requirements definition, use the test configuration as shown in Table 3-1 to Table 3-7. Only define 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz for NTN PUCCH performance requirements. Also consider SAN 4Rx and 8Rx for NTN PUCCH requirements definition except multi-slot PUCCH. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration. |
| R4-2209880 | Simulation results on satellite NTN demod PUCCH | Huawei, HiSilicon | Simulation results for PUCCH |
| R4-2209881 | Discussion on satellite NTN demod PRACH | Huawei, HiSilicon | - For NTN PRACH requirements, use the following test configuration. Parameter; Value. Preamble format; A2/B4/C2. Antenna configuration; 2/4/8 Rx for LRA=139, 2Rx for LRA=1151 and 571. SCS; 15kHz for LRA=139 and 1151, 30kHz for LRA=139 and 571. Propagation; AWGN, NTN-TDLA. Time error tolerance; 0.522μ+Tdelay (Same formula as legacy BS requirements). Doppler; 200Hz. Delay spread; 250ns. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration. |
| R4-2209882 | Simulation results on satellite NTN demod PRACH | Huawei, HiSilicon | Simulation results for PRACH |
| R4-2209681 | TP to TS 38.108: remaining annexes for FRC (SAN demodulation requirements) | Huawei, HiSilicon | Postponed to future RAN4 meetings after we have stable enough progress on performance part as per chairman’s guidance |

Open issues summary

Background information:

As per the approved WF R4-2207198, the corresponding agreements made in last RAN4#102-e meeting are copied under each open issues with square bracket for convenience.

Sub-topic 1-1 PUSCH

Issue 1-1-1: Scope of PUSCH requirements

Candidate options - Option 1: Do not consider 2 step RACH case - Option 2: Consider the 2 step RACH case

- Proposals

  - Option 1 (Huawei): Consider the 2 step RACH case

  - Option 2 (Samsung): Not consider 2-step RACH case

- Recommended WF

  - TBA

Issue 1-1-2: SCS/CBW set for PUSCH requirements

Candidate options - Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz - Option 2: a few of PRBs for all SCS

- Proposals

  - Option 1 (Huawei): Only 5MHz for 15kHz SCS and 10MHz for 30kHz SCS

  - Option 2 (Nokia): 4 PRBs allocation for all SCS

  - Option 3 (Ericsson): A few of PRBs allocation

  - Option 4 (Samsung): 5MHz/20MHz for 15KHz SCS, and 5MHz/20MHz for 30KHz SCS

- Recommended WF

  - TBA

Issue 1-1-3: Antenna configuration for PUSCH requirements

Agreements - UE 1Tx - SAN 2Rx Candidate options - FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

- Proposals

  - Option 1 (Huawei):

    - Also consider SAN 4Rx and 8Rx for NTN PUSCH requirements definition.

    - Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

  - Option 2 (Nokia, Ericsson): Consider UE 1 Tx and SAN 1Rx, SAN 2 Rx.

    - Option 2a (Ericsson): Only take LOS channel for 1Tx1Rx requirements.

  - Option 3 (Samsung): Only consider 2 Rx

- Recommended WF

  - TBA

Issue 1-1-4: MCS

Agreements - Select MCS4 for PUSCH requirements

- Proposals

  - Option 1 (Ericsson): The decision of MCS depends on the simulation with agreed channel model, but the proper channel model should be investigated at the first.

  - Other options

- Recommended WF

  - TBA

Issue 1-1-5: Waveform

- Proposals

  - Option 1 (Samsung): CP-OFDM only.

  - Option 2 (Nokia): DFT-s-OFDM

  - Other options

- Recommended WF

  - TBA

Issue 1-1-6: Test configuration for PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

- Recommended WF

  - Companies are encouraged to check whether parameters in the above table are agreeable?

Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | Same as normal PUSCH |
| MCS |  | MCS4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Same as normal PUSCH |
|  | Starting PRB index | Based on RB assignment |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | Based on RB assignment |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: Same as PUSCH for NTN-TDLA |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

- Recommended WF

  - Companies are encouraged to check whether parameters in the above table are agreeable?

Issue 1-1-8: Test configuration for PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel model |  | NTN-TDL-A, Doppler and delay same as normal PUSCH |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource assignment | RB assignment | Same as normal PUSCH |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

- Recommended WF

  - Companies are encouraged to check whether parameters in the above table are agreeable?

Sub-topic 1-2 PUCCH

Issue 1-2-1: Scope of PUCCH requirements

Agreements - In addition to PUCCH format 0/1/2/3/4, RAN4 to define NTN multi-slot PUCCH demodulation requirements - Prioritize UCI with HARQ on PUCCH demodulation requirement

- Proposals

  - Option 1 (Huawei):

    - For PUCCH format 2/3/4, addition note should be added that The UCI information does not contain CSI part 1 and CSI part 2.

    - For PUCCH format 0, select 2 symbol case from legacy BS requirements.

    - For PUCCH format 2, select both two cases with different UCI information bits from legacy BS requirements

    - For PUCCH format 3, only select 1 PRB with 14 symbol case from legacy BS requirements

    - For PUCCH format 3/4, only select additional DMRS configuration disabled case from legacy BS requirements

  - Option 2 (Ericsson):

    - If the same BS manufactory declaration in Rel-15 would be applied for SAN, PUCCH format 3/4 requirements should be defined for both with and without additional DM-RS to avoid no requirement situation.

- Recommended WF

Issue 1-2-2: SCS/CBW set for PUCCH requirements

Agreements - To follow the same SCS/CBW set as PUSCH as the start point Candidate options - Option 1: No need to reduce test - Option 2: Need to reduce test cases (specify if any)

- Proposals

  - Option 1 (Ericsson):

    - If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths.

    - If full bandwidth can be used for UL, only requirements for minimum and maximum bandwidths could be introduced. The Rel-15 FR1 PUCCH applicability rule on bandwidth can be reused.

    - The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part

  - Option 2 (Huawei): Only 5MHz for 15kHz SCS and 10MHz for 30kHz SCS.

- Recommended WF

  - TBA

Issue 1-2-3: Antennal configuration

Agreements - UE 1Tx - SAN 2Rx Candidate options - FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

- Proposals

  - Option 1 (Ericsson):

    - Define SAN PUCCH requirements for 1Tx2Rx and 1Tx1Rx.

    - Only take LOS channel model for 1Tx1Rx requirements.

  - Option 2 (Huawei):

    - Also consider SAN 4Rx and 8Rx for NTN PUSCH requirements definition.

    - Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

- Recommended WF

  - TBA

Issue 1-2-4: Channel model

- Proposals

  - Option 1 (Ericsson): PUCCH could take the same channel as PUSCH.

  - Other options.

- Recommended WF

- Companies are encouraged to check whether Option 1 is OK?

Sub-topic 1-3 PRACH

Issue 1-3-1: Preamble Format

- Proposals

  - Option 1 (Ericsson):

    - Option 1a: Preamble format 0 and all short formats

    - Option 1b: Preamble format 0, A2, B4 and C2

  - Option 2 (Huawei): Preamble format A2, B4 and C2

  - Option 3 (Samsung): Preamble format 0, FFS on format 1 and format 2

- Recommended WF

  - TBA

Issue 1-3-2: SCS

- Proposals

  - Option 1 (Ericsson): 1.25kHz, 15kHz and 30kHz SCS.

  - Option 2 (Huawei): 15kHz for LRA=139 and 1151, 30kHz for LRA=139 and 571.

- Recommended WF

  - For short preamble format (if agreed), define SAN PRACH demodulation requirements for 15kHz and 30kHz SCS.

  - FFS 1.25kHz based on discussion on Issue 1-3-1.

Issue 1-3-3: Sequence length for short preamble format

- Proposals

  - Option 1 (Huawei): LRA=139 and 1151 for 15kHz, LRA=139 and 571 for 30kHz.

  - Other options

- Recommended WF

  - TBA

Issue 1-3-4: Antenna configuration

- Proposals

  - Option 1 (Ericsson): 1Tx2Rx

  - Option 2 (Huawei):

    - 2/4/8Rx for LRA=139

    - 2Rx for LRA=1151 and 571

- Recommended WF

  - TBA

Issue 1-3-5: Time error tolerance

- Proposals

  - Option 1 (Ericsson):

    - Use same time error tolerance of AWGN as Rel-15 FR1 for 15kHz and 30kHz SCS.

    - Use the same method to calculate time error tolerance of multi-path channel as Rel-15.

  - Option 2 (Huawei): 0.522μ+Tdelay where Tdelay is corresponding to the tap with the second largest delay of the propagation delay channel. (Same formula as legacy BS requirements)

- Recommended WF

  - Use same time error tolerance of AWGN as Rel-15 FR1 for 15kHz and 30kHz SCS.

  - Use the same method to calculate time error tolerance of multi-path channel as Rel-15: 0.522μ+Tdelay where Tdelay is corresponding to the tap with the largest or the second largest delay of the propagation delay channel

    - Company needs to confirm the Tdelay used in Rel-15: the largest or the second largest delay is used?

Issue 1-3-6: Frequency offset for PRACH Channel model

- Proposals

  - Option 1 (Ericsson):

    - Use the same NLOS multi-path fading channel model as PUSCH.

    - Define SAN PRACH demodulation requirements with 400Hz frequency offset.

  - Option 2 (Huawei): AWGN with 0Hz frequency offset, NTN-TDLA100-200 with 200Hz frequency offset

- Recommended WF

  - Channel model related open issues are recommended to be discussed in email thread [103-e][322] NR_NTN_Demod_Part1 together for both DL and UL, this open issue will be focused on frequency offset for PRACH

    - AWGN with 0Hz frequency offset

    - FFS for fading channel

Issue 1-3-7: Ncs

- Proposals

  - Option 1 (Ericsson): Use the same Ncs as Rel-15 normal PRACH requirement.

  - Other options.

- Recommended WF

  - Companies are encouraged to confirm whether Option 1 is OK based on the PRACH formats to be agreed?

Companies views’ collection for 1st round

Open issues

Sub topic 1-1 PUSCH

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: Scope of PUSCH requirements Issue 1-1-2: SCS/CBW set for PUSCH requirements Issue 1-1-3: Antenna configuration for PUSCH requirements Issue 1-1-4: MCS Issue 1-1-5: Waveform Issue 1-1-6: Test configuration for PUSCH Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment Issue 1-1-8: Test configuration for PUSCH repetition type A |
| Thales | Issue 1-1-1: Scope of PUSCH requirements The 2-step RACH is an important feature for NTN to cope with the substantial propagation delay. We support including 2-step RACH in the scope of PUSCH requirements for Msg3 transmission. Support Option 1. Issue 1-3-2: SCS Few PRBs is more representative of NTN PUSCH allocations but it does not preclude testing higher bandwidth allocation if deemed necessary. Support Option 3 and optionally Option 4. Issue 1-1-3: Antenna configuration for PUSCH requirements We do not recall RAN4 to agree on considering SAN 2 RX as the baseline configuration. At least support SAN 1 RX (most common satellite configuration). Other configuration may be considered. Support Option 2. Issue 1-1-4: MCS Option 1 Issue 1-1-5: Waveform Support Option 2. Since NTN UL link budget may be rather challenging even with 1 TX, we support considering DFT-s OFDM to maximize the terminal eirp. Issue 1-1-6: Test configuration for PUSCH No comments Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment UE speed : Our understanding is that airplaine and HST connectivity shall also be supported (see Table 7.4.2-1 from TS 22.261). Thus, UE speed up to 1000 km/h should also be considered (but it is also true that a dedicated channel model may also be required for at least airplane connectivity). |
| Ericsson | Issue 1-1-1: Scope of PUSCH requirements The 2-step RACH is benefit for NTN deployment to reduce the access time when channel condition is good enough, such LOS for LEO. RAN2 have agreement that 2-step RACH can be supported for NTN. But we think a separate requirement for 2-step RACH seems not so necessary considering UE pre-compensation. The timing error could be very small from different NTN UE, and the MsgA PUSCH would be the as normal PUSCH. In that case, only normal PUSCH requirement would be enough. Issue 1-1-2: SCS/CBW set for PUSCH requirements For UL, we think it is not typical for UE to transmit on full bandwidth based on the link budget analysis. A few of PRB would be suitable for the requirement. From the demodulation point of view, the performance would be similar between small PRBs and full bandwidth. In addition, only a few of PRB requirements could be applied for all bandwidth per SCS. We can accept Option 2 to use 4 PRBs as baseline. Issue 1-1-3: Antenna configuration for PUSCH requirements The SAN antenna should refer to satellite antenna not GW antenna. All relevant tests and requirements are defined on the satellite antenna RIB. So typical satellite antenna configuration 1Rx/2Rx should be considered for SAN requirement. To save simulation and test effort, we suggest only consider 1Tx1Rx for LOS channel regarding no combination gain and -3dB depolarization. Issue 1-1-4: Channel model (to be discussed in email thread [322]) Support recommended WF. Issue 1-1-5: MCS We can accept MCS4 as the start point. Issue 1-1-6: Waveform We support Option 1 to only consider CP-OFDM because there is very small performance difference from DFT-s-OFDM based on previous simulations. The few of PRB transmission from UE and very large cell range would typically allow multiple UE access. Issue 1-1-7: Test configuration for PUSCH We are OK with the configurations but open for further discussion. Issue 1-1-8: Test configuration for PUSCH with UL timing adjustment RB allocation should depend on the PUSCH CBW agreement. Issue 1-1-9: Test configuration for PUSCH repetition type A We can accept configurations in the table for initial simulation. |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: Scope of PUSCH requirements Support Option 2: Not consider 2-step RACH case as timing erorr is small and it is not necessary to define additional PUSCH demod requirements. Issue 1-1-2: SCS/CBW set for PUSCH requirements Support Option 2, and we are open to discuss other small fixed PRB allocation for all SCS. Issue 1-1-3: Antenna configuration for PUSCH requirements Support Option 2 as 2 RX SAN antennas is the most common and higher Rx antenna if any provide a predictable diversity gain. Issue 1-1-4: MCS The MCS4 agreement is reasonable for any channel and enough for NTN PUSCH demodulation requirements. Issue 1-1-5: Waveform Support option 2, NTN UL is coverage limited scenario and energy efficient waveform should be used. Using DFT-s-OFDM allows to increase the transmit power compared to CP-OFDM due to lower PAPR (smaller MPR), and thus enhance the coverage even if BLER performance is similar for both waveforms. Issue 1-1-6: Test configuration for PUSCH Yes, we are Ok with the proposed parameters. Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment Most of the parameters are agreeable, and we propose to update the following: - No need to change the DMRS position from previous table. -Transform precoding should follow PUSCH discussion and agreement (same as normal PUSCH). Issue 1-1-8: Test configuration for PUSCH repetition type A We are okay with this table and we propose to modify the following: - Use MCS4 as the previous agreement (Same as normal PUSCH) - Transform precoding to follow above discussions and agreement (Same as normal PUSCH). |
| Samsung | Issue 1-1-1: Scope of PUSCH requirements We support option2, the pre-compensation of timing and frequency offset can be performed at UE side for UL transmission, the residual timing offset impact will be minor. Existing 2-step RACH requirements can be applied. There is no necessary to define additional requirement with 2-step RACH Issue 1-1-2: SCS/CBW set for PUSCH requirements We prefer option 3 with taking the test effort into account and aligning the FRC discussed for Rx requirement Issue 1-1-3: Antenna configuration for PUSCH requirements 2Rx can meet the basic test purpose, therefore, we prefer to consider 2Rx only for SAN requirement with reducing test effort, Issue 1-1-4: MCS Issue 1-1-5: Waveform There is no restriction NR operation in non-terrestrial networks about waveform, meanwhile the performance difference is minor, we prefer to focus on CP-OFDM firstly, Issue 1-1-6: Test configuration for PUSCH Ok with option 1 as start point Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment Ok with option1 as start point Issue 1-1-8: Test configuration for PUSCH repetition type A Ok with option1 as start point |
| Huawei | Issue 1-1-1: Scope of PUSCH requirements We think 2-step RACH can be used in NTN scenario. However, as some companies pointed out, different from the legacy BS, there is no timing offset cycling configured since it is pre-compensated by all UEs. Then it is same as the legacy PUSCH from the demodulation point of view. Considering the 2-step RACH performance can be ensured by normal PUSCH requirement, we are OK to not introduce 2-step RACH PUSCH requirements for NTN. Issue 1-1-2: SCS/CBW set for PUSCH requirements We prefer Option1 to consider small PRB allocation considering the link budget and also the test effort. As per our evaluation, Option 1 is feasible at least for LEO600 scenario. Issue 1-1-3: Antenna configuration for PUSCH requirements We prefer Option 1 to give manufacture more flexible to only test the configurations they support. Applicability can be defined that only highest of supported Rx number shall be tested based on manufacture declaration rather than both lowest and highest. Issue 1-1-4: MCS We prefer to keep the previous agreement. Issue 1-1-5: Waveform We prefer Option 1 considering that there is negligible performance difference between CP-OFDM and DFT-s-OFDM as per legacy BS requirements. Issue 1-1-6: Test configuration for PUSCH We are OK with these parameters to align with the legacy TN BS requirements. Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment We are OK with these parameters to align with the legacy TN BS requirements. For the UE speed, considering UE pre-compensation, there is not large timing for adjustment. We think 120km/h is enough to verify satellite TA implementation from demodulation point of view. For the DMRS position, pos2 is considered for the legacy BS requirements since it is discussed in HST WI. However, considering there is no Doppler shift modeled, we are also OK to consider pos1 for NTN UL TA requirements. For the transform precoding, we prefer to only consider CP-OFDM to align with the legacy BS TA requirements. Issue 1-1-8: Test configuration for PUSCH repetition type A We are OK with these parameters to align with the legacy TN BS requirements. For the MCS, we prefer to consider MCS 5 in Table 3 that is same as legacy TN BS requirements. For the transform precoding, we prefer to only consider CP-OFDM to align with the legacy BS TA requirements. |

Sub topic 1-2 PUCCH

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1: Scope of PUCCH requirements Issue 1-2-2: SCS/CBW set for PUCCH requirements Issue 1-2-3: Antennal configuration Issue 1-2-4: Channel model |
| Thales | Issue 1-2-1: Scope of PUCCH requirements Support Option 2 Issue 1-2-2: SCS/CBW set for PUCCH requirements Support Option 1 Issue 1-2-3: Antennal configuration We do not recall RAN4 to agree on considering SAN 2 RX as the baseline configuration. At least support SAN 1 RX (most common satellite configuration). Other configuration may be considered. Support Option 1 Issue 1-2-4: Channel model Support Option 1 |
| Ericsson | Issue 1-2-1: Scope of PUCCH requirements Basically, we think it would be good to keep same test coverage as legacy BS requirement. For PF0/2 and PF3 PRB/symbol, Option 1 configurations could be applied. For DM-RS in PF3/4, we suggest companies check if the same declaration as Rel-15 will be applied for SAN. If yes, it might be good to define requirements to cover both 1+0 and 1+1 to avoid no requirement condition. Issue 1-2-2: SCS/CBW set for PUCCH requirements We suggest considering following option which is based on Option 1 and 2. o Option 1a:  If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths.  If full bandwidth can be used for UL, only requirements for minimum bandwidths could be introduced. The Rel-15 FR1 PUCCH applicability rule on bandwidth can be reused.  The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part Issue 1-2-3: Antennal configuration Same comments as PUSCH part. PUCCH could follow PUSCH approach. Issue 1-2-4: Channel model Agree with Option 1. |
| Nokia, Nokia Shanghai Bell | Issue 1-2-2: SCS/CBW set for PUCCH requirements Support option 1. Issue 1-2-3: Antennal configuration Support option 1, for the same reason as in PUSCH Issue 1-1-3, we can follow PUSCH discussion and agreement. Issue 1-2-4: Channel model Support option 1. |
| Huawei | Issue 1-2-1: Scope of PUCCH requirements We prefer Option 1. Issue 1-2-2: SCS/CBW set for PUCCH requirements We think this issue depends on the SCS/CBW for PUSCH requirements. Currently we prefer to reuse the same value as the legacy BS requirements. Further modification is needed if the PRB number for PUCCH is larger than that of PUSCH requirements. Issue 1-2-3: Antennal configuration We are OK to align with the PUSCH. Issue 1-2-4: Channel model We are OK with Option 1. |

Sub topic 1-3 PRACH

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1: Preamble Format Issue 1-3-2: SCS Issue 1-3-3: Sequence length for short preamble format Issue 1-3-4: Antenna configuration Issue 1-3-5: Time error tolerance Issue 1-3-6: Frequency offset for PRACH Channel model Issue 1-3-7: Ncs |
|  | Issue 1-3-1: Preamble Format At least one long format should be included. The NTN link budgets can be rather limited so it is valuable to get a preamble configuration with limited bandwidth occupation (assuming the frequency error can be handheld thanks to UE pre-compensation). Support Option 1a or 1b Issue 1-3-2: SCS Support Option 1 Issue 1-3-3: Sequence length for short preamble format 20 MHz bandwidth allocation can be quite challenging in terms of link budget for preamble transmission in NTN. As a consequence, LRA = 139 may be prioritized. Issue 1-3-4: Antenna configuration Option 1 and 2 are not acceptable. At least support SAN 1 RX (most common satellite configuration). Other configuration can also be considered. Issue 1-3-5: Time error tolerance Support recommended WF Issue 1-3-6: Frequency offset for PRACH Channel model If a frequency estimation error of 0.1 ppm is made in DL RX (i.e. 200 Hz @2GHz), then a transmission frequency error of 400 Hz may be expected on the UL for PRACH transmission. Issue 1-3-7: Ncs Support Option 1 |
| Ericsson | Issue 1-3-1: Preamble Format Regarding UE pre-compensation, the timing error could be very small between different UEs. In that case, all preamble formats could be possibly applied. In that case, we propose consider Option 1 to cover long and short preamble formats to secure the test coverage. But we don’t have strong opinion on this issue. Currently, we slightly prefer 1b. Issue 1-3-2: SCS If long preamble formats are introduced, 1.25kHz SCS should be considered, otherwise, only 15/30kHz SCS are considered. Support recommended WF. Issue 1-3-3: Sequence length for short preamble format We also think long sequence (Lra=571/1151) would be challenging for NTN scenario. Short sequence Lra=139 should be the base line. Issue 1-3-4: Antenna configuration We are OK to consider both 1Rx and 2Rx. Issue 1-3-5: Time error tolerance We support recommended WF. For T_delay, we think it should be the largest non-neglectable delay which has relative high power. In Rel-15, the largest tap power in TDLA is lower than -26dB which could be neglected compared with other taps, but the largest tap power in NTN-TDLA is -6.48dB which seems non-neglectable. Issue 1-3-6: Frequency offset for PRACH Channel model We support recommended WF. Issue 1-3-7: Ncs We support Option 1. |
| Nokia, Nokia Shanghai Bell | Issue 1-3-1: Preamble Format Support option 3, as NTN is coverage limited scenario, and the long format could be mostly used If short formats are required to be included in the requirements, we support option 1b. Issue 1-3-2: SCS The proposed WF is fine for us. Issue 1-3-3: Sequence length for short preamble format If short format is agreed, we are Ok with option 1. Issue 1-3-4: Antenna configuration Support option 1 for the same reasons (predictable Rx diversity gain, and typical SAN antenna configuration) as other sub-topic issues. Issue 1-3-6: Frequency offset for PRACH Channel model Support option 2, we can use as PUSCH 200Hz maximum frequency offset: Consider 200Hz as the maximum Doppler shift for UL in service link Issue 1-3-7: Ncs Support option 1. |
| Samsung | Issue 1-3-1: Preamble Format Since the common propagation delay can be compensated, then the NTN PRACH signal design will depend on the relative propagation delay, which is limited up to 200km in current specifications regarding TA range, we prefer with long format, format 0 can be applied, If short format is considered, we support with only C2 Issue 1-3-2: SCS Pending on the format agreed Issue 1-3-3: Sequence length for short preamble format Not sure whether long sequence should be considered for short format Issue 1-3-4: Antenna configuration We support with option 1, Issue 1-3-5: Time error tolerance Ok with the time error tolerance of AWGN, For fading channel, if rel-15 applied, it should be the second largest delay, we are also to support the second largest delay Issue 1-3-6: Frequency offset for PRACH Channel model Pending on frequency offset discussed for UL in service link. We are ok with option 2, by assuming UE pre-compensation for UL, the residual frequency shift is 0.1 ppm Issue 1-3-7: Ncs Ok with option 1 as starting point |
| Huawei | Issue 1-3-1: Preamble Format We prefer Option 2. Considering there is very small residual frequency and timing error after UE pre-compensation, we think all preamble formats are feasible. We prefer to select A2, B4 and C2 that is more typical. Issue 1-3-2: SCS We are OK with the recommended WF. Issue 1-3-3: Sequence length for short preamble format We are OK to only consider LRA=139 for both 15kHz and 30kHz to move forward. Issue 1-3-4: Antenna configuration If only LRA=139 is considered, we prefer to align with PUSCH. Issue 1-3-5: Time error tolerance The issue is that whether the last tap NTN-TDL-A propagation condition is negligible. We slightly prefer to consider the second largest delay for Tdelay calculation but we don’t have strong view. Issue 1-3-6: Frequency offset for PRACH Channel model We prefer to consider 200Hz for frequency offset that is 0.1ppm of fc. Issue 1-3-7: Ncs We are OK with Option 1. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 PUSCH | Issue 1-1-1: Scope of PUSCH requirements Moderator: As pointed by several companies, due to the timing and frequency offset pre-compensation at the UE side, the timing error at the BS side could be very small, it is not necessary to set the timing offset cycling for the testing, i.e. it is same as the normal PUSCH performance requirements, no additional requirement is needed for MsgA in 2-step RACH. Moderator would like to check if it is acceptable for all companies not to consider 2-step RACH case for NTN. Tentative agreements: - Not consider 2-step RACH case for SAN performance requirements Candidate options - Option 1 (Huawei, Thales): Consider the 2 step RACH case - Option 2 (Samsung, Huawei, Ericsson, Nokia): Not consider 2-step RACH case Recommendations for 2nd round: - Company can raise concern on the recommended tentative agreements if any. Issue 1-1-2: SCS/CBW set for PUSCH requirements Moderator: All companies are fine with small number of PRB, but from demodulation performance requirement definition point of view, specific number of PRB needs to be specified, so proponent of Option 3 is encouraged to share the preferred number of PRB in the 2nd round. To facilitate the 2nd round discussion, moderator list the following candidate options for further discussion. Tentative agreements: - One test cases with few PRBs for each SCS - Reuse Rel-15 test applicability rule defined in clause 8.1.2.1.2 Applicability of requirements for different channel bandwidths Candidate options - Option 1: 5MHz/15kHz (i.e. 25 PRBs) and 10MHz/30kHz (i.e. 24 PRBs) - Option 2: 4 PRBs for all SCS Recommendations for 2nd round: - Continue the discussion on the specific number of PRBs. Issue 1-1-3: Antenna configuration for PUSCH requirements Moderator: As per agreements reached in last RAN4#102-e (R4-2207198), UE 1Tx and SAN 2Rx have been agreed, whether additional 1Rx, 4Rx and 8Rx need to be considered. As per the 1st round discussion, companies still have diverse views. Companies are encouraged to give compromise to move forward in the 2nd round discussion. Tentative agreements: N/A Candidate options for additional number of Rx antenna configuration besides 2Rx: - Option 1 (Huawei): 4Rx and 8Rx with test applicability rule - Only the highest supported Rx number shall be tested based on manufacture declaration. - Option 2 (Nokia, Ericsson, Thales): 1Rx. - Option 2a (Ericsson): Only take LOS channel for 1Tx1Rx requirements. - Option 3 (Samsung): Only 2 Rx Recommendations for 2nd round: - Continue the discussion. Issue 1-1-4: MCS Moderator: MCS4 is agreed in last meeting, considering the low code rate and investigation by companies in previous meeting, moderator suggest to keep MCS4 agreement, but if any technical issues are figured out later based on the agreed channel model, further updates are not precluded. Tentative agreements: MCS4 unless any technical issues figured out Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-1-5: Waveform Moderator: Companies still have diverse view on the waveform to be used for the performance requirements definition, further discussion is needed. Tentative agreements: N/A Candidate options: - Option 1 (Samsung, Ericsson, Huawei): CP-OFDM only. - Option 2 (Nokia, Thales): DFT-s-OFDM only Recommendations for 2nd round: - Continue the discussion. Issue 1-1-6: Test configuration for PUSCH Moderator: All companies are fine with the following parameters. Considering only UE 1Tx is considered, the test configuration “TPMI index for 2Tx two-layer spatial multiplexing transmission” need to be removed. Tentative agreements: Agree to use the following test parameters for NTN PUSCH performance requirements definition unless any issues figured out Parameter; Value. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port; {0}. DM-RS sequence generation; NID0=0, nSCID =0, group hopping and sequence hopping are disabled. Time domain; PUSCH mapping type; A, B. resource; Start symbol; 0. assignment; Allocation length; 14. Frequency domain resource assignment; Frequency hopping; Disabled. TPMI index for 2Tx two-layer spatial multiplexing transmission; 0. Code block group based PUSCH transmission; Disabled. Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-1-7: Test configuration for PUSCH with UL timing adjustment Moderator: Companies are fine with most of the test parameters listed, except the UE speed, number of RB allocated, additional DM-RS configuration and waveform that can be further discussed in the 2nd round. Tentative agreements: Agree to use the following test parameters for NTN PUSCH UL TA performance requirements definition unless any issues figured out Parameter; Value. Channel bandwidth; Same as normal PUSCH. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. DM-RS position (l0); 2. Additional DM-RS position; pos2. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port; {0}. Time domain resource assignment; DM-RS sequence generation; NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE. PUSCH mapping type; Both A and B. Allocation length; 14. Frequency domain resource assignment; RB assignment; Depends on normal PUSCH. Starting PRB index; Based on RB assignment. Frequency hopping; Disabled. SRS resource allocation; Slots in which sounding RS is transmitted (Note 1); slot #1 in radio frames. SRS resource allocation; Based on RB assignment. NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively.. Parameter; Scenario X. Channel model; Stationary UE: AWGN Moving UE: Same as PUSCH for NTN-TDLA. CP length; Normal. A; 15 kHz: 10 s 30 kHz: 5 s. ; Depends on UE speed. Issue 1-1-7a: UE speed for PUSCH with UL timing adjustment Candidate options - Option 1 (Thales): Up to 1000 km/h - Option 2 (Huawei): 120km/h Recommendations for 2nd round: - Further discuss in the 2nd round. Issue 1-1-7b: Additional DM-RS position for PUSCH with UL timing adjustment Tentative agreements: pos1 Candidate options - Option 1 (Nokia, Huawei): pos1 - Option 2: pos2 Recommendations for 2nd round: N/A Issue 1-1-7c: Transform precoding for PUSCH with UL timing adjustment Candidate options - Option 1 (Nokia): Same as normal PUSCH - Option 2 (Huawei): CP-OFDM only Recommendations for 2nd round: - Discuss this issue after Issue 1-1-5. Issue 1-1-8: Test configuration for PUSCH repetition type A Agree to use the following parameters for NTN PUSCH. Parameter; Value. Channel model; NTN-TDL-A, Doppler and delay same as normal PUSCH. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 3, 0, 3 [Note 2]. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port; 0. DM-RS sequence generation; NID0=0, nSCID =0. Time domain resource assignment; PUSCH mapping type; A, B. Start symbol; 0. Allocation length; 14. PUSCH aggregation factor; n2. Frequency domain resource assignment; RB assignment; Same as normal PUSCH. Frequency hopping; Disabled. Code block group based PUSCH transmission; Disabled. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation.. Issue 1-1-8a: MCS for PUSCH repetition type A Candidate options - Option 1 (Nokia): MCS4 same as normal PUSCH - Option 2 (Huawei): MCS5 in Table 3 same as legacy TN PUSCH repetition type A Recommendations for 2nd round: - Further discuss in the 2nd round Issue 1-1-8b: Transform precoding for PUSCH repetition type A Candidate options - Option 1 (Nokia): Same as normal PUSCH - Option 2 (Huawei): CP-OFDM only Recommendations for 2nd round: - Discuss this issue after Issue 1-1-5. |
| Sub-topic #1-2 PUCCH | Issue 1-2-1: Scope of PUCCH requirements Moderator: Generally all companies are fine with Option 1, except the DMRS configuration for PUCCH format 3/4: both 1+0 and 1+1 or only 1+0 considered that can be further discussed in the 2nd round. Tentative agreements: Agree with Option 1 with FFS the additional DMRS configuration for PUCCH format 3/4 - For PUCCH format 2/3/4, addition note should be added that The UCI information does not contain CSI part 1 and CSI part 2. - For PUCCH format 0, select 2 symbol case from legacy BS requirements. - For PUCCH format 2, select both two cases with different UCI information bits from legacy BS requirements - For PUCCH format 3, only select 1 PRB with 14 symbol case from legacy BS requirements - For PUCCH format 3/4, FFS DMRS configuration: - Option 1: DM-RS 1+0 - Option 2: Both DM-RS 1+0 and 1+1 Based on the above tentative agreement, reuse other Rel-15 test parameters for all PUCCH formats, the whole picture for each PUCCH formats are as below, companies can further check them in the 2nd round discussion: Table 1 PUCCH format 0 Parameter; Test. PUCCH format; 0. Number of UCI information bits; 1. Number of PRBs; 1. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; Enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 12. Number of OFDM symbols; 2. Test metric; ACK missed detection probability <1%. Table 2 PUCCH format 1 Parameter; Test. PUCCH format; 1. Number of information bits; 2. Number of PRBs; 1. Number of symbols; 14. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (nrofPRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 0. Index of orthogonal cover code (timeDomainOCC); 0. Cyclic Prefix; Normal. Test metric; NACK to ACK probability<0.1%, ACK missed detection probability <1%. Table 3 PUCCH format 2 for ACK missed detection Parameter; Value. PUCCH format; 2. Modulation order; QSPK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; N/A. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Number of PRBs; 4. Number of symbols; 1. The number of UCI information bits; 4. First symbol; 13. DM-RS sequence generation; NID0=0. Cyclic Prefix; Normal. Test metric; NACK to ACK probability<0.1%, ACK missed detection probability <1%. Table 4 PUCCH format 2 for UCI BLER Parameter; Value. PUCCH format; 2. Modulation order; QSPK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. Frist PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Number of PRBs; [9]. Number of symbols; 2. The number of UCI information bits; 22. First symbol; 12. DM-RS sequence generation; NID0=0. Cyclic Prefix; Normal. Test metric; UCI block error probability<1%. Table 5 PUCCH format 3 Parameter; Value. PUCCH format; 3. Modulation order; QPSK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Number of PRBs; 1. Number of symbols; 14. The number of UCI information bits; 16. First symbol; 0. Cyclic Prefix; Normal. Additional DM-RS configuration; No additional DM-RS. Test metric; UCI block error probability<1%. Table 6 PUCCH format 4 Parameter; Value. PUCCH format; 4. Modulation order; QPSK. First PRB prior to frequency hopping; 0. Number of PRBs; 1. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Number of symbols; 14. The number of UCI information bits; 22. First symbol; 0. Length of the orthogonal cover code; n2. Index of the orthogonal cover code; n0. Cyclic Prefix; Normal. Additional DM-RS configuration; No additional DMRS. Test metric; UCI block error probability<1%. Table 7 PUCCH format 1 (multi-slot) Parameter; Test. PUCCH format; 1 (multi-slot). Number of information bits; 2. Number of PRBs; 1. Number of symbols; 14. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; disabled. Inter-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (nrofPRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 0. Index of orthogonal cover code (timeDomainOCC); 0. Number of slots for PUCCH repetition; 2. Cyclic Prefix; Normal. Test metric; NACK to ACK probability<0.1%, ACK missed detection probability <1%. Recommendations for 2nd round: - Continue discuss the DM-RS configuration for PUCCH format 3/4. Issue 1-2-1a: DM-RS configuration for PUCCH format 3/4 requirements Tentative agreements: N/A Candidate options - Option 1 (Huawei): - Only select additional DMRS configuration disabled case from legacy BS requirements - Option 2 (Ericsson, Thales): - If the same BS manufacture declaration in Rel-15 would be applied for SAN, PUCCH format 3/4 requirements should be defined for both with and without additional DM-RS to avoid no requirement situation. Recommendations for 2nd round: - Continue discussion in the 2nd round Issue 1-2-2: SCS/CBW set for PUCCH requirements Moderator: Ericsson updated Option 1 to only consider minimum bandwidth if full bandwidth can be used for UL. Considering the small number of PRB to be used for PUSCH and the link budget for NTN, moderator think that maximum bandwidth Tentative agreements: - If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths. - If full bandwidth can be used for UL, only requirements for minimum bandwidths could be introduced. - The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part - Reuse Rel-15 test applicability rule defined in clause 8.1.2.2.3 Applicability of requirements for different channel bandwidths Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-2-3: Antennal configuration Tentative agreements: - Align with the antenna configuration for PUSCH Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-2-4: Channel model Tentative agreements: - Align with the channel model for PUSCH Candidate options: N/A Recommendations for 2nd round: N/A |
| Sub-topic #1-3 PRACH | Issue 1-3-1: Preamble Format Tentative agreements: N/A Candidate options: - Option 1 (Ericsson, Thales, Nokia): Preamble format 0, A2, B4 and C2 - Option 2 (Huawei): Preamble format A2, B4 and C2 - Option 3 (Samsung): Preamble format 0 and C2 Recommendations for 2nd round: - Continue discussion in the 2nd round. Issue 1-3-2: SCS Tentative agreements: - Long preamble format (if agreed): 1.25kHz - Short preamble format (if agreed): 15kHz and 30kHz SCS. Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-3-3: Sequence length for short preamble format Tentative agreements: - Only consider LRA=139 for short preamble format. Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-3-4: Antenna configuration Tentative agreements: - Align with the PUSCH. Candidate options: N/A Recommendations for 2nd round: N/A Issue 1-3-5: Time error tolerance Tentative agreements: - Use same time error tolerance of AWGN as Rel-15 FR1 for 1.25kHz, 15kHz and 30kHz SCS. - Use the same method to calculate time error tolerance of multi-path channel as Rel-15: 0.522μ+Tdelay where Tdelay is corresponding to the tap delay of the propagation channel - Option 1 (Ericsson, Huawei): The largest delay of the propagation channel - Option 2 (Samsung, Huawei): The second largest delay of the propagation channel Candidate options: N/A Recommendations for 2nd round: - Further discussion the Tdelay value Issue 1-3-6: Frequency offset for PRACH Channel model Tentative agreements: - AWGN: 0Hz - Fading channel: FFS Candidate options for fading channel: - Option 1 (Ericsson, Thales): 400Hz - Option 2 (Huawei, Nokia, Samsung): 200Hz (0.1ppm@2GHz) Recommendations for 2nd round: - Continue discussion on the frequency offset for PRACH under fading channel. Issue 1-3-7: Ncs Tentative agreements: - Use the same Ncs as Rel-15 normal PRACH requirement, i.e. Burst format; SCS (kHz); Ncs; Logical sequence index; v. 0; 1.25; 13; 22; 32. A1, A2, A3,; 15; 23; 0; 0. B4, C0, C2; 30; 46; 0; 0. Candidate options: N/A Recommendations for 2nd round: - Confirm to reuse Logical sequence index and v value as Rel-15 normal PRACH requirements |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NTN SAN demodulation performance requirements | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2208015 |  | Discussion on PUSCH requirement for SAN demodulation | Ericssion | Noted |  |
| R4-2208016 |  | Discussion on PUCCH requirement for SAN demodulation | Ericssion | Noted |  |
| R4-2208017 |  | Discussion on PRACH requirement for SAN demodulation | Ericssion | Noted |  |
| R4-2208878 |  | Discussion on PUSCH demodulation requirements for NTN | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2209877 |  | Discussion on satellite NTN demod PUSCH | Huawei, HiSilicon | Noted |  |
| R4-2209878 |  | Simulation results on satellite NTN demod PUSCH | Huawei, HiSilicon | Noted |  |
| R4-2209879 |  | Discussion on satellite NTN demod PUCCH | Huawei, HiSilicon | Noted |  |
| R4-2209880 |  | Simulation results on satellite NTN demod PUCCH | Huawei, HiSilicon | Noted |  |
| R4-2209881 |  | Discussion on satellite NTN demod PRACH | Huawei, HiSilicon | Noted |  |
| R4-2209882 |  | Simulation results on satellite NTN demod PRACH | Huawei, HiSilicon | Noted |  |
| R4-2208085 |  | View on NTN SAN demodulation requirement | Samsung | Noted |  |
| R4-2209681 |  | TP to TS 38.108: remaining annexes for FRC (SAN demodulation requirements) | Huawei, HiSilicon | Postponed | postponed to future RAN4 meetings after we have stable enough progress on performance part |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Nicholas Pu | Nicholas.pu@ericsson.com |
| Nokia, Nokia Shanghai Bell | Anthony Lo | Anthony.Lo@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)