3GPP TSG-RAN4 Meeting #103-e	R4-2210379

E-meeting, 9 – 20 May, 2022												   (Revision of R4-2207896)

Source: 	Ligado Networks

Title: 	TP for TR 38.717-02-01: Updates to CA_n24-n77 RefSens requirements clause

Agenda item:	8.5.2

Document for: 	Approval

# Introduction

This contribution reflects MSD related updates for CA_n24-n77 to TR 38.717-02-01. In the USA, band n77 operation is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz.  The 2nd harmonic of UL n24 does not fall within this region. Therefore, MSD for n77 for CA_n24-n77 due to 2nd UL harmonic is not necessary. This proposal update the MSD value to N/A to reflect this.

# Reference

- TR 38.717-02-01 V0.8.0, Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1,2).

# Updates for CA_n24-n77 in 38.717-02-01

## CA_n24-n77

<<Unchanged Text Skipped>>

#### 6.52.1.3		Co-existence studies

Table 6.52.1.3-1 summarizes frequency ranges where harmonics occur for CA_n24-n77.

Table 6.52.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3253 | 3321 | 4879.5 | 4981.5 | 6506 | 6642 | 8132.5 | 8302.5 |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |  |

Based on above table, the 2nd harmonic of n24 UL may fall into the lower DL frequency range of n77.

<<Unchanged Text Skipped>>

#### 6.52.1.5		REFSENS requirements

From Table 6.52.1.3-1, 2nd harmonic of Band 24 in CA_n24A-n77A falls between 3253 – 3321 MHz. However, in the USA, band n77 is restricted to operation in frequency ranges of 3450- 3550 MHz and 3700 – 3980 MHz. Therefore, no MSD values for Band n77 due to 2nd harmonic of Band n24 in CA_n24A-n77A needs to be specified and is accordingly captured in Table 6.52.1.5-1 as N/A.

Table 6.52.1.5-1: MSD due to UL harmonic issue for CA_n24-n77

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n24 | n7716 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | NOTE 16: In the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. There is no UL harmonic due to n24 UL in downlink for n77 operating in 3450 – 3550 MHz and 3700 – 3980 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The uplink configuration for reference sensitivity exceptions due to UL harmonic interference for CA_n24A-n77A are captured in Table 6.52.1.5-2.

Table 6.52.1.5-2 Uplink configuration due to UL harmonic interference

|  |  | NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n24 | n77 | 12 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

#### 6.52.1.6		OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

### 6.52.2		Specific for 2 bands UL CA

#### 6.52.2.1	Maximum output power for inter-band CA

Table 6.52.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n24A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

#### 6.52.2.2		UE co-existence studies

Table 6.52.2.2-1: Impact of Intermodulations

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1626.5 | 1660.5 | 3300 | 4200 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1639.5 | 2573.5 | 4926.5 | 5860.5 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 947 | 21 | 4939.5 | 6773.5 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6553 | 7521 | 8226.5 | 10060.5 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 679.5 | 1681.5 | 8239.5 | 10973.5 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8179.5 | 9181.5 | 11526.5 | 14260.5 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3279 | 5147 | 9853 | 11721 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 2306 | 3342 | 11539.5 | 15173.5 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9806 | 10842 | 14826.5 | 18460.5 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3520.5 | 1618.5 | 6579 | 9347 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13153 | 15921 | 11479.5 | 13381.5 |

No IMD impact to the n77 is expected since it is TDD band. Based on the table above, the 4th order IMDs may fall into Rx frequencies of band n24.  However, within USA, the frequency of operation of n77 is going to be above 3450 MHz.  Moreover, n24 uplink operation is restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz.  For this combination of n77 and n24 UL operating frequency ranges, there are no IMDs falling on top of n24 Rx frequencies, and therefore no additional MSD needs to be specified for n24.

Table 6.52.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.52.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 |  |  |  |  |  |  |  |

#### 6.52.2.3		REFSENS requirements

Table 6.52.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n24A-n77A1 | n24 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: There is no IMD4 product in band n24 downlink for n77 operating in the USA since n77 is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz and n24 uplink is restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. |  |  |  |  |  |  |  |

---------------------------<End of Changes>--------------------------