3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2210419

Electronic Meeting, May 9th – 20th, 2022

Title:	TP to TR 38.717-04-02: Addition of CA_n1-n7-n8-n40

Source:	Nokia, OPTUS

Agenda item:	8.9.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-04-02 to add CA_n1A-n7A-n8A-n40A with 2UL. Note this TP is depending on approval of lower order fallbacks submitted at the same meeting. These are found as: R4-2209941 (CA_n1A-n7A-n8A), R4-2209942 (CA_n1A-n7A-n40A), R4-2209944 (CA_n7A-n8A-n40A), R4-2209943 (CA_n1A-n8A-n40A)

************************************* Start of TP*****************************************

## 5.1.x	CA_n1-n7-n8-n40

## 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands of CA_n1-n7-n8-n40

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n1-n7-n8-n40 | n1, n7, n8, n40 |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported bandwidths per CA_n1-n7-n8-n40

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n7A-n8A-n40A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n40A CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |

CA_n1A-n7A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n1A-n8A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n1A-n40A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n7A-n8A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n7A-n40A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n8A-n40A 2UL defined in 38.101-1 Table 5.5A.3.1-1

#### 5.1.x.3		UE co-existence studies

The coexistence studies have been captured into the constituent fallback modes in TR 38.717-03-02, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 5.1.x.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c  could reuse the values for CA_n1-n7-n8-n40 with 1UL that have been captured  into TR 38.717-04-01.

### 5.1.x.5	REFSENS requirements

MSD requirements are captured in lower order combinations.

************************************* End of TP*****************************************