3GPP TSG-RAN WG4 Meeting # 103-e 												R4-2210474

Electronic Meeting, May 09 – May 20, 2022

Agenda item:			9.3.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [103-e][205] NR_RF_FR1_enh_RRM

Document for:	Information

Introduction

This email thread discusses the RRM test cases of R17 NR FR1 RF enhancement (WID is [RP-202088]).

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to provide comments in each sub-topic.

- 2nd round: TBA

Topic #1: RRM test case of R17 Tx switching enhancements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207867 | China Telecom | Test case |
| R4-2208389 | CMCC | Test case |
| R4-2208958 | Huawei, Hisilicon | Test case |

Open issues summary

Open issues

Background

The test case list and parameters have already been approved in [R4-2206836].The following six test cases are to be verified.

Six DL interruption test cases in SA need to be verified: - DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors for UL inter-band CA #1: NR UL FDD carrier 1 with 2Tx and NR UL TDD carrier 2 with 2Tx, and the two uplink carriers are in different bands; #2: NR UL TDD carrier 1 with 2Tx and NR UL TDD carrier 2 with 2Tx, and the two uplink carriers are in different bands, and carrier 1 and carrier 2 have different UL-DL pattern. - DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors for UL inter-band CA #3: NR UL FDD carrier 1 in band A with 1TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers; #4: NR UL TDD carrier 1 in band A with 1TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers and with the same TDD pattern. Carrier 1 and carrier 2 have different UL-DL pattern. - DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors for UL inter-band CA #5: NR UL FDD carrier 1 in band A with 2TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers; #6: NR UL TDD carrier 1 in band A with 2TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers and with the same TDD pattern. Carrier 1 and carrier 2 have different UL-DL pattern.

In this meeting, R4-2207867(from China Telecom) provides test case #1and #2, R4-2208958 (from Huawei) provides test case #3and #4, and R4-2208389 (from CMCC) provides test case #5 and #6.

No open issues are to be discussed in this section. Please directly comment on CRs listed in section 1.2.2.

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207867 Draft CR (China Telecom) | Huawei: the CR is fine. |
|  | MTK: Regarding the SRS configuration, the CR has “SRS configuration in Table A.4.4.1.1.1-3 is applied except that:resourceMappingstartPosition: 0, resourceMappingnrofSymbols: n2”. However, when we check Table 4.4.1.1.1-3, there are still 3 configurations (SRSConf.1, SRSConf.2, SRSConf.3) with different periodicityAndOffset-p. We are not sure which one we will use here. It would be good to further clarify it. |
| R4-2208958 Draft CR (Huawei) | MTK: - Same comment to 7867 - Suggest lowering the SS-RSRP of Cell#2 and #3 by 3 or 4 dB. The reason is that one CC already contributes Io = -52.86dBm. This means 2 CCs on the same band will result in Io > -50 dBm. We understand in the current spec, we may have some ambiguity on whether -50dBm is a limit to a single CC or CA. But reducing the SS-RSRP can avoid us opening the discussion. |
|  | Company B |
| R4-2208389 Draft CR (CMCC) | Huawei: the CR is fine. |
|  | MTK: Same comments to 8958 |

Summary for 1st round

    - Open issues

    - CRs/TPs

Some comments are received for the three CRs.  In 2nd round, please authors of the CRs revise the CR accordingly.

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2207867 | Draft CR: Test case for DL interruptions at 2Tx-2Tx switching between two uplink carriers in FDD+TDD and TDD+TDD inter-band CA | China Telecom | Revised |  |
| R4-2208958 | Test case for R17 Tx switching enhancement | Huawei | Revised |  |
| R4-2208389 | Draft CR for introducing test cases for DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors for UL inter-band CA (section A.6.5.7.1C, A.6.5.7.2C) | CMCC | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Status | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2207867 | R4-2210990 | Draft CR: Test case for DL interruptions at 2Tx-2Tx switching between two uplink carriers in FDD+TDD and TDD+TDD inter-band CA | China Telecom | Agreeable |  |
| R4-2208958 | R4-2210991 | Test case for R17 Tx switching enhancement | Huawei | Agreeable |  |
| R4-2208389 | R4-2210992 | Draft CR for introducing test cases for DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors for UL inter-band CA (section A.6.5.7.1C, A.6.5.7.2C) | CMCC | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)