3GPP TSG-RAN WG4 Meeting # 103-e                                				R4-2210554

Electronic Meeting, 9– 20 May, 2022

Source: 	Huawei, HiSilicon, MediaTek, Qualcomm, Skyworks

Title:	TP on TR 37.717-11-11 Update on the MSD value of DC_(n)3

Agenda Item:	8.3

Document for:	Approval

# Introduction

Due to small duplex distance in n3, the need to MSD analysis of DC_(n)3AA  combination was raised during RAN WG4 Meeting # 101-e [1]. During After several meetings the multiple values were proposed that are captured in the Table 1 and also, recorded at the first round of the current meeting Email summary[2]. The analysis based on the WF R4-2107805, showed that the most critical test point belongs to the case of an NR Tx 30MHz and a LTE Rx 15MHz, where the highest MSD is obtained with the smallest Rx CBW

Table 1 Proposed MSD values for that corresponds to the worst case degradation

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | Mediatek: [6.6] Qualcomm: [7.3] Skyworks: [3.9] |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |

It was agreed during the first round discussion to take the average of the proposed MSD values for the final MSD test point, as given in Table 2:

Table 1 MSD value obtained by averaging the proposed MSD values

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |

# Reference

- R4-2119803 TP for TR 37.717-11-11: DC_(n)3AA

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_103-e/Inbox/Drafts/%5B103e%5D%5B115%

5D%20NR_Baskets_Part_1/Round%201/draftR42210250%20email%20summary%20Rd1%20115.docx

# Text Proposal

----- Start of TP -----

6.3.2.5	MSD analysis for DC

Only single switched UL mode is supported and UL-DL separation is not large (20 MHz). Hence, the following MSD value is proposed:

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |

----- End of TP -----