3GPP TSG-RAN WG4 Meeting # 103-e 													R4-2210558

Electronic Meeting, May 9 – 20, 2022

Agenda item:			8.21.2.2

Source:	Qualcomm

Title:	WF on adding 100 MHz channel BW in NR-U bands n46 and n96

Document for:	Approval

# Topic #2: NR-U bands n46 and n96 – 100 MHz channel BW

### Sub-topic 2-1: Common to both bands n46 and n96

Issue 2-1-1: UE support of 100 MHz channel BW in bands n46 and n96 (and n102)

Agreement

- The UE support of 100 MHz channel BW shall be optional.

### Sub-topic 2-2: Band n46

Issue 2-2-1: Channel raster for band n46

Agreement

- Four channel raster: 5200, 5300, 5520 and 5865 MHz.

Issue 2-2-2: Additional channel raster when the absence of other technologies is guaranteed.

Agreement

- Further study would be needed to guarantee the absence of other technologies and to consider an expanded channel raster.

- A note will be added to the Chairman’s notes to highlight that additional channel raster could be added in the future when the absence of other technologies is guaranteed.

### Sub-topic 2-3: Band n96

Issue 2-3-1: Channel raster

Agreement

- The following channel raster (NREF) will be specified for n96, 100 MHz channel BW: 799668, 803668, 810332, 814332, 821000, 825000, 831668, 835668, 842332, 846332, 853000, 857000, 863668, 867668, 869000, 870332, 871668.

Issue 2-3-2: MPR

Agreement

- Specify following MPR mapping for 100 MHz channel BW:

  - Sub-band config A: 11111, 01111, 11110, 00111, 01110, 11100, 00011, 00110, 01100, 11000

  - Sub-band config B: None

Issue 2-3-3: SEM

Agreement

- The SEM for 100 MHz channel BW shall be as specified in table 6.5F.2.2-1.

Issue 2-3-4: A-MPR

Agreement

- A-MPR is postponed to Rel-18.

Issue 2-3-5: UL CA

Agreement

- All aggregated bandwidth up to 160MHz and including 100 MHz are specified in Release 17. 100+100 MHz is not precluded.

Issue 2-3-6: REFSENS

Agreement

- REFSENS for 100 MHz channel BW will be specifed via an equation-based approach, adding 100 MHz for band n96 (and n102)  in Table 7.3F.2-1.

Issue 2-3-7: ACS

Agreement

- ACS for 100 MHz channel BW will be specified via an equation-based approach, adding 100 MHz for band n96 (and n102)   in Tables 7.5F.1-1 and 7.5F.1-2.

Issue 2-3-8: In-band blocking

Agreement

- In-band blocking for 100 MHz channel BW will be specified via an equation-based approach, adding 100 MHz for band n96 (and n102)   in Table 7.6F.2.1-1.

Issue 2-3-9: Out-of-band blocking

Agreement

- Out-of-band blocking for 100 MHz channel BW will be specified via an equation-based approach, adding 100 MHz for band n96 (and n102)   in Table 7.6F.3.1-1.

Issue 2-3-10: Spurious

Agreement

- Spurious for 100 MHz channel BW will be specified via an equation-based approach, adding 100 MHz for band n96 (and n102)   in Tables 7.7F.1-1  and 7.8F.2-1.

Issue 2-3-11: Triple Punctured Channel SEM

Agreement

- Triple Punctured Channel SEM for 100 MHz channel BW is specified with -25 dBr at 10 MHz from the edges.

### Sub-topic 2-4: Band n102

Issue 2-4-1: Introduce 100 MHz channel BW in band n102

Agreement

- Add 100 MHz channel BW in band n102 as well.

Issue 2-4-2: Channel raster

Agreement

- Add the following NREF values for 100 MHz channel BW: 799668, 803668, 810332, 814332, 821000, 825000.