3GPP TSG-RAN WG4 Meeting #103-e	  R4-2210574

Electronic Meeting, May 09 – May 20, 2022

Agenda Item:	9.4.3.1, 9.4.3.2 and 9.4.6.3

Source:	Apple

Title:	WF on FR2 RF enhancement part 2: UL gaps

Document for:	Approval

# 1	Tx power management: RF aspect

Summary of 1st round email discussion is captured in [1]. The following agreements were captured in GTW discussion.

Agreement: Step 3 discussion is decoupled with UL gap.

The step 3 which was captured in R4-2206604 is copied below for easier reference.

FFS: 3. Measure the EIRP where the UL duty cycle is configured lower than the maxUplinkDutyCycle-FR2 (or UL duty cycle = [10] % if UE does not report the maxUplinkDutyCycle-FR2) and without the UL gap configured.  P-bit = 0 for UE report the maxUplinkDutyCycle-FR2 or enhanced EIRP2 (should be at least reference EIRP + [3] dB) for UE does not report the maxUplinkDutyCycle-FR2.

-  no P-MPR should be applied when the configured UL duty cycle is lower than the UE reported capability maxUplinkDutyCycle-FR2 per Rel-15 agreement.

-  For UE does not report maxUplinkDutyCycle-FR2, it is still correct UE behaviour to lower the PMPR with reduced uplink duty cycle.

Agreement: When UL gap is not configured/activated and PHR is configured during the test, P bit in PHR shall be 1 during the UL gap test.

Agreement: UE will meet OFF power requirement defined in this clause for the band with activated UL Gap.

Agreement: Add the following clarification to TS38.101-2 6.2: The measured peak EIRP PUMAX,f,c_GAP_ON shall be measured outside of the UL gap symbol.

In addition, the following agreements are proposed pending on 2nd round email discussion.

On UE capability for inter-band UL CA,

Agreement:

Introduce the UE capability for inter-band UL CA on whether UL transmission in different FR2 within gap is feasible when UL gap is activated.

    - For CBM, the capability is per band per band combination

    - For IBM, the capability is per band per band combination.

On RMC:

Agreement: new RMC is introduced based on UL gap pattern configuration #1 and 3.

- The RMC table can be further updated if additional UL duty cycle needs to be introduced

- Table A.2.3-1: Additional reference channels parameters for TDD

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSUUUU | 7DS8U |
| Special Slot Configuration (Note 2) |  | S=4D+6G+4U | S=12D+2G |
| referenceSubcarrierSpacing |  | 60 kHz | 120 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 2 ms | 2 ms |
|  | nrofDownlinkSlots | 3 | 7 |
|  | nrofDownlinkSymbols | 4 | 12 |
|  | nrofUplinkSlot | 4 | 8 |
|  | nrofUplinkSymbols | 4 | 0 |
| Indexes of active UL slots |  | mod(slot index, 40) = {36,…,39} | mod(slot index, 80) = {72,…,79} |
| Indexes of active UL slots for UL Gap test |  | mod(slot index, 40) = {12,…,15, 36,…,39} | mod(slot index, 80) = {24,…,31 ,72,…,79} |
| Indexes of the UL slots for UL Gap when UL gap pattern configuration 3 (IE name for configurations) is configured |  | mod(slot index,40)={7, 28} | mod(slot index, 80) = {15,56} |
| Indexes of the UL slots for UL Gap when UL gap pattern configuration 1 (IE name for configurations) is configured |  | mod(slot index,160)={20, 21, 22,23, 28, 29,30,31} | mod(slot index, 320) = {8, … ,15} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |  |

On test procedure and methodology:

Agreement:

No interruption related test is needed when all active FR2 CCs stop UL transmission during UL gap.

- FFS if interruption related test is needed when some active FR2 CCs continue UL transmission during UL gap.

# 2	UL Tx power management: RRM aspect

Summary of 1st round email discussion is captured in [1]. The following agreements are proposed pending on 2nd round email discussion.

On procedures to be prioritized over UL gap,

Agreement:

On top of the agreement to prioritize RACH procedure, CG-PUSCH (type 1 and 2) and PUCCH allocations for SR and LRR over UL gap, further agree to prioritize:

- The valid CSI report and/or valid L1-RSRP report during SCell activation procedure, where the valid CSI report is valid CQI with non-zero CQI index defined in clause 5.2.2.1, TS 38.214 and the valid L1-RSRP report is non lowest L1-RSRP defined in clause 10.1.6.

  - The UE need not apply UL gap prioritization rules specified above for SCell activation procedure if the time period between UL gap colliding with CSI report of non-zero CQI or L1-RSRP and the slot where the SCell activation MAC CE or CSI report activation command is received is less than [X ms].

# 3	UL coherent MIMO

Summary of 1st round email discussion is captured in [1]. The following agreements are proposed pending on 2nd round email discussion.

Agreement:

Enable implicit triggering of the UL gap for UL coherent MIMO, by defining K2_min_cal which include the PUSCH preparation time plus the calibration time.

Agreement:

The following gap patterns are agreeable.

- Table 1. UL Gap length candidates

| UGL | SCS of active BWP | UGL |  |
| --- | --- | --- | --- |
|  |  | ms | #slots |
| UGL #0 | 120kHz | 0.25 | 2 |
|  | 60kHz | 0.25 | 1 |

Agreement:

Introduce per UE capability on whether UE support capability of performing coherent UL MIMO calibration in UL gap.

Agreement:

- Proposal 1: The channel estimation should be determined based on DMRS REs only.

- Proposal 2: The TE should perform a CFO correction on a slot-by-slot basis using a common frequency correction at the two uplink physical antennas. (Anritsu)

- Proposal 3: The common frequency correction should be calculated as the average of the signed frequency errors at the two uplink physical antennas. (Anritsu)

# 4 References

[1] draft R4-221xxxx, “Email discussion summary for [103-e][129] NR_RF_FR2_enh2_Part_2”, Apple