3GPP TSG-RAN WG4 Meeting #103-e	R4-2210608

Electronic Meeting, May 09 – May 20, 2022

Title: 	WF on HST FR2 RRM Core Requirement Maintenance

Agenda Item:	9.8.2

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

## Topic #1: UL timing

## Sub-topic 1-1: Conditions when one-shot large UL timing adjustment apply

Issue 1-1-1: A need for DL timing difference threshold

GtW Agreement:

UE shall apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than a timing difference threshold

Issue 1-1-2: The value of DL timing difference threshold

Agreement:

The value of DL timing difference threshold equals 4.5*64*Tc = Tq= CP/4

NOTE: the value of the threshold in bi-directional deployment shall be evaluated further in Rel-18.

## Sub-topic 1-2: UL transmit timing accuracy

GtW Agreement:

If new TCI state within active TCI state list: Adopt ±Te immediately after TCI state switch as the accuracy otherwise ±[7Ts] adopted.

## Sub-topic 1-3: highSpeedLargeOneStepUL-TimingFR2-r17 is not enabled

Issue 1-3-1: Requirements for the case when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled

Agreement:

No need to introduce new UL timing requirements for the case when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled.

Issue 1-3-2: Scheduling/Transmit restriction after TCI state switch

Way Forward:

Further discussion is needed whether and how to define transmit or scheduling restriction for UL after the TCI state switch when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled.

- Option 2: No impact on UE behavior

- Option 3: Define scheduling restriction on DL and UL after inter-RRH TCI state switch and before PRACH transmission when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled

- Option 4: after the TCI state switch, the UE shall not transmit except for RACH preamble in the new target TCI before one of the following conditions is fulfilled:

-	the new timing advance is acquired and applied in the target TCI state according to the requirements in clause 7.3;

-	the UL transmission is scheduled by the gNB.
    In this case, the requirements in clause 7.1.2.1 apply.

- Other options are not precluded

## Sub-topic 1-2: UL capabilities

Issue 1-4-1: Support of one shot large UL timing adjustment feature group

GtW Agreement:

Define feature as optional with capability signaling

# Topic #2: Remaining issues in RRM requirements

Issue 2-1-1: Scheduling restriction on SSB

Agreement:

FR2 HST requirement is applicable only when the two SSBs from adjacent RRHs are not on the adjacent symbols.

Issue 2-1-2: Time period for PSS/SSS detection and Measurement period for intra-frequency measurements

Agreement:

Define Mpss/sss_sync_w/o_gaps = 18, and Mmeas_preriod_w/o_gaps = 18 for set 2.

Issue 2-1-3: SMTC limit in HST FR2 enhanced requirements

GtW Agreement:

Option 1 agreed as starting point and further work on the drafting CR revision including table heading and note 3.

Way forward:

Further discussion is needed on how to define HST FR2 requirements if SMTC periodicity is > 40 ms.

- Option 1: Apply the FR2 HST enhanced requirement only when SMTC <=40ms cases. When SMTC period > 40ms, requirements in Table 9.2.5.2-2 apply.

- Option 2: Delete NOTE 3, keep table titles without changes and set M2 = 1.5

- Other options are not precluded

Issue 2-1-5: Intra-frequency measurements with gaps

Agreement:

For the enhancement of SA intra-frequency measurements with gaps in connected mode for FR2 HST, the same enhancement as for intra-frequency measurements without gaps should be applied.

Issue 2-1-6: L1-SINR measurements with SSB-based CMR and dedicated IMR

Way forward:

Open issue needs further discussion

- Option 1: For L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST, the same enhancements as SSB-based L1-RSRP measurements should be applied.

- Option 2: Do not define enhancement for L1-SINR

- Other options are not precluded