3GPP TSG-RAN WG4 Meeting #103-e		R4-2210658

Electronic Meeting, 9 - 20 May, 2022

Agenda item:	9.11.2.3.1

Source:	CMCC

Title:	Simulation assumptions for CRS-IM (30kHz SCS TDD)

Document for:	Agreement

Introduction

In this paper we provide detailed simulation assumptions for CRS-IM with serving cell 30kHz SCS. Based on the agreed WF in [1].

Simulation assumptions for CRS-IM under scenario 2 with serving cell 30kHz SCS

Scenario 2: Serving cell is operating in NR mode and interference cell is operating in LTE mode.

Table 2-1:	Serving cell configuration

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| TDD UL-DL pattern |  |  | 7DS2U with S=6D+4G+4U |
| Rank |  |  | 1 |
| MCS |  |  | 13 |
| MCS table |  |  | Table 1 |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | false |
| Propagation channel |  |  | TDLA 30-10 |
| Antenna configuration |  |  | ULA Low 4 x 2 and 4 x 4 |
| Receiver Type |  |  | MMSE-IRC |
| Overhead for TBS determination |  |  | 0 |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 0 and #1 |
| PDSCH configuration (Note 1) | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 2 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| Max HARQ re-transmission number |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 in TS 30.101-4 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot with PRB bundling granularity |
| LTE interference cell configuration |  |  | Specified in Table 2-2 |
| Note 1: No PDSCH data scheduling on slots with LTE PBCH/PSS/SSS, no PDSCH scheduling on the specials slot. |  |  |  |

Table 2-2:	LTE interference cells configuration

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Transmission mode |  |  | TM4 | TM4 |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| TDD UL-DL pattern (Note 1) |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 20 | 20 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH loading level |  |  | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 1.5 | -0.5 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Network-based CRS interference mitigation (CRS muting) |  |  | Disabled | Disabled |
| MBSFN configuration |  |  | No MBSFN is configured | No MBSFN is configured |
| Note 1: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 2: The channel for interference cells and serving cell are independent. |  |  |  |  |

- References

- R4-2207240 “WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS”, CMCC, RAN4 #102-e, March 2022.