3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210664

# Electronic Meeting, 9 – 20 May, 2022

Agenda item:	10.16.10

Source:	Intel Corporation

Title:	WF on general and BS aspects for FR2-2 demodulation requirements

Document for:	Approval

# Background

- Corresponding Email summary in RAN4#103-e

  - R4-2210528 Email discussion summary for [103-e][325] NR_exto71GHz_Demod_Part1.

# WF on topic #1: General

## General aspects of demodulation requirement definition

Issue 1-0-1: Updated work plan

The updated work plan is as follows.

| RAN4#102-e: |  |
| --- | --- |
|  | - Discussion and agreement on work plan. - Discussion on performance requirements scope - Initial discussion on work split - Initial discussion on simulation assumptions |
| RAN4#103-e: |  |
|  | - Finish discussions on performance requirements scope per each physical channel - Finish discussions on simulation assumptions per each physical channel - Finish discussions on work split - Initial round of simulation results collection and alignment |
| RAN4#104 |  |
|  | - Another round of simulation results collection and alignment - Initial draft CRs for TS 38.104 Base Station (BS) radio transmission and reception - Initial draft CRs for TS 38.141-2 Base Station (BS) conformance testing Part 2: Radiated conformance testing - Initial draft CRs for TS 38.101-4 User Equipment (UE) radio transmission and reception; Part 4: Performance requirements |
| RAN4#104-bis-e |  |
|  | - Another round of simulation results collection and alignment - CRs for TS 38.104 Base Station (BS) radio transmission and reception - CRs for TS 38.141-2 Base Station (BS) conformance testing Part 2: Radiated conformance testing - CRs for TS 38.101-4 User Equipment (UE) radio transmission and reception; Part 4: Performance requirements |
| RAN4#105-e |  |
|  | - Final round of simulation results collection and alignment - Final CRs for TS 38.104 Base Station (BS) radio transmission and reception - Final CRs for TS 38.141-2 Base Station (BS) conformance testing Part 2: Radiated conformance testing - Final CRs for TS 38.101-4 User Equipment (UE) radio transmission and reception; Part 4: Performance requirements |

## LBT

Issue 1-1-1: Whether to define FR2-2 demodulation requirement without LBT model for both DL and UL

Rel-17 RAN4 focused on defining FR2-2 demodulation requirement without LBT model for UL/DL.

It’s not precluded to discuss and specify specific requirements with LBT model in future releases.

## Phase noise

Issue 1-2-1: How to consider phase noise model for the FR2-2 demodulation simulation

Assume PN model in Receiver side with CPE compensation for initial simulation purpose for both DL and UL

- FFS when and how to consider ICI including performance gain, implementation complexity and test feasibility

Issue 1-2-2: Whether TE PN impact is ignored or not in the simulations

Invite TE vendors to give comment if FR2-2 phase noise impact by TE can be ignored or not.

In case there is no input in the next meeting, the group does not consider TE PN impact.

Since there is no comment from TE vendors in this meeting, we will not consider the Tx PN impact in the simulations. The Tx PN impact is considered in the impairment results pending on BS implementation.

## Channel model for requirements definition

Issue 1-3-1: How to consider the delay profile for TDLA channel model for FR2-2?

FFS the delay profile details.

Issue 1-3-2: How to consider the minimum tap resolution for TDL channel model?

Invite TE vendors to confirm the minimum tap resolution for TDL channel model they could deliver.

- Proposal 1: 2ns delay resolution

- Proposal 2: 2.5ns delay resolution

- Proposal 3: TBA

New Issue 1-3-3: Channel model for UL requirements

| SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 480 | 400 | 4 | TDLA10-650 | FFS 960 | 400 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |
|  | 400 | 4 | TDLA10-650 |  | FFS 1600 | 4 | TDLA10-650 |  | FFS 2000 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |

Note 1: Interested companies can provide simulation results for other cases not included in above table, as needed.

# WF on topic #2: BS performance requirements

## General issues

Issue 2-1-1: Whether to define demodulation requirements for 960kHz SCS?

FFS whether dedicated UL performance requirements will be introduced for 960kHz SCS

FFS whether dedicated DL performance requirements will be introduced for 960kHz SCS

Companies are encouraged to discuss simulation assumption for 960kHz SCS

## PUSCH performance requirements

Issue 2-2-2: How to consider transform decoding?

Specify requirements for FR2-2 PUSCH demodulation with transform precoding enabled.

Agreed Issue 2-2-4: Parameter list for PUSCH performance requirements (Agreements in the last meeting)

General PUSCH test setup

Consider the following simulation assumptions at starting point for PUSCH performance requirements:

- MCS 4, 16, 20

  - FFS MCS 4, 16, 20 for 1 Tx and MCS 4, 16 for 2 Tx

- DM-RS/PT-RS configuration Rel-15 assumptions

- 1+1 DMRS configuration

- PTRS Tx on

- Temporary PRB number (for simulations):

  - (66)(264)(66)([124])([248])(33)([62])([124])(148) for SCS (kHz CWB (MHz)) = (120 100)(120 400)(480 400) (480 800)(480 1600)(960 400)(960 800)(960 1600)(960 2000)

  - The numbers are modified according to RF agreements in this meeting

Detailed PUSCH test setup

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric | Normalized throughput | 70% |

Rx processing assumptions

Define PUSCH performance requirements by using phase noise compensation

- Companies are encouraged to bring results for CPE and CPE+ICI compensation methods.

## PUCCH performance requirements

Updated Issue 2-3-1: How many PRBs and symbols are considered for each of the PUCCH format?

Introduce below test cases

- For Format 0:

  - 1RB /1 symbol

  - FFS for [10/16] RB/1 symbol

- For format 1:

  - 1 RB/14 symbols

  - [10/16] RB/14 symbols

- For format 2: Reusing FR2-1

- For format 3: Reusing FR2-1

- For format 4:

  - 1 RB/14 symbols

  - [10/16] RB/14 symbols

Agreed Issue 2-3-4: Parameter list for PUCCH performance requirements (Agreements in the last meeting)

Introduce below test cases

- For Format 0:

  - Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1

  - Test metric: ACK missed detection rate < 1%

- For format 1:

  - Intra-slot frequency hopping: enabled

  - Test metric: ACK missed detection rate < 1% and NACK to ACK probability < 0.1%

- For format 2:

  - Intra-slot frequency hopping: enabled

  - Test metric: ACK missed detection rate < 1%, UCI BLER < 1%

- For format 3:

  - Intra-slot frequency hopping: enabled

  - Test metric: UCI BLER < 1%

- For format 4:

  - Intra-slot frequency hopping: enabled

  - Test metric: UCI BLER < 1%

## PRACH performance requirements

Agreed Issue 2-4-4: Parameter list for PUCCH performance requirements (Agreements in the last meeting)

Scope of PRACH requirements

Define the following PRACH performance requirements:

- False alarm probability requirements

- PRACH miss detection requirements

Sequence length

PRACH sequence length:

- 120 kHz:

  - 1151, 571, 139

- 480 kHz:

  - 571, 139

- 960 kHz:

  - 139

PRACH formats

PRACH formats: A2, B4, C2

Frequency offset

- 7100 Hz

Time error tolerance

Time error tolerance:

- 0.07us for AWGN

- FFS for multi-path fading

(Ncs, logical sequence index, v)

- Consider the following PRACH parameters as starting point: Ncs = 69, logical sequence index = 0, v =0 as starting point

- Other options are not precluded

# Work split (for information)

Table 1. Demod CR work split for TS 38.104

| draftCR and bigCR work-split - TS 38.104 |  |  | Company |
| --- | --- | --- | --- |
| TS 38.104 bigCR |  |  | Intel |
| TS 38.104 BS Performance requirements (draftCRs) |  |  |  |
|  | 11 Radiated performance requirements |  |  |
|  |  | PUSCH requirements | Intel |
|  |  | PUCCH requirements | Huawei |
|  |  | PRACH requirements | Nokia |
|  | Appendix |  |  |
|  |  | PUSCH FRCs | Ericsson |
|  |  | , PRACH Test preambles, propagation conditions | Samsung |

Table 2. Demod CR work split for TS 38.141-2

| draftCR and bigCR work-split - TS 38.141-2 |  |  | Company |
| --- | --- | --- | --- |
| TS 38.141-2 bigCR |  |  | Nokia |
| TS 38.141-2 BS Radiated conformance testing (draftCRs) |  |  |  |
|  | 8 Radiated performance requirements |  |  |
|  |  | PUSCH requirements | Huawei |
|  |  | PUCCH requirements | Intel |
|  |  | PRACH requirements | Nokia |
|  | Appendix |  |  |
|  |  | PUSCH FRCs | Ericsson |
|  |  | PRACH Test preambles, propagation conditions, test tolerance | Samsung |

Table 3. Demod CR work split for TS 38.101-4

| draftCR and bigCR work split - TS 38.101-4 |  |  |  | Company |
| --- | --- | --- | --- | --- |
| TS 38.101-4 bigCR |  |  |  | Huawei |
| TS 38.101-4 UE performance requirements (draftCRs) |  |  |  |  |
|  | 7 Demodulation performance requirements (Radiated requirements) |  |  |  |
|  |  | General (incl. applicability rule) |  | Qualcomm |
|  |  | PDSCH requirements |  | Huawei, Ericsson |
|  |  | PDCCH requirements |  | Nokia |
|  |  | (PBCH requirements) |  | Qualcomm |
|  |  | SDR requirements |  | Ericsson |
|  | 8 CSI reporting requirements (Radiated requirements) |  |  |  |
|  |  |  | General (incl. applicability rule), CQI reporting requirements | Apple |
|  | Appendix |  |  |  |
|  |  | PDSCH, PDCCH, (PBCH) RMCs |  | Apple |
|  |  | CSI RMCs and propagation conditions |  | Apple |