3GPP TSG-RAN WG4 Meeting #103-e	R4-2210681

Electronic Meeting, May 9- May 20, 2022

Agenda item:	9.11.2.1.1, 9.11.2.1.2

Source:	Ericsson

Title:	WF on PDSCH demod and CSI requirements for inter-cell interference MMSE-IRC

Document for:	Approval

# Introduction

This is the WF to capture all agreements and open issues in [103-e][336] for WF on PDSCH demod and CSI requirements for inter-cell interference MMSE-IRC in RAN4#103-e meeting.

# PDSCH Demodulation requirements

Issue 1-1-1: Interference modelling in PDCCH region

- Option 1: All the REs in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports.

- Option 2: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell.

- Option 3: Assume PDCCH transmission from interference cells which is non-overlapping with serving cell.

# CQI Requirements

Issue 2-1-1: Test setup methodology for signal power

GTW agreement:

- Define test based on SINR

Issue 2-1-2: SINR for 2 Rx

GTW agreement:

- -2 dB

Issue 2-1-3: T-put gain for 2 Rx

- Option 1: 2

- Option 2: 1.5

Issue 2-1-4: SINR for 4 Rx

GTW agreement:

- -2 dB

Issue 2-1-5: T-put gain for 4 Rx

- Option 1: 2.5

- Option 2: 2

- Option 3: 1.5

# SNR test point

RAN4 will make decision on the SNR values in the next meeting based on the simulation result summary.

Issue 3-1-1: SNR deriving rule

RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes X dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within X dB.

- Option 1: X = 2dB

- Option 2: X = 2.5dB

# Reference

[1] R4-2207243, “WF on CSI requirements for inter-cell interference MMSE-IRC”, Ericsson, RAN4#101e, Feb,2022