3GPP TSG-RAN WG4 Meeting #103-e	R4-2210731

9 – 20 May, 2022

Source:	Qualcomm Incorporated

Title:	V2X RF switching issues

Agenda item:	9.14.2

Document for:	Approval

# 1. 	TP for TR38.785

--Start of changes

To decide the switching position for TDM operation in same/different carrier, RAN4 will follow the priority rules outlined in TS38.321 and TS38.213.

--End of changes

--Start of changes

### 5.2.1	Intra-band V2X operation scenarios and basic assumptions

For intra-band V2X operation, both TDM and FDM between Uu and SL are considered. The prioritization of operating scenarios including TDM and FDM and respective basic RF architecture for operating scenarios are defined as follows:

- 1st priority: TDM with same carrier or different carrier (Single RF chain for Tx as baseline)

  - RAN4 allow TDM operation between spectrally partially used PC5 SL and Uu UL/DL operation in a licensed TDD band regardless of adjacent/ non-adjacent carrier

- 2nd priority: FDM with adjacent carrier (Separate RF chain for Tx as baseline)

- 3rd priority: FDM with non-adjacent carrier (Separate RF chain for Tx as baseline)

Other UE RF architecture is not precluded from implementation perspective.

In Rel-17, RAN4 focus on TDD band for intra-band V2X operation and deprioritize FDD band. FDD band can be studied once operators have a request.

For intra-band contiguous and non-contiguous V2X con-current operation with FDM operation, simultaneous UL transmission and SL reception in TDD band are not allowed.

Release independence to earlier releases is optional for both inter-band and intra-band con-current V2X operation.

In Rel-17, SL transmission timing remains aligned with  DL timing as defined in Rel-16. When it comes to intra-band Uu and V2X operation in a licensed band, it is more likely to have self-interference between Uu and SL, especially for the same carrier. Uu timing and SL timing are illustrated in figure 5.2.1-1.

--End of changes