3GPP TSG-RAN WG4 Meeting #103e	R4-2210802

Electronic Meeting, 9th – 20th May, 2022

Source:	ZTE Corporation

Title:	TP revision on Section 6.1.1

Agenda Item:	11.2.2.3

Document for:	Discussion

# TP revision on Section 6.1.1

### 6.1.1	General Aspects

This clause describes, in general terms, how to utilize an irregular Channel Bandwidth by deploying the “larger channel Bandwidth” method.

The premise idea is that the system is configured with the larger channel bandwidth, as indicated in the broadcast System Information, or UE is reconfigured to the larger channel bandwidth in connected mode, but the actual number of scheduled RBs is restricted so that it matches actual spectrum allocation ensuring sufficiently large guard bands.

The smaller channel bandwidth may be used in UL in case UE UL emissions for the larger channel bandwidth are not compliant to regulations. The UL channel bandwidth is assumed to be the smaller channel bandwidth in this study.

Figure 6.1.1-1: Using the next larger channel bandwidth (example for 7MHz).

NOTE:	It should be checked further whether it is possible to configure the next larger channel so that it goes over the band edge and which implications it has.

NOTE:  The UE UL channel bandwidth is assumed to be the next-smaller channel bandwidth (in MHz) in this study.