3GPP TSG-RAN4 Meeting # 103-e	R4-2210841

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR out of band gain |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters_core |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The out of band gain section has general text in the minimum requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | The General section has limited text followed by an explanation of out of band gain in the minimum requirement section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information is not in the correct location. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4, 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of change>

## 6.4	Out of band gain

### 6.4.1 	General

Out of band gain refers to the gain of the repeater outside the passband.

The intended use of a repeater in a system is to amplify the in-band signals and not to amplify signals outside of the passband.

In the intended application of the repeater, the out of band gain is less than lowest expected the coupling loss to the nearest source of emissions.

### 6.4.2 	Minimum requirement

The intended use of a repeater in a system is to amplify the in-band signals and not to amplify signals outside of the passband.

In the intended application of the repeater, the out of band gain is less than lowest expected the coupling loss to the nearest source of emissions.

The gain outside the passband shall not exceed the maximum level specified in table 6.4.2-1 or table 6.4.2-2, where:

-	f_offset_CW is the offset between the outer channel edge frequency of the outer channel in the passband and a CW signal.

Table 6.4.2-1: Out of band gain limits 1 for bands below 2496 MHz

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0,2 f_offset_CW < 1,0 MHz | 60 dB |
| 1,0 f_offset_CW < 5,0 MHz | 45 dB |
| 5,0 f_offset_CW < 10,0 MHz | 45 dB |
| 10,0 MHz f_offset_CW | 35 dB |

Table 6.4.2-2: Out of band gain limits 1 for bands above 2496 MHz

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| [0,2] < f_offset_CW < 4,0 MHz | 60 dB |
| 4,0 < f_offset_CW < 15,0 MHz | 45 dB |
| 15,0 MHz < f_offset_CW | 35 dB |

< next change >

## 7.4	OTA out of band gain

### 7.4.1 General

Out of band gain refers to the gain of the repeater outside the passband.

The intended use of a repeater in a system is to amplify the in-band signals and not to amplify signals outside of the passband.

In the intended application of the repeater, the out of band gain is less than lowest expected the coupling loss to the nearest source of emissions.

### 7.4.2 Minimum requirement

The intended use of a repeater in a system is to amplify the in-band signals and not to amplify signals outside of the passband.

In the intended application of the repeater, the out of band gain is less than lowest expected the coupling loss to the nearest source of emissions.

The gain outside the passband shall not exceed the maximum level specified in table 7.4.2-1, where:

-	f_offset_CW is the offset between the outer channel edge frequency of the outer channel in the passband and a CW signal.

Table 7.4.2-1: Out of band gain limits 1

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 68 dB |
| 150 MHz  f_offset_CW < 400 MHz | 55 dB |
| 400 MHz  f_offset_CW < f_offset_max | 35 dB |

< end of change >