3GPP TSG-RAN WG4 Meeting #103-e	R4-2210896

# Electronic Meeting, 9th – 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR Updates to test parameters for NR Rel-16 UE requirements (38.101-4, Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf NR_L1enh_URLLC-perf 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There is wrong csi-ReportingBand configuration for Multiple PMI with 16TX Enhanced Type II Codebook. The csi-ReportingBand should be “1111111111111” for FDD 15kHz case and “11111111111111” for TDD 30kHz case. - The propagation condition for test case 7.2.2.2.3-3 is still within brackets. - In table 11.1.7.1.1-1, the number of transmitted UE should be n as Note 1. However, the parameter” 0 ≤ i ≤ n” indicate that there is n+1 transmitted UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the csi-ReportingBand configuration for Multiple PMI with 16TX Enhanced Type II Codebook. - Removing the brackets from propagation condition. - Modify the paramter “0 ≤ i ≤ n” to “0 ≤ i < n” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsistence between the specification 38.101-4 and RAN 4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.6, 6.3.2.2.6, 6.3.3.1.6, 6.3.3.2.6, 7.2.2.2.3, 11.1.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

##### 6.3.2.1.6	Multiple PMI with 16TX Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.1.6-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.6-2.

Table 6.3.2.1.6-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.2.1.6-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 2.2 |

<END OF THE CHANGE 1>

# <START OF THE CHANGE 2>

##### 6.3.2.2.6	Multiple PMI with 16Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.2.6-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.6-2.

Table 6.3.2.2.6-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 11111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.2.2.6-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 2.2 |

# <END OF THE CHANGE 2>

# <START OF THE CHANGE 3>

##### 6.3.3.1.6	Multiple PMI with 16Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.3.1.6-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.6-2.

Table 6.3.3.1.6-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.3.1.6-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 2.2 |

# <END OF THE CHANGE 3>

# <START OF THE CHANGE 4>

##### 6.3.3.2.6	Multiple PMI with 16Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.3.2.6-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.6-2.

Table 6.3.3.2.6-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 11111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.3.2.6-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 2.2 |

# <END OF THE CHANGE 4>

# <START OF THE CHANGE 5>

##### 7.2.2.2.3	Minimum requirements for PDSCH Mapping Type B

The performance requirements are specified in Table 7.2.2.2.3-3, with the addition of test parameters in Table 7.2.2.2. 3-2 and the downlink physical channel setup according to Annex C.5.1. The purpose is to verify the performance of PDSCH Type B scheduling.

The test purposes are specified in Table 7.2.2.2.3-1.

Table 7.2.2.2.3-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH mapping Type B performance under 2 receive antenna conditions | 1-1 |

Table 7.2.2.2.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
| PDSCH configuration | Mapping type |  | Type B |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 0 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |

Table 7.2.2.2.3-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH. 5-1.2 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | 1.3 |

# <END OF THE CHANGE 5>

# <START OF THE CHANGE 6>

##### 11.1.7.1.1	Minimum requirement

The purpose of this test is to verify the maximum number of HARQ processes per TTI supported by the V2X UE.

The minimum requirement is specified in Table 11.1.7.1.1-2 with the test parameters specified in Table 11.1.7.1.1-1.

Table 11.1.7.1.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i < n (Note 1,2) |
| Sidelink UE i, 0 ≤ i < n | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern |  | {2} |
|  | Time gap between initial transmission and retransmission | Slots | [n (Note 3)] |
|  | Timing offset (Note 4) | s | 0 |
|  | Frequency offset (Note 5) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
|  | Redundancy version coding sequence |  | {0,2} |
| PSFCH resource period |  | Slots | 1 |
| Note 1: n is the number of HARQ process UE can support (based on IE harq-RxProcessSidelink) Note 2: When n = 16 or 24, sidelink UEs transmit one by one circularly for every slot; When n=32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, and the 32nd UE transmits signal in the first slot but in the second subchannel; When n=48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 17 UEs transmit signal in the same slot as the first 17 UEs but in the second subchannel; When n=64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal in the same slot as the first 2 UEs in the third subchannel Note 3: k = n if n < 32, otherwise k = 31 Note 4: Time offset of transmitted Sidelink UE signal is with respect to GNSS reference timing. Note 5: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.7.1.1-2: Minimum performance

| Test num. | Bandwidth (MHz) / Subcarrier spacing(kHz) | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 / 30 | R.PSSCH.2-1.5 | AWGN | 5 | 10.9 |

# <END OF THE CHANGE 6>