3GPP TSG-RAN4 Meeting #103e	R4-2210927

# Electronic Meeting, 09 May - 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TDD PDSCH CRS-IM demod requirements for DSS Scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define requirements for CRS-IM demod requirements for DSS Scenario |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new sections for requirements for TDD CRS-IM demod requirements for DSS Scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be defined for TDD CRS-IM demod requirements for DSS Scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.x (New) 5.2.3.2.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2209695 |  |  |  |  |  |  |  |  |

<Start of change 1>

##### 5.2.2.2.x	Minimum requirements for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

The performance requirements are specified in Table 5.2.2.2.x-4, with the addition of test parameters in Table 5.2.2.2.x-2 for the serving cell and Table 5.2.2.2.x-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.x-1.

Table 5.2.2.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell. | 1-1 |

Table 5.2.2.2.x-2: Tests parameters for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate Matchin (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | Hz | 20 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier |  |  |  |

Table 5.2.2.2.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause TBA | As specified in clause TBA |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN and Network-based CRS interference mitigation configured for neighboring LTE cells |  |  |  |  |

Table 5.2.2.2.x-4: Minimum performance for Rank 1

| Test num. |  | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) |  | Modulation format and code rate |  | TDD UL-DL pattern |  | Propagation condition |  | Correlation matrix and antenna configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNR (dB) |  |
| 1-1 | R.PDSCH.1-1.3 TDD |  | 20 / 15 |  | 16QAM, 0.48 |  | FR1.15-1 |  | TDLA30-10 |  | 4x2, ULA Low |  | 70 |  | TBA |  |

<End of change 1>

<Start of change 2>

##### 5.2.3.2.x	Minimum requirements for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

The performance requirements are specified in Table 5.2.3.2.x-4, with the addition of test parameters in Table 5.2.3.2.x-2 for the serving cell and Table 5.2.3.2.x-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.x-1.

Table 5.2.3.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 4 receive antenna conditions with CRS rate matching configured for the serving cell. | 1-1 |

Table 5.2.3.2.x-2: Test parameters for the serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate Matchin (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | Hz | 20 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier |  |  |  |

Table 5.2.3.2.x-3: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause TBA | As specified in clause TBA |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN and Network-based CRS interference mitigation configured for neighboring LTE cells |  |  |  |  |

Table 5.2.3.2.x-4: Minimum performance for Rank 1

| Test num. |  | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) |  | Modulation format and code rate |  | TDD UL-DL pattern |  | Propagation condition |  | Correlation matrix and antenna configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNR (dB) |  |
| 1-1 | R.PDSCH.1-1.3 TDD |  | 20 / 15 |  | 16QAM, 0.48 |  | FR1.15-1 |  | TDLA30-10 |  | 4x4, ULA Low |  | 70 |  | TBA |  |

<End of change 2>