3GPP TSG-RAN WG4 Meeting #103-e                              R4-2211220

Electronic Meeting, 9 May– 20 May, 2022

Agenda item:	9.12.5

Source:	Qualcomm

Title:	TP for 38.101-5 on frequency error

Release:	Rel-17

Document for:	Approval

# 1	Introduction

We provided a text proposal for frequency error for satellite UE in this paper that included the behavior documented in 38.300 section 16.14.2.

# 2	Text Proposal

<start of TP>

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to TS38.300 clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment. ]

Requirement will be verified for at least two cases of which one has zero doppler conditions.

<end of TP>