3GPP TSG-RAN WG4 Meeting #103-e	R4-2209262

# Electronic Meeting, 9 - 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1089 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on Introduction of completed SUL band combinations into TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R17-Core |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the TR 38.862, it’s agreed to simplify the configuration table. Thus, the configuration table format is simplified in Table 5.5C-1, 5.5C-2, 5.5C-3 and 5.5C-4. Correcting SUL configuration table. - To introduce SUL configurations as below SUL_n79C-n83A for CA_n28A_SUL_n79C-n83A SUL_n41C-n83A for CA_n28A_SUL_n41C-n83A SUL_n79C-n80A for CA_n3A_SUL_n79C-n80A SUL_n78C-n80A for CA_n3A_SUL_n78C-n80A SUL_n78C-n84A for CA_n1A_SUL_n78C-n84A SUL_n79C-n95A SUL_n79C-n80A SUL_n41C-n95A - To introduce SUL band combinations as below CA_n28A-n79A_SUL_n41A-n83A CA_n28A-n41A_SUL_n79A-n83A CA_n79A_SUL_n41A-n97A CA_n41A_SUL_n79A-n97A |  |  |  |  |  |  |  |  |
| Summary of change: |  | the configuration table format is simplified in Table 5.5C-1, 5.5C-2, 5.5C-3 and 5.5C-4. - To introduce SUL configurations as below SUL_n79C-n83A for CA_n28A_SUL_n79C-n83A SUL_n41C-n83A for CA_n28A_SUL_n41C-n83A SUL_n79C-n80A for CA_n3A_SUL_n79C-n80A SUL_n78C-n80A for CA_n3A_SUL_n78C-n80A SUL_n78C-n84A for CA_n1A_SUL_n78C-n84A SUL_n79C-n95A SUL_n79C-n80A SUL_n41C-n95A - To introduce SUL band combinations as below CA_n28A-n79A_SUL_n41A-n83A CA_n28A-n41A_SUL_n79A-n83A CA_n79A_SUL_n41A-n97A CA_n41A_SUL_n79A-n97A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The simplified configuration table can not be implemented. SUL configurations and SUL band combinations can’t be introduced into R17 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2C, 5.5C, 6.2C.2, 7.3C.3.2.2, 7.3C.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n24-n992 | n24, n99 |
| SUL_n41-n80 | n41, n80 |
| SUL_n41-n81 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n952 | n41, n95 |
| SUL_n41-n972 | n41, n97 |
| SUL_n41-n982 | n41, n98 |
| SUL_n41-n992 | n41, n99 |
| SUL_n48-n992 | n48, n99 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n77-n992 | n77, n99 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n84 | n79, n84 |
| SUL_n79-n952 | n79, n95 |
| SUL_n79-n972 | n79, n97 |
| SUL_n79-n982 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-2: Operating SUL band combination with intra-band non-contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41(*)-n992 | n41, n99 |
| SUL_n48(*)-n992 | n48, n99 |
| SUL_n77(*)-n992 | n77, n99 |
| SUL_n78(*)-n862 | n78, n86 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in table 5.5C-2. |  |

Table 5.2C-3: Operating SUL band combination with intra-band contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n80 | n41, n80 |
| SUL_n41-n83 | n41, n83 |
| SUL_n41-n95 | n41, n95 |
| SUL_n78-n80 | n78, n80 |
| SUL_n78-n84 | n78, n84 |
| SUL_n79-n80 | n79, n80 |
| SUL_n79-n83 | n79, n83 |
| SUL_n79-n95 | n79, n95 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-4: Operating SUL band combination with inter-band CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_SUL_n78-n80 | n1, n78, n80 |
| CA_n1_SUL_n78-n84 | n1, n78, n84 |
| CA_n3_SUL_n41-n80 | n3, n41, n80 |
| CA_n3_SUL_n78-n80 | n3, n78, n80 |
| CA_n3_SUL_n79-n80 | n3, n79, n80 |
| CA_n28_SUL_n41-n83 | n28, n41, n83 |
| CA_n28_SUL_n79-n83 | n28, n79, n83 |
| CA_n41_SUL_n79-n80 | n41, n79, n80 |
| CA_n41_SUL_n79-n83 | n41, n79, n83 |
| CA_n41_SUL_n79-n97 | n41, n79, n97 |
| CA_n79_SUL_n41-n80 | n41, n79, n80 |
| CA_n79_SUL_n41-n83 | n41, n79, n83 |
| CA_n79_SUL_n41-n97 | n41, n79, n97 |
| CA_n28-n79_SUL_n41-n83 | n28, n41, n79, n83 |
| CA_n28-n41_SUL_n79-n83 | n28, n41, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

<<End of Change>>

<<Start of Change>>

5.5C	Configurations for SUL

The configuration tables for SUL describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5. , where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5.

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n24A-n99A | n24 | 5, 10 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n41A-n80A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n41A-n81A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n41A-n83A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n41A-n95A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n95 | 5, 10, 15 |  |
| SUL_n41A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| SUL_n41A-n98A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n41A-n99A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n48A-n99A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n77A-n80A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
| SUL_n77A-n84A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n77A-n99A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n78A-n80A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n78A-n81A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n78A-n82A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n82 | 5, 10, 15, 20 |  |
| SUL_n78A-n83A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n83 | 5, 10, 15, 20 |  |
|  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n78A-n84A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
|  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| SUL_n78A-n86A | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n86 | 5, 10, 15, 20 |  |
| SUL_n79A-n80A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n79 | 40, 50, 60, 80, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n79A-n81A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n79A-n83A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n79A-n84A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n79A-n95A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n95 | 5, 10, 15 |  |
| SUL_n79A-n97A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  | n79 | 40, 50, 60, 80, 100 | 1 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n79 | See n79 channel bandwidths in Table 5.3.5-1 for each carrier | BCS4 and BCS5 |
|  | n97 | See n97 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| SUL_n79A-n98A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

Table 5.5C-2: Supported channel bandwidths per SUL band combination with intra-band non-contiguous CA

| SUL band combination with intra-band non-contiguous CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SUL_n41(2A)-n99A | SUL_n41A-n99A | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n99 | 5, 10 |  |
| SUL_n48(2A)-n99A | SUL_n48A-n99A | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n99 | 5, 10 |  |
| SUL_n77(2A)-n99A | SUL_n77A-n99A | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n99 | 5, 10 |  |
| SUL_n78(2A)-n86A | SUL_n78A-n86A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n86 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 5.5C-3: Supported channel bandwidths per SUL band combination with intra-band contiguous CA

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SUL_n41C-n80A | SUL_n41A-n80A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 | 0 |
|  | SUL_n41C-n80A | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n41C-n83A | SUL_n41A-n83A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 | 0 |
|  | SUL_n41C-n83A | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n41C-n95A | SUL_n41A-n95A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 | 0 |
|  | SUL_n41C-n95A | n95 | 5, 10, 15 |  |
| SUL_n78C-n80A | SUL_n78A-n80A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 | 0 |
|  | SUL_n78C-n80A | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n78C-n84A | SUL_n78A-n84A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 | 0 |
|  | SUL_n78C-n84A | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| SUL_n79C-n80A | SUL_n79A-n80A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  | SUL_n79C-n80A | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n79C-n83A | SUL_n79A-n83A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  | SUL_n79C-n83A | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n79C-n95A | SUL_n79A-n95A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 | 0 |
|  | SUL_n79C-n95A | n95 | 5, 10, 15 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 5.5C-4: Supported channel bandwidths per SUL band combination with inter-band CA

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A_SUL_n78A-n84A | SUL_n78A-n84A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A_SUL_n78C-n84A | SUL_n78A-n84A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | SUL_n78C-n84A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A_SUL_n41A-n80A | SUL_n41A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_SUL_n41C-n80A | SUL_n41A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_SUL_n78A-n80A | SUL_n78A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_SUL_n78C-n80A | SUL_n78A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | SUL_n78C-n80A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_SUL_n79A-n80A | SUL_n79A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_SUL_n79C-n80A | SUL_n79A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | SUL_n79C-n80A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n28A_SUL_n41A-n83A | SUL_n41A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_SUL_n41C-n83A | SUL_n41A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  | SUL_n41C-n83A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_SUL_n79C-n83A | SUL_n79A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  | SUL_n79C-n83A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41A_SUL_n79A-n80A | SUL_n79A-n80A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A_SUL_n79A-n83A | SUL_n79A-n83A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41A_SUL_n79A-n97A | SUL_n79A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n79A_SUL_n41A-n80A | SUL_n41A-n80A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79A_SUL_n41A-n83A | SUL_n41A-n83A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79A_SUL_n41A-n97A | SUL_n41A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n79A_SUL_n41A-n83A | SUL_n41A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A-n41A_SUL_n79A-n83A | SUL_n79A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

<<End of Change>>

<<Start of Change>>

6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| SUL_n41-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n80 | 0.5 |
| SUL_n41-n81 | n41 | 0.3 |
|  | n81 | 0.3 |
| SUL_n41-n83 | n41 | 0.3 |
|  | n83 | 0.3 |
| SUL_n41-n97 | n41 | 0.5 |
|  | n97 | 0.5 |
| SUL_n41-n98 | n41 | 0.5 |
|  | n98 | 0.5 |
| SUL_n41-n99 | n41 | 0.41 |
|  |  | 0.92 |
|  | n99 | 0.3 |
| SUL_n48-n99 | n48 | 0.6 |
|  | n99 | 0.8 |
| SUL_n77-n80 | n77 | 0.8 |
|  | n80 | 0.6 |
| SUL_n77-n84 | n77 | 0.8 |
|  | n84 | 0.6 |
| SUL_n77-n99 | n77 | 0.6 |
|  | n99 | 0.8 |
| SUL_n78-n80 | n78 | 0.8 |
|  | n80 | 0.6 |
| SUL_n78-n81 | n78 | 0.8 |
|  | n81 | 0.6 |
| SUL_n78-n82 | n78 | 0.8 |
|  | n82 | 0.6 |
| SUL_n78-n83 | n78 | 0.8 |
|  | n83 | 0.5 |
| SUL_n78-n84 | n78 | 0.8 |
|  | n84 | 0.3 |
| SUL_n78-n86 | n78 | 0.8 |
|  | n86 | 0.6 |
| SUL_n79-n83 | n79 | 0.8 |
|  | n83 | 0.5 |
| SUL_n79-n97 | n79 | 0.8 |
|  | n98 | 0.3 |
| SUL_n79-n98 | n79 | 0.8 |
|  | n98 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. |  |  |

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| CA_n1_SUL_n78-n84 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n84 | 0.6 |
| CA_n3_SUL_n41-n80 | n3 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n80 | 0.5 |
| CA_n3_SUL_n78-n80 | n3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| CA_n3_SUL_n79-n80 | n3 | 0.3 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| CA_n28_SUL_n41-n83 | n28 | 0.3 |
|  | n41 | 0.3 |
|  | n83 | 0.3 |
| CA_n28_SUL_n79-n83 | n28 | 0.5 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |
| CA_n41_SUL_n79-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| CA_n41_SUL_n79-n83 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |
| CA_n41_SUL_n79-n97 | n41 | 0.5 |
|  | n79 | 0.5 |
|  | n97 | 0.5 |
| CA_n79_SUL_n41-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| CA_n79_SUL_n41-n83 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |
| CA_n79_SUL_n41-n97 | n41 | 0.5 |
|  | n79 | 0.5 |
|  | n97 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

Table 6.2C.2-3: ΔTIB,c for SUL band combination (Four bands)

| Band combination for SUL | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n28-n79_SUL_n41-n83 | n28 | 0.3 |
|  | n41 | 0.3 |
|  | n79 | 0.5 |
|  | n83 | 0.3 |
| CA_n28-n41_SUL_n79-n83 | n28 | 0.3 |
|  | n41 | 0.3 |
|  | n79 | 0.5 |
|  | n83 | 0.3 |

<<End of Change>>

<<Start of Change>>

7.3C.3	ΔRIB,c for SUL

7.3C.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

7.3C.3.2	SUL band combination

For the UE which supports SUL band combiantion, the minimum requirement for reference sensitivity in clause 7.3C.2 shall be increased by the amount given in ΔRIB,c defined in clause 7.3C.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

7.3C.3.2.1	ΔRIB,c  for two bands

Table 7.3C.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| SUL_n41-n80 | n41 | 0.5 (note) |
| SUL_n41-n95 | n41 | 0.2 |
| SUL_n41-n98 | n41 | 0.2 |
| SUL_n48-n99 | n48 | 0.5 |
| SUL_n77-n80 | n77 | 0.5 |
| SUL_n77-n84 | n77 | 0.5 |
| SUL_n77-n99 | n77 | 0.5 |
| SUL_n78-n80 | n78 | 0.5 |
| SUL_n78-n81 | n78 | 0.5 |
| SUL_n78-n82 | n78 | 0.5 |
| SUL_n78-n83 | n78 | 0.5 |
| SUL_n78-n84 | n78 | 0.5 |
| SUL_n78-n86 | n78 | 0.5 |
| SUL_n79-n83 | n79 | 0.5 |
| SUL_n79-n97 | n79 | 0.5 |
| SUL_n79-n98 | n79 | 0.5 |
| NOTE: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

7.3C.3.2.2	ΔRIB,c  for three bands

Table 7.3C.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n1_SUL_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n3_SUL_n41-n80 | n41 | 0.5 (note) |
| CA_n3_SUL_n78-n80 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n3_SUL_n79-n80 | n79 | 0.5 |
| CA_n28_SUL_n41-n83 | n28 | 0.2 |
| CA_n28_SUL_n79-n83 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n41_SUL_n79-n80 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n41_SUL_n79-n83 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n41_SUL_n79-n97 | n41 | 0 |
|  | n79 | 0.8 |
| CA_n79_SUL_n41-n80 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n79_SUL_n41-n83 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n79_SUL_n41-n97 | n41 | 0 |
|  | n79 | 0.8 |
| NOTE: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

7.3C.3.2.3	ΔRIB,c  for four bands

Table 7.3C.3.2.3-1: ΔRIB,c due to SUL (four bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n28-n79_SUL_n41-n83 | n28 | 0.2 |
|  | n41 | 0 |
|  | n79 | 0.8 |
| CA_n28-n41_SUL_n79-n83 | n28 | 0.2 |
|  | n41 | 0 |
|  | n79 | 0.8 |

<<End of Change>>