3GPP TSG-RAN4 #104-e	R4-2211540

Electronic Meeting, 15th –26th Aug 2022

Title:	Views on channel model for FR2-2

Source:			Anritsu Corporation

Agenda Item:			9.14.8.1

Document for:			Approval

# 1.	Introduction

WF on general and BS aspects for FR2-2 demodulation requirements [2], and WF on UE demodulation performance requirements definition for 52.6 - 71 GHz [3] were approved at the RAN4 #103-e meeting in May. In this paper we would like to discuss on the possible delay profile, tap resolution and channel model based on the minimum requirement in TS38.101-2 and feasibility of our current hardware in the test equipment.

Below is the extract from the WFs [2][3] for reference.

---Extract from WF R4-2210664 [2]---

  - Channel model for requirements definition

Issue 1-3-1: How to consider the delay profile for TDLA channel model for FR2-2?

FFS the delay profile details.

Issue 1-3-2: How to consider the minimum tap resolution for TDL channel model?

Invite TE vendors to confirm the minimum tap resolution for TDL channel model they could deliver.

- Proposal 1: 2ns delay resolution

- Proposal 2: 2.5ns delay resolution

- Proposal 3: TBA

New Issue 1-3-3: Channel model for UL requirements

| SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 480 | 400 | 4 | TDLA10-650 | FFS 960 | 400 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |
|  | 400 | 4 | TDLA10-650 |  | FFS 1600 | 4 | TDLA10-650 |  | FFS 2000 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |

Note 1: Interested companies can provide simulation results for other cases not included in above table, as needed.

---Extract from WF R4-2210665 [3]---

  - Sub-topic 1-2 Channel Model

Issue 1-2-1/1-2-2: TDL Channel Model and RMS Delay Spread

- RAN4 will specify demodulation requirements covering TDL-A10 (baseline) and TDL-D10 (for specific test cases)

  - FFS whether TDL-A20 needed or not

Issue 1-2-3: Max Doppler Frequency

- TDL-D with 200Hz Max doppler

- TDL-A channel:

- Option 1: 200Hz

- Option 2: 650Hz

- Option 3: both 200Hz, and 650Hz

  - Sub-topic 1-3 SCS/CBW Combinations

Issue 1-3-1: 960 kHz SCS

- Option 1: Define the requirements

- Option 2: Do not define the requirements

Issue 1-3-2: Whether to define requirements with 1600 MHz CBW for 480 kHz SCS

- Option 1: Yes

- Option 2: No

Issue 1-3-3: CBW for 960 kHz SCS (if agreed to define requirements in Issue 1-3-1)

- Option 1: 400MHz only

- Option 2: Both 400MHz and 2000MHz

Issue 1-3-4: Whether to update number of PRBs for 960kHz/2000MHz to 156 (if agreed to define requirements in Issue 1-3-1, 1-3-3)

- If requirements for 960kHz SCS are agreed to be defined, update number of PRBs after RF session makes the final decision on number of PRBs.

---- End of extract ----

# 2.	Discussion

  - Considerations on the delay profile from TE PoV

As commented in [1] at the #103-e meeting, due to the limitation of the bandwidth with the test equipment (i.e. 400 MHz), our achievable tap resolution is 2.5 ns. Thus we need to merge taps for the delay profiles. Examples of the delay profile are shown below.

Observation 1: Due to the limitation of BW (i.e. 400 MHz) with the test equipment, there is a need to merge taps for the delay profiles.

Table 2.1-1: Example of delay profile for TDLA5

| Tap # | Delay [ns] | Power [dB] | Fading distribution | Example of proposed delay [ns] |
| --- | --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh | 0 |
| 2 | 1.5 | 0 | Rayleigh |  |
| 3 | 2.5 | -5.1 | Rayleigh | 3.5 |
| 4 | 3.5 | -5.1 | Rayleigh |  |
| 5 | 4 | -9.6 | Rayleigh |  |
| 6 | 8.5 | -8.2 | Rayleigh | 8.5 |
| 7 | 11 | -13.1 | Rayleigh | 12 |
| 8 | 12.5 | -11.5 | Rayleigh |  |
| 9 | 17.5 | -11.0 | Rayleigh | 17.5 |
| 10 | 22.5 | -16.2 | Rayleigh | 22.5 |
| 11 | 25 | -16.6 | Rayleigh | 25 |
| 12 | 48.5 | -26.2 | Rayleigh | 48.5 |

Table 2.1-2: Example of delay profile for TDLA10

| Tap # | Delay [ns] | Power [dB] | Fading distribution | Example of proposed delay [ns] |
| --- | --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh | 0 |
| 2 | 3.5 | 0 | Rayleigh | 2.5 |
| 3 | 5 | -5.1 | Rayleigh | 5 |
| 4 | 6.5 | -5.1 | Rayleigh |  |
| 5 | 8.5 | -9.6 | Rayleigh | 8.5 |
| 6 | 16.5 | -8.2 | Rayleigh | 16.5 |
| 7 | 21.5 | -13.1 | Rayleigh | 21.5 |
| 8 | 25 | -11.5 | Rayleigh | 25 |
| 9 | 35 | -11.0 | Rayleigh | 35 |
| 10 | 45 | -16.2 | Rayleigh | 45 |
| 11 | 50 | -16.6 | Rayleigh | 50 |
| 12 | 96.5 | -26.2 | Rayleigh | 96.5 |

There is one point that we need to confirm on the influence of the merge. Due to the reduction of tap number, RMS delay spread would have some increase compared to a delay of original PDP. In a case of examples above, RMS delay spread becomes 5.46 ns for TDLA5 and 10.24ns for TDLA10.

Observation 2: Influence of the merge of taps needs to be confirmed if it is acceptable. For example, the RMS delay spread may become 5.46ns for TDLA5 and 10.24ns for TDLA10.

2.2 Minimum tap (delay) resolution from TE PoV

As mentioned above, considering the current hardware limit at baseband in our test equipment, i.e. 400 MHz BW, 1/400 MHz = 2.5 ns is the achievable minimum tap resolution.

Proposal 1: Apply 2.5ns delay resolution at FR2-2.

2.3 Channel model

Next we consider presumable channel model. As shown in TS38.101-2 Table 5.3.2-1 [4], mandatory SCS support in Rel-17 specification is 120 kHz, and SCS 480 kHz and 960 kHz are optional. Hence the channel models for 480 kHz and 960 kHz shall also be optional.

Observation 3: SCS for 480 kHz and 960 kHz are optional in TS 38.101-2.

Proposal 2: Channel models for 480 kHz and 960 kHz SCS shall be optional.

From the viewpoint of the TE hardware limitation, channel BW 1600 MHz and 2000 MHz need more time to study the feasibility.

Observation 4: Channel BW 1600 MHz and 2000 MHz need more time to study the feasibility from TE PoV.

Considering the observations above, presumable channel models are shown below.

| SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 480 Note 1 | 400 | 4 | TDLA10-650 | FFS 960 Note 1 | 400 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |
|  | 400 | 4 | TDLA10-650 |  | FFS 1600 | 4 | TDLA10-650 |  | FFS 2000 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |
| Note 1: Optional in Rel-17. |  |  |  |  |  |  |  |  |  |  |  |

2.4 TDL Channel Model and RMS Delay Spread

TDLA10 and TDLD10 models are feasible by the test equipment except for the point that we have a limitation of the tap resolution. (i.e. There is a need to merge some taps.)

Observation 5: TDLA10 and TDLD10 channel models are feasible by the test equipment.

2.5 Max Doppler frequency

There is no strong view on the max doppler frequency from the TE PoV.

Observation 6: No strong views on the max doppler frequency from the TE implementation PoV.

- Conclusion

In this paper we discussed on the possible delay profile, tap resolution and channel model based on the minimum requirement in TS38.101-2 and feasibility of our current hardware in the test equipment.

Observation 1: Due to the limitation of BW (i.e. 400 MHz) with the test equipment, there is a need to merge taps for the delay profiles.

Observation 2: Influence of the merge of taps needs to be confirmed if it is acceptable. For example, the RMS delay spread may become 5.46ns for TDLA5 and 10.24ns for TDLA10.

Proposal 1: Apply 2.5ns delay resolution at FR2-2.

Observation 3: SCS for 480 kHz and 960 kHz are optional in TS 38.101-2.

Proposal 2: Channel models for 480 kHz and 960 kHz SCS shall be at least optional.

Observation 4: Channel BW 1600 MHz and 2000 MHz need more time to study the feasibility from TE PoV.

Observation 5: TDLA10 and TDLD10 channel models are feasible by the test equipment.

Observation 6: No strong views on the max doppler frequency from the TE implementation PoV.

# 4.	References

[1] R4-2210528, “Email discussion summary for [103-e][325] NR_exto71GHz_Demod_Part1”, Moderator (Intel Corp.), RAN4 #103-e, Electronic meeting

[2] R4-2210664, “WF on general and BS aspects for FR2-2 demodulation requirements”, Moderator (Intel Corp.), RAN4 #103-e, Electronic meeting

[3] R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Moderator (Qualcomm Inc.), RAN4 #103-e, Electronic meeting

[4] TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, v17.6.0, 2022-06