3GPP TSG-RAN WG4 #104-e													R4-2211629

e-Meeting, Aug TBD, 2022

Agenda item:	9.14.3.2

Source: 	Qualcomm Incorporated

Title: 	60GHz UE RX

Document for:	Approval

# Introduction

In this paper we analyze and propose various requirements for the UE RX in n263. In general, the approach is to retain the format of the FR2-1 specified parameters. Values may change based on the performance of the circuits in the frequency range up to 71 GHz.

# Reference sensitivity for intraband contiguous CA

Proposal 1: Use the same values as in FR2-1. For 1600-2000 MHz FR2-2 EIS relaxation dB value as [1.5]

Table 7.3A.2.1-1: EIS Relaxation for CA operation by aggregated channel bandwidth

| Aggregated Channel BW 'BWChannel_CA' (MHz) | (dB) |
| --- | --- |
| BWChannel_CA ≤ 800 | 0 |
| 800 < BWChannel_CA ≤ 1200 | 0.5 |
| 1200 < BWChannel_CA ≤ 1600 | 1.0 |
| 1600 < BWChannel_CA ≤ 2000 | [1.5] |

# Maximum input level for Intra-band contiguous CA

Proposal 2: Use the FR2-1 max input for intra-band contiguous requirement for FR2-2.

Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | Level |
| --- | --- | --- |
| Power summed over transmission bandwidth configurations of all active DL CCs | dBm | -25 (NOTE 2) -27 (NOTE 3) |
| NOTE 1: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2 NOTE 2: Reference measurement channel in each CC is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. NOTE 3: Reference measurement channel is specified in Annex A.3.3.5: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |

# Adjacent channel selectivity

ACS was discussed and agreed to be 21 dB in R4-2202367. Case1 and case2 tables are shown below.

Proposal 3: Agree ACS the Case1 and Case 2 tables based on 21 dB ACS.

Table 7.5-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257, n258, n261 | dB | 23 | 23 | 23 | 23 | N.A. | N.A. | N.A. |
| n259, n260, n262 | dB | 22 | 22 | 22 | 22 | N.A. | N.A. | N.A. |
| n263 | dB | N.A. | 21 | N.A. | 21 | 21 | 21 | 21 |

Table 7.5-2: Adjacent channel selectivity test parameters, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5 dB | REFSENS +35.5 dB | REFSENS +35.5 dB | N.A. | N.A. | N.A. |
| PInterferer for band n259, n260, n262 | dBm | REFSENS + 34.5 dB | REFSENS +34.5 dB | REFSENS +34.5 dB | REFSENS +34.5 dB | N.A. | N.A. | N.A. |
| PInterferer for band n263 | dBm | N.A. | REFSENS +33.5 dB | N.A. | REFSENS +33.5 dB | REFSENS +33.5 dB | REFSENS +33.5 dB | REFSENS +33.5 dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |  |  |  |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 | 800 / -800 NOTE 3 | 1600 / -1600 NOTE 3 | 2000 / -2000 NOTE 3 |

Table 7.5-3: Adjacent channel selectivity test parameters, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Ptxbc for bands n257, n258, n261 | dBm | -46.5 | -46.5 | -46.5 | -46.5 | -N.A. | -N.A. | -N.A. |
| Ptxbc for bands n259, n260, n262 | dBm | -45.5 | -45.5 | -45.5 | -45.5 | -N.A. | -N.A. | -N.A. |
| Ptxbc for band n263 | dBm | -N.A. | -44.5 | -N.A. | -44.5 | -44.5 | -44.5 | -44.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 | 800 / -800 NOTE 2 | 1600 / -1600 NOTE 2 | 2000 / -2000 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern TDD as described in Annex A and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 3: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. NOTE 4: Ptxbc is the power in the transmission bandwidth configuration |  |  |  |  |  |  |  |  |

# Adjacent channel selectivity for Intra-band contiguous CA

Adopt 21 dB ACS for CA as in single CC. Partial tables with the n263 values are shown below.

Proposal4:  Agree ACS the Case1 and Case 2 tables changes based on 21 dB ACS.

Table 7.5A.1-1: Adjacent channel selectivity for intra-band contiguous CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n257, n258, n261 | dB | 23 |
| n259, n260, n262 | dB | 22 |
| n263 | dB | 21 |

Table 7.5A.1-2: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 1

| Rx Parameter | Units | All CA bandwidth Classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| PInterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| PInterferer for band n259, n260, n262 | dBm | Aggregated power + 20.5 |
| PInterferer for band n263 | dBm | Aggregated power + 19.5 |

Table 7.5A.1-3: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 2

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n257, n258, n261 | dBm | - 46.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n259, n260, n262 | dBm | - 45.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n263 | dBm | -44.5 |

# In-band blocking

Extend the FR2-1 in-band blocking requirement and method to FR2-2. Use the values in 7.6.2-1 and add requirements for 800, 1600, and 2000 MHz. To add those, mimic the FIoffset and FInterferer placement as in FR2-1. The part of the table with n263 values is shown below.

Table 7.6.2-1: In band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |  | 800 MHz |  | 1600 MHz |  | 2000 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |  | 800 |  | 1600 |  | 2000 |  |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |  | N.A. |  | N.A. |  | N.A. |  |
| PInterferer for bands n259, n260, n262 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB |  | N.A. |  | N.A. |  | N.A. |  |
| PInterferer for band n263 | dBm | N.A. | REFSENS + 33.5 dB | N.A. | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |  | ≤ -1600 & ≥ 1600 NOTE 5 |  | ≤ -3200 & ≥ 3200 |  | ≤ -4000 & ≥ 4000 |  |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |  | FDL_low + 400 to FDL_high - 400 |  | FDL_low + 800 to FDL_high - 800 |  | FDL_low + 1600 to FDL_high - 1600 |  |

Proposal 5:  Agree the IBB values as shown in the table.

# DL CA In-band blocking

We propose to extend the FR2-1 requirement and method to n263. Pinterferer for n263 to be 19.5 dBm.

Table 7.6A.2.1-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for bands n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for bands n260, n262 | dBm | Aggregated power + 20.5 |
| Pinterferer for band n263 | dBm | Aggregated power + 19.5 |

Proposal 6: For DL CA IBB extend the FR2-1 requirement and method to n263. Pinterferer for n263 to be 19.5 dBm.

# Conclusions

Proposal 1: Use the same values as in FR2-1. For 1600-2000 MHz FR2-2 EIS relaxation dB value as [1.5]

Proposal 2: Use the FR2-1 max input for intra-band contiguous requirement for FR2-2.

Proposal 3: Agree ACS the Case1 and Case 2 tables based on 21 dB ACS.

Proposal4:  Agree ACS the Case1 and Case 2 tables changes based on 21 dB ACS.

Proposal 5:  Agree the IBB values as shown in the table.

Proposal 6: For DL CA IBB extend the FR2-1 requirement and method to n263. Pinterferer for n263 to be 19.5 dBm.

# References

[1] 3GPP TR 38.803 v 14.3.0

[2] R4-2010176