3GPP TSG-RAN WG4 Meeting #104-e                                                                                      R4-2211647

Electronic Meeting, August 15 – August 26, 2022

Source: 	CATT

Title: 	Discussion on the BS receiver conformance testing for 52.6-71GHz

Agenda item:	9.14.5.3

Document for:	Approval

- Introduction

In RAN4#103-e, a WF [1] on BS RF conformance testing for FR2-2 was approved. Some the remaining issues in the WF need to be further discussed

In this contribution, we provide our views on the remaining issues for receiver test.

- Discussion

Frequency range for RX spurious emission and out-of-band blocking

As per the WF [1], the remaining issues concerning frequency range for RX spurious emission are shown as below:

- Sub-topic#3-4 Measurement system frequency capabilities and OOB blocking related / spurious emissions related frequency parameters Agreement: Companies are encouraged to do further study for at least - Upper frequency limit, with and without mixer solutions - Capability to generate OOB blocking interferer signal power - Increasing lower limit of measurement above 30 MHz

Same with the discussion in transmitter conformance testing [2], the max limit frequency point should be slightly higher than maximum Fstep,6 of all FR2-2 band. At the same time, max limit frequency point is also feasible to be measured. The operating band for RX spurious emission is same with operating band for TX spurious emission. So we need to align the maximum limit frequency point for RX with that for TX.

Proposal 1: Align the maximum limit frequency point for RX with that for TX

FR2 Interferer signal step size for blocking

From Table 7.5.2.4.2-1 of TS 38.141-2, the measurement step size for FR2 interferer signal step size for OTA in-band blocking and OTA out-of-band blocking varied with minimum supported BS channel bandwidth.

Table 7.5.2.4.2-1: FR2 Interferer signal step size Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). 50; 15. 100; 30. 200; 60. 400; 60.

The minimum supported BS channel bandwidth for 52.6-71GHz will be larger than 400MHz, such as 800MHz, 1600Hz, and 2000MHz. To reduce testing time, 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz and 1600MHz.

Proposal 2: 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz, 1600MHz, and 2000MHz CBW for OTA in-band blocking and OTA out-of-band blocking.

- Conclusion

This contribution provides analysis on BS receiver conformance testing for 52.6-71GHz. The following proposals are concluded as follows:

Proposal 1: Align the maximum limit frequency point for RX with that for TX.

Proposal 2: 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz, 1600MHz, and 2000MHz CBW for OTA in-band blocking and OTA out-of-band blocking.

- References

- R4-2210638, WF on BS RF conformance testing for FR2-2, ZTE, RAN4#103-e

- R4-2211646, Discussion on the BS receiver conformance testing for 52.6-71GHz, CATT, RAN4#104-e