3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211789

# Electronic Meeting, August 15 – August 26, 2022

Source:	China Telecom

Title:	Discussion on UE MMSE-IRC receiver requirements for both inter-cell and intra-cell inter-user scenarios

Agenda Item:	9.10.1.1

Document for:	Discussion

- Introduction

In the RAN4#103-e meeting, the test parameters for MMSE-IRC receiver were discussed, and the WF was approved in [1] that captures all the agreements and remain issues for both inter-cell and intra-cell inter-user scenarios.

In this paper, we give our views on the remaining open issues.

- Discussion

Issue 1-1-1: Interference modelling in PDCCH region

Status in the WF in [1]:

  - Option 1: All the REs in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports.

  - Option 2: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell.

  - Option 3: Assume PDCCH transmission from interference cells which is non-overlapping with serving cell.

We believe it is not practical to assume that only the PDSCH signals between cells are overlapped in the test under inter-cell interference scenarios.

According to our last meeting’s contribution in [2], the overlapped PDCCH between cells which is the same test configuration with LTE MMSE-IRC tests, will not introduce any PDCCH decoding error issues.

In addition, this issue has been well discussed in the last meeting and option 2 has been technically clear enough and has been accepted by most of the companies.

Considering the above, we have the following proposal:

Proposal 1: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell.

Issue 2-1-3: T-put gain for 2 Rx

Status in the WF in [1]:

  - Option 1: 2

  - Option 2: 1.5

Issue 2-1-5: T-put gain for 4 Rx

Status in the WF in [1]:

  - Option 1: 2.5

  - Option 2: 2

  - Option 3: 1.5

According to all companies’ simulation results summarized in [3], the averaged T-put gain is at least 2.11 for 2Rx, and 2.53 for 4Rx. Therefore, we support T-put gain requirement of 2.0 for 2Rx and 2.5 for 4Rx.

Observation 1: According to all companies’ simulation results summarized in [3], the averaged T-put gain is at least 2.11 for 2Rx, and 2.53 for 4Rx.

Proposal 2: T-put gain requirement of 2.0 for 2Rx and 2.5 for 4Rx.

Issue 3-1-1: SNR deriving rule

Status in the WF in [1]:

RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes X dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within X dB.

  - Option 1: X = 2dB

  - Option 2: X = 2.5dB

In NR Rel-15 SNR requirement derivation, the SPAN of the ideal results was kept within 2dB according to [4] and the same rule is reused for BS PUSCH 256QAM requirement definition.

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. - If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; - Else if the ideal span is larger than [2]dB: - …

For the purpose of having the more accurate SNR requirements, we propose to keep the SPAN of the ideal results within 2.0dB for both inter-cell and intra-cell inter-user requirements.

Observation 2: In NR Rel-15 SNR requirement derivation, the SPAN of the ideal results was kept within 2dB and the same rule is reused for BS PUSCH 256QAM requirement definition.

Proposal 3: Keep the SPAN of the ideal results within 2.0dB for both inter-cell and intra-cell inter-user requirements.

- Conclusions

In this paper, we give our views on the remaining open issues.

Proposal 1: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell.

Observation 1: According to all companies’ simulation results summarized in [3], the averaged T-put gain is at least 2.11 for 2Rx, and 2.53 for 4Rx.

Proposal 2: T-put gain requirement of 2.0 for 2Rx and 2.5 for 4Rx.

Observation 2: In NR Rel-15 SNR requirement derivation, the SPAN of the ideal results was kept within 2dB and the same rule is reused for BS PUSCH 256QAM requirement definition.

Proposal 3: Keep the SPAN of the ideal results within 2.0dB for both inter-cell and intra-cell inter-user requirements.

- References

R4-2210681, WF on PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC, Ericsson, RAN4 #103-e, May 2022.

R4-2209414, On PDSCH requirements for UE MMSE-IRC receiver for inter-cell interference suppression, China Telecom. RAN4 #103-e, May 2022.

R4-2209443, Summary of simulation results for Inter-cell MMSE-IRC CQI reporting, Ericsson, RAN4 #103-e, May 2022.

R4-1904713, WF for requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4#90-bis