3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211848

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.3.3

Source:	Apple

Title:	On RRM measurement relaxations for RedCap UE

Document for:	Discussion

# Introduction

In RAN4 #103e meeting, the WF[1] for RedCap RRM measurement relaxations requirements was agreed, but there are still some open issues as followings,

Issue 2-2-3: Higher priority inter-frequency measurement Relaxation when only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ Agreement (2nd round): No consensus

RAN4 received couple LSs from RAN2 in last meeting and RAN4 clarification action is needed, e.g.,

- R2-2206418 Reply LS to RAN4 on RRM relaxation

In this contribution, we discuss the above remaining issues as well as the issues raised in RAN2 incoming LSs.

# Discussion on RAN4 remaining issues

RAN2 agreed that,

When the criteria are satisfied, e.g. only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RRM relaxation for higher priority frequency would be performed similar to equal/lower priority frequency. The detailed relaxation methods are up to RAN4.

However, we don’t understand when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, why equal/lower priority frequency needs to be measured. In RAN4, the only conclusion we can have based on the incoming LS is: for inter-frequency measurement relaxation requirement of R17 RedCap, if only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the same relaxation shall be applied as the case when both Rel-17 criteria are satisfied. Since Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is a condition for cell center, and the R16/R17 not-at-cell-edge condition shall be more relaxed than cell center condition, i.e., if UE is at cell center, it shall have at least same or even more relaxed RRM measurement than when R16/R17 not-at-cell-edge condition is met.

In Previous RAN4 WF (R4-2009265) for R16 power saving WI, it was agreed that,

- RAN4 not specify intra/inter-frequency requirement when the threshold configured for not-at-cell-edge condition is higher than that for cell center condition of “SnonIntraSearchP” or “SnonIntraSearchQ”. - This rule also applies in “SIntraSearchP” or “SIntraSearchQ”.

That also showed that the cell center condition of SnonIntraSearchP and SnonIntraSearchQ has higher RSRP than not-at-cell-edge condition. Thus, we propose that,

Proposal 1: For inter-frequency measurement relaxation requirement of R17 RedCap, if only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the same relaxation shall be applied as the case when both Rel-17 criteria are satisfied.

# Discussion on RAN2 LS

In RAN2 R2-2206418 (Reply LS to RAN4 on RRM relaxation), it was proposed that,

RAN2 would like to thank RAN4 for the LS on RRM relaxation. Based on further RAN4 progress, RAN2 discussed the coexistence of Rel-16 and Rel-17 RRM relaxation, and achieved the following agreements: Regarding the coexistence cases of Rel-17 and Rel-17 RRM relaxation: - Simultaneous configuration of R16 not-at-cell-edge criterion and R17 stationary criterion for idle/inactive mode is a valid configuration from the network’s point of view, where the network supports RRM relaxation for both R16 and R17 UEs in idle/inactive mode. - From signalling’s point of view, any R16 RRM relaxation criterion and any R17 RRM relaxation criterion for idle/inactive mode can be configured in a same cell at a same time, as independent criteria (i.e., without requiring a UE to fulfil both the R16 and the R17 criteria in order to relax its RRM measurements). - If combined with a not-at-cell-edge criterion (i.e. for Rel-17 stationary & Rel-17 not-at-cell-edge), the R17 stationary criterion can only be combined with the R17 not-at-cell-edge criterion, not with the R16 one. In this way, RAN2 kindly request RAN4 to consider support cases#8 and case #9 in the previous LS [R2-2204487/R4-2207109]. But it is up to RAN4 to make the final decision on whether support case#8 and case#9, for example, considering other reasons from RAN4 (if any). Besides, RAN2 also discussed the relaxation criteria, and achieved the following agreements: Regarding the relaxation criteria: - RAN2 assume to change the “Srxlev” for stationary criterion to “SS-RSRP” in RRC_CONNECTED, pending confirmation by RAN4. Please find the below TP (which has been agreed and will be implemented in TS 38.331) for your reference: #### 5.7.4.4 Relaxed measurement criterion for a stationary UE The relaxed measurement criterion for a stationary UE is met when: - (SrxlevSS-RSRP RefStationaryConnected – SS-RSRPSrxlev) < SSearchDeltaP-StationaryConnected, Where: - Srxlev SS-RSRP = current Srxlev value of the PCell current L3 RSRP measurement of the PCell based on SSB (dB). - Srxlev SS-RSRP RefStationaryConnected = reference SS-RSRP Srxlev value of the PCell cell (dB), set as follows: - At the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or - If (Srxlev SS-RSRP–Srxlev SS-RSRP RefStationaryConnected) > 0; or - If the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected: - The UE shall set the value of Srxlev SS-RSRP RefStationaryConnected to the current Srxlev SS-RSRP value of the serving cell.. RAN2 would like to confirm with RAN4 whether it is reasonable, or whether there is any concern from RAN4.

Regarding the criteria of R16 and R17, the agreement from last meeting WF[1] is:

| No | Rel-16 relaxation criterion | Rel-17 relaxation criterion | Applicability |
| --- | --- | --- | --- |
| 7 | Rel-16 low mobility | Rel-17 stationary | Allowed |
| 8 | Rel-16 not-at-cell-edge | Rel-17 stationary | FFS |
| 9 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary | Allowed |
| 10 | Rel-16 low-mobility | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 11 | Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 12 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |

Based on RAN2 LS, simultaneous configuration of R16 not-at-cell-edge criterion and R17 stationary criterion for idle/inactive mode is a valid configuration from the network’s point of view, where the network supports RRM relaxation for both R16 and R17 UEs in idle/inactive mode. So, we think case 8 is also an available configuration from network perspective. However, RAN2 also emphasized that, from signalling’s point of view, any R16 RRM relaxation criterion and any R17 RRM relaxation criterion for idle/inactive mode can be configured in a same cell at a same time, as independent criteria (i.e., without requiring a UE to fulfil both the R16 and the R17 criteria in order to relax its RRM measurements).

In previous RAN4 meeting, it was agreed that (R4-2207105 WF),

Relaxation when multiple criteria of Rel-16 and Rel-17 are satisfied UE is allowed to meet the requirements that are the most relaxed out of Rel-16 and Rel-17 requirements

Thus, in case 8 configuration, if UE can meet both Rel-16 not-at-cell-edge and Rel-17 stationary conditions, the UE is allowed to meet the requirements that are the most relaxed out of Rel-16 not-at-cell-edge and Rel-17 stationary RRM relaxation requirements.

Proposal 2: It’s allowed to configure both Rel-16 not-at-cell-edge and Rel-17 stationary criteria to UE, i.e., case 8. If UE can meet both Rel-16 not-at-cell-edge and Rel-17 stationary conditions, the UE is allowed to meet the requirements that are the most relaxed out of Rel-16 not-at-cell-edge and Rel-17 stationary RRM relaxation requirements.

Regarding the TP from RAN2, RAN2 assumed to change the “Srxlev” for stationary criterion to “SS-RSRP” in RRC_CONNECTED, pending confirmation by RAN4. Since the purpose of such SS-RSRP measurement is to verify if UE is in a stationary condition and each cell may have multiple SSBs, it would be more accurate to say SS-RSRP is the current L3 RSRP measurement of the PCell based on an identical SSB. That means, on a same SSB (Tx beam) of serving cell, if UE cannot see big RSRP fluctuation during a period, UE can be assumed as stationary.

Proposal 3: the SS-RSRP in stationary condition TP from RAN2 LS shall be revised as:

SS-RSRP = current L3 RSRP measurement of the PCell based on an identical SSB (dB)

# Conclusion

In this contribution, we discuss the above remaining issues as well as the issues raised in RAN2 incoming LSs.

Proposal 1: For inter-frequency measurement relaxation requirement of R17 RedCap, if only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the same relaxation shall be applied as the case when both Rel-17 criteria are satisfied.

Proposal 2: It’s allowed to configure both Rel-16 not-at-cell-edge and Rel-17 stationary criteria to UE, i.e., case 8. If UE can meet both Rel-16 not-at-cell-edge and Rel-17 stationary conditions, the UE is allowed to meet the requirements that are the most relaxed out of Rel-16 not-at-cell-edge and Rel-17 stationary RRM relaxation requirements.

Proposal 3: the SS-RSRP in stationary condition TP from RAN2 LS shall be revised as:

SS-RSRP = current L3 RSRP measurement of the PCell based on an identical SSB (dB)

# References

[1] R4-2210596, WF on eDRX and RRM measurement relaxations requirements for Redcap, vivo, RAN4 #103e.