3GPP TSG-RAN4 Meeting #104-e	R4-2211892

# Online, 15 – 26 August, 2022

Agenda item:	9.9.1.1

Source:	Apple

Title:	Discussion on RAN2 LS on LocationMeasurementIndication contents

Document for:	Discussion

# Introduction

In RAN2#118e, an LS was approved to RAN4 on LocationMeasurementIndication contents:

In the LS from R4-2206789, it has been mentioned that the following has been agreed in R4 For PRS measurements within measurement gaps when a Pre-configured MG is provided by the network: - RAN4 considers that a Pre-configured MG that is not always ON (activated) as determined from the signalling provided by the network, or if no such signalling is provided (i.e. autonomous rules are applied to determine the status of the Pre-configured MG), is not sufficient to perform PRS measurements. - In the above scenario, the UE will inform the network that it is going to start/stop PRS measurements with the configured Pre-configured MG by initiating the existing LocationMeasurementIndication procedure.. Regarding the above agreements in R4, when the UE autonomously activates/deactivates the pre-configured measurement gap by sending the RRC message LocationMeasurementIndication, RAN2 would like to understand: - Question1: - whether the request for configuration in LocationMeasurementIndication can be un-ambiguously mapped to a certain pre-configured measurement gap? - whether anything needs to be added to the current contents of the LocationMeasurementIndication? - Question2: whether the UE should only autonomously activate/deactivate the pre-configured measurement gap indicated by LocationMeasurementIndication?

RAN4 is expected to provide answers to the two questions from RAN2. In this contribution, we provide our views and prepare draft reply after discussion.

# Discussion

The first bullet under question 1 implies that UE can be configured with multiple gap patterns, including Pre-MG. However, RAN4 doesn’t support such configuration. Specifically, joint configuration of Pre-MG and other gap patterns is not supported in R17. Although in concurrent gap design, UE can be configured with two gap patterns with one of which is configured as per-UE gap used for PRS measurement, Pre-MG is not in scope. In other word, if the UE is configured with Pre-MG (either as a FR1 gap or as a per UE gap), UE cannot be configured with other gap patterns in FR1.

Observation 1: the first bullet under question 1 implies that UE can be configured with multiple gap patterns. However, joint configuration of Pre-MG and other gap pattern in the same frequency range is not supported in R17 RAN4 requirements.

Proposal 1: inform RAN2 that joint configuration of Pre-MG and other gap pattern in the same frequency range is not supported in R17 RAN4 requirements.

In the second bullet under question 1, RAN2 asked whether anything needs to be added to the current contents of the LocationMeasurementIndication. It is unclear to us whether the second bullet only targets at Pre-MG. If so, According to observation 1, in RAN4 scope there is only one Pre-MG is configured so that no additional information is needed in LocationMeasurementIndication. RAN4 requirements of joint configuration is to be introduced in R18 according to R18 MG further enhancement WID, which seems to be out of scope of this LS in our understanding. However, in RAN4 LS to RAN2 approved in previous RAN4 meetings, RAN4 asked RAN2 to take into account the joint working among pre-configured gap, concurrent gap and NCSG in the signalling design. Therefore, if from RRC configuration point of view UE can be configured with Pre-MG and other gap patterns, it is beneficial for network and UE to have same understanding on which gap pattern is to be used for PRS measurement. From this perspective, additional information in LocationMeasurementIndication about gap pattern information is useful.

If R17 concurrent gap is also considered in the second bullet, then we believe it is beneficial to also include gap pattern information LocationMeasurementIndication to let NW know which gap pattern to be used for PRS measurement.

Observation 2: if the second bullet under Q1 only targets at Pre-MG, no additional information is needed in LocationMeasurementIndication from R17 RAN4 scope point of view. However, if RAN2 R17 RRC design allows UE to be configured with Pre-MG and other gap pattern, it is beneficial to let UE indicate NW via LocationMeasurementIndication on which gap pattern is to be used for PRS measurement.

Observation 3: if the second bullet under Q1 also targets at R17 concurrent gaps, it is beneficial to let UE indicate NW via LocationMeasurementIndication on which gap pattern is to be used for PRS measurement.

Proposal 2: inform RAN2 it is beneficial to consider additional information in LocationMeasurementIndication to allow UE to indicate which gap pattern is to be used for PRS measurement when multiple gap patterns are configured.

The second question is whether the UE should only autonomously activate/deactivate the pre-configured measurement gap indicated by LocationMeasurementIndication?

As discussed in RAN4#103e, some company mentioned that in legacy LocationMeasurementIndication procedure, it is up to network whether to respond the request from UE on measurement gap for PRS measurement. It makes sense to us if 3GPP keep the network flexibility unchanged in R17. In other word, UE shall not autonomously activate/deactivate the Pre-MG when it is requested to perform PRS measurement. The expected UE behavior is that UE shall inform network that it is requested to perform PRS measurement and what is the expected gap pattern to be used for the PRS measurement. Then UE shall wait for network command for the next step. There are several choices at network side, e.g. 1) network can update the flag in the active BWP to activate Pre-MG for PRS measurement. 2) network can switch active BWP to the one according to which Pre-MG shall be activated, such that UE can use Pre-MG for PRS measurement. 3) network can reconfigure a legacy gap for the PRS measurement, e.g. with proper MGL and MGRP. 4) network can even ignore such request. If no gap is configured or gap is not sufficient for PRS measurement, UE will not perform PRS measurement.

In summary, out point is that UE shall not autonomously activate/deactivate the pre-configured measurement gap when it is requested to perform PRS measurement.

Proposal 3: inform RAN2 that UE shall not autonomously activate/deactivate the pre-configured measurement gap when it is requested to perform PRS measurement. UE only needs to inform network that it is requested to perform PRS measurement and the expected gap configuration/pattern. It is up to network how to respond the request from UE.

# Conclusion

In this contribution, we discuss the RAN2 LS on LocationMeasurementIndication contents and measurement gap parameters. After discussion, the following conclusions are provided:

Observation 1: the first bullet under question 1 implies that UE can be configured with multiple gap patterns. However, joint configuration of Pre-MG and other gap pattern in the same frequency range is not supported in R17 RAN4 requirements.

Proposal 1: inform RAN2 that joint configuration of Pre-MG and other gap pattern in the same frequency range is not supported in R17 RAN4 requirements.

Observation 2: if the second bullet under Q1 only targets at Pre-MG, no additional information is needed in LocationMeasurementIndication from R17 RAN4 scope point of view. However, if RAN2 R17 RRC design allows UE to be configured with Pre-MG and other gap pattern, it is beneficial to let UE indicate NW via LocationMeasurementIndication on which gap pattern is to be used for PRS measurement.

Observation 3: if the second bullet under Q1 also targets at R17 concurrent gaps, it is beneficial to let UE indicate NW via LocationMeasurementIndication on which gap pattern is to be used for PRS measurement.

Proposal 2: inform RAN2 it is beneficial to consider additional information in LocationMeasurementIndication to allow UE to indicate which gap pattern is to be used for PRS measurement when multiple gap patterns are configured.

Proposal 3: inform RAN2 that UE shall not autonomously activate/deactivate the pre-configured measurement gap when it is requested to perform PRS measurement. UE only needs to inform network that it is requested to perform PRS measurement and the expected gap configuration/pattern. It is up to network how to respond the request from UE.

# References

- R2-2206388, LS on LocationMeasurementIndication contents and measurement gap parameters, RAN2

# Annex: draft LS to RAN2

3GPP TSG-RAN4#104e Meeting	R4-22xxxxx

# Electronic, 15th– 26th August, 2022

Title:	Reply LS on LocationMeasurementIndication contents and measurement gap parameters

Response to:	R2-2206388

Release:	Rel-17

Work Items:	NR_pos_enh-Core, NR_MG_enh-Core

Source:	RAN WG4

To:	RAN WG2

CC:

Contact Person:

#### Name:	     Qiming Li

E-mail Address:	li_qiming@apple.com

Attachment:

1. Overall Description:

RAN4 thanks RAN2 for the LS R2-2206388 on LocationMeasurementIndication contents and measurement gap parameters. In RAN4#104e RAN4 discussed the questions raised by RAN2 and reached the following agreements:

- Question1:

  - whether the request for configuration in LocationMeasurementIndication can be un-ambiguously mapped to a certain pre-configured measurement gap?

[RAN4 feedback]: the first bullet under question 1 implies that UE can be configured with multiple gap patterns. However, joint configuration of Pre-MG and other gap pattern in the same frequency range is not supported in R17 RAN4 requirements

  - whether anything needs to be added to the current contents of the LocationMeasurementIndication?

[RAN4 feedback]: in RAN4 common understanding it is beneficial to consider additional information in LocationMeasurementIndication to allow UE to indicate which gap pattern/configuration is expected to be used for PRS measurement when multiple gap patterns are configured.

- Question2: whether the UE should only autonomously activate/deactivate the pre-configured measurement gap indicated by LocationMeasurementIndication?

[RAN4 feedback]: it is RAN4 understanding that UE shall not autonomously activate/deactivate the pre-configured measurement gap when it is requested to perform PRS measurement. UE only needs to inform network that it is requested to perform PRS measurement and the expected gap configuration/pattern. It is up to network how to respond the request from UE.

2. Actions:

To RAN WG2

ACTION:

- RAN4 respectfully asks RAN2 to take above agreements into consideration.

3. Date of Next RAN WG4 Meetings:

RAN4#104bis-e										10 – 19 October							Electronic Meeting