3GPP TSG-RAN4 Meeting #104-e	R4-2211898

# Online, 15 – 26 August, 2022

Agenda item:	9.9.2.3

Source:	Apple

Title:	On NCSG performance

Document for:	Discussion

# Introduction

NCSG performance part design was discussed in the last RAN4#103e meeting. Corresponding agreements are captured in the approved WF [1].

- NCSG related functionalities to be tested Agreement: - Measurement with NCSG on deactivated SCC - Intra-frequency measurement with NCSG - Inter-frequency measurement with NCSG - Inter-RAT measurement with NCSG - Interruption on the serving cells - Test cases applicability Agreement: - If a UE supports per-FR NCSG, it only needs to pass test case with per-FR NCSG, otherwise, UE only needs to pass test case with per-UE NCSG. - Test configuration Agreement: - RAN4 will not introduce test case for NCSG in dual connectivity. - For FR1 tests, NCSG pattern #0 is used. FFS on whether other NCSG pattern is needed. - For FR2 tests, NCSG pattern is FFS. - Test case list Agreement: No.; Test case; Clause; Responsibility. 1; Event triggered reporting test on SCC with deactivated Scell in FR1; A.6.6.X3.4; Ericsson. 2; Event triggered reporting test on SCC with deactivated Scell in FR2; A.7.6.X3.3; MTK. 3; Event triggered reporting test on inter-frequency in FR1; A.6.6.X3.2; Huawei. 4; Event triggered reporting test on inter-frequency in FR2; A.7.6.X3.2; CATT. 5; Event triggered reporting test on inter-RAT LTE in FR1; A.6.6.X3.3; Intel. 6; Event triggered reporting test on intra-frequency in FR1; A.6.6.X3.1; OPPO. 7; Event triggered reporting test on intra-frequency in FR2; A.7.6.X3.1; ZTE.

As can be observed, there are still some FFS in test configuration. In this contribution, we will further discuss the issues and provide corresponding proposals.

# Discussion

Test configuration

Agreement:

- RAN4 will not introduce test case for NCSG in dual connectivity.

- For FR1 tests, NCSG pattern #0 is used. FFS on whether other NCSG pattern is needed.

- For FR2 tests, NCSG pattern is FFS.

The first FFS is besides NCSG pattern #0, whether other NCSG pattern is needed for FR1 tests. To keep test case design simple, we think testing NCSG pattern #0 is enough, even though NCSG pattern #1 is also a mandatory pattern. Similarly, both legacy gap patterns #0 and #1 are mandatory. However, legacy gap pattern #0 is widely used in legacy RRM test case while gap pattern #1 is never used.

Regarding FR2 tests, it was agreed in [1] that:

- NCSG pattern #13 and #14 are mandatory for UE supports per-FR MG or UE capable of FR2 standalone mode.

It is straightforward that UE which needs to pass FR2 test shall support FR2 standalone mode. Thus NCSG pattern #13 and #14 are supported. Similar with FR1 tests design, we think testing NCSG pattern #13 is enough.

Proposal 1: for FR1 tests, only NCSG pattern #0 is used.

Proposal 2: for FR2 test, only NCSG pattern #13 is used.

# Conclusion

In this contribution, we further discuss NCSG test configuration. After discussion the following conclusions are provided:

Proposal 1: for FR1 tests, only NCSG pattern #0 is used.

Proposal 2: for FR2 test, only NCSG pattern #13 is used.

# References

- R4-2210588, WF on NCSG, Apple