3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212046

Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2444 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to pre-configured Pos gap activation limitation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  |  | Date: |  | 2022-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following agreement has been achieved in the last meeting and the spec needs to be updated accordingly. Agreements: - For PRS measurement with POS MG: - the PRS requirements apply provided that only one POS MG is activated, and the RRM requirements do not apply for RRM measurement that requires MG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For RRM requirements applicability, specify that only one pre-configured Pos gap could be activated simultaneously. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing gap-based positioning requirements may incorrectly apply when more than one pre-configured Pos gaps are activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

9.9.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements defined in TS 38.215 [4], including RSTD, PRS-RSRP, UE Rx-Tx time difference, NR E-CID, and PRS-RSRPP measurements.

9.9.1.1	General Aspects of Gap-based Measurement

For gap-based RSTD, PRS-RSRP, UE Rx-Tx time difference, and PRS-RSRPP measurements, the requirements in clauses 9.9.2, 9.9.3, 9.9.4 and 9.9.x1 apply provided:

-	the UE is configured or pre-configured with measurement gaps

-	if the measurement gap is pre-configured, the gap must be activated throughout the measurement period, and

-	if pre-configured Pos gaps are configured via posMeasGapPreConfig, only one gap can be activated simultaneously, and

-	if concurrent measurement gaps are configured, all positioning frequency layers are associated with only one of the measurement gaps, and

-	if the UE does not support PRS measurements with per-FR gaps, the configured or pre-configured gap used to perform the PRS measurements must be of per-UE type, and

-	No active BWP switching occurs during the measurement gaps for PRS measurement, and

All measurement requirements specified in clause 9.9.2, 9.9.3 and 9.9.4 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

UE is only required to measure PRS resources that are fully or partially overlapped with measurement gaps, and the requirements in clause 9.9.2, 9.9.3, 9.9.4 and 9.9.x1 are applicable to PRS resources that are fully or partially overlapped with measurement gaps.

A PRS resource is considered to be fully (partially) overlapped with measurement gaps if all (some) of its instances are overlapped with a measurement gap occasion. A PRS resource instance is considered to be overlapped with measurement gap occasion if the minimum number of unmuted repetitions of the instance taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD is fully covered by the MGL excluding RF switching time, where the minimum number is given in the accuracy requirements in clause 10.1.23, 10.1.24, 10.1.25 and 10.1.X for RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP, respectively.

When UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

# < End of change 1>