3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212100

Electronic Meeting, 15-26 Aug., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC

Agenda item:	9.10.1.1

Document for:	Discussion

Introduction

During the last RAN4#103-e meeting, good progress was made on the topic of PDSCH demodulation requirements and CQI reporting requirements for inter-cell interference MMSE-IRC. Open topic which were captured in WF[1] are:

- Interference modelling in PDCCH region for defining PDSCH demodulation requirements

- Throughput ratios for CQI requirements definition

In this contribution we will express our views on the captured open topic and open new discussions, if necessary.

Discussion

Interference model for scenario 1, PDSCH demodulation

During the discussions in RAN4#103-e following options were considered for modelling the interference in PDCCH region [1]:

Issue 1-1-1: Interference modelling in PDCCH region - Option 1: All the REs in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports. - Option 2: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell. - Option 3: Assume PDCCH transmission from interference cells which is non-overlapping with serving cell.

It is our understanding that PDCCH decoding will not have any impact on the PDSCH demodulation performance at any of the to be specified SNR values. However, assuming non-overlapping PDCCH transmission from interference cells (Option 3) is not practical as it cannot be guaranteed in real world deployments. Option 1 which emulates PDCCH like interference or Option 2 with overlapping PDCCH transmission from interference cells with OCNG in unoccupied REs are the most practical options.

Considering the timeline of this work item, option 1 is the easiest because option 2 will require companies to reach an agreement on defining the PDCCH parameters for option 2

Use option 1 which is to reuse LTE PDSCH IRC test approach of filling unused RE’s in control region with QPSK randomly modulated symbols with random precoding.

T-put ratio for 2 Rx, CQI reporting

From the discussions in RAN4#103-e [1] following are the options for 2 Rx Throughput ratio (γ)

Issue 2-1-3: T-put gain for 2 Rx - Option 1: 2 - Option 2: 1.5

In the last meeting SINR value of -2dB was agreed upon for defining the requirement for 2 Rx. Our simulations result for FDD and TDD setup are captured in [2] for -2 dB. From the captured results it is seen that throughput ration of ~2 is possible for both FDD and TDD requirements.

Use TP ratio (γ) value of 2 (Option 1) for 2 Rx CQI reporting requirements

T-put ratio for 4 Rx, CQI reporting

From the discussions in RAN4#103-e [1] following are the options for 4 Rx Throughput ratio (γ)

Issue 2-1-5: T-put gain for 4 Rx - Option 1: 2.5 - Option 2: 2 - Option 3: 1.5

In the last meeting SINR value of -2dB was agreed upon for defining the requirement for 4 Rx. From the results presented in [2] we can see that Option 1 is not feasible in all scenarios. We prefer Option 2 in order to have same throughput ratio as 2 Rx.

Use TP ratio (γ) value of 2 (Option 2) for 4 Rx CQI reporting requirements

Conclusion

In this contribution we have provided our views on the open topics of PDSCH demodulation rand CQI reporting requirements.

We have made the following observations and proposals:

Interference model for scenario 1, PDSCH demodulation

- Use option 1 which is to reuse LTE PDSCH IRC test approach of filling unused RE’s in control region with QPSK randomly modulated symbols with random precoding.

T-put ratio for 2 Rx, CQI reporting

- Use TP ratio (γ) value of 2 (Option 1) for 2 Rx CQI reporting requirements

T-put ratio for 4 Rx, CQI reporting

- Use TP ratio (γ) value of 2 (Option 2) for 4 Rx CQI reporting requirements

- References

- R4-2210681 - WF on PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC

- R4-2212101 - Simulation results on PDSCH demodulation and CQI requirements for intercell interference MMSE-IRC