3GPP TSG-RAN WG4 Meeting # 104-e														R4-2212171

Electronic Meeting, August 15 – August 26, 2022

Source:	Verizon, Samsung, Ericsson

Title:	TP for TR 38.xxx-xx-xx for single UL CA/DC_n2-n66-n261 (x=1)

Agenda Item:	10.11.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.xxx-xx-xx to introduce CA/DC_n2-n66-n261 with single uplink configuration according to the request in [1].

# 2. Reference

[1]  RP-221877: New WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2)

# 3. Text Proposal

<Start of Text Proposal>

### 5.x	CA_n2-n66-n261

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n2+n66+n261

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n66A-n261A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n66A-n261I | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261I |  |
| CA_n2A-n66A-n261J | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261J |  |
| CA_n2A-n66A-n261K | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261K |  |
| CA_n2A-n66A-n261L | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261L |  |
| CA_n2A-n66A-n261M | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261M |  |
| CA_n2A-n66A-n261(2G) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n66A-n261(G-H) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n66A-n261(A-G-H) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n66A-n261(G-I) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n66A-n261(2H) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n66A-n261(A-G-I) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n66A-n261(H-I) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66A | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n261 | CA_n261(H-I) |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

Table 5.x.1.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n2-n66-n261 | n2 | 0.5 |
|  | n66 | 0.5 |
|  | n261 | 0 |

Table 5.x.1.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n2-n66-n261 | n2 | 0.3 |
|  | n66 | 0.3 |
|  | n261 | 0 |

<End of Text Proposal>

<Start of Text Proposal>

## 6.x	DC_n2-n66A-n261

### 6.x.1	Configurations for DC_n2-n66A-n261

Table 6.x.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n2A-n66A-n261A DC_n2A-n66A-n261I DC_n2A-n66A-n261J DC_n2A-n66A-n261K DC_n2A-n66A-n261L DC_n2A-n66A-n261M | - |
| DC_n2A-n66A-n261(2G) DC_n2A-n66A-n261(G-H) DC_n2A-n66A-n261(A-G-H) DC_n2A-n66A-n261(G-I) DC_n2A-n66A-n261(2H) DC_n2A-n66A-n261(A-G-I) DC_n2A-n66A-n261(H-I) | - |

<End of Text Proposal>