3GPP TSG-RAN WG4 Meeting #104-e	R4-2212198

Online Meeting, Aug 15 – 26, 2022

Agenda item:	9.19.2.2.3

Source: 	Qualcomm Incorporated

Title: 	On performance requirements for PRS-RSRPP

Document for:	Discussion

# Introduction

RAN4 continued discussing performance requirements for PRS-RSRPP in RAN4#103-e. Agreements and remaining issues were documented in a WF [1].

In this brief paper we will discuss the following topics:

- Simulation results for PRS-RSRPP accuracy

# Discussion

Simulation assumptions to derive PRS-RSRPP measurement accuracy requirements were agreed in RAN4#103-e [2].

Simulated accuracy for the two-tap channel model (38.101-4 Annex B.2.4) with parameters a = 1, τd=0.45 µs and fD=5 Hz is shown in the tables below for FR1 with SCS = 15 kHz, 30 kHz and 60 kHz. The reported accuracy does not include RF impairment margin.

Proposal 1: Defne PRS-RSRPP absolute accuracy requirements with 4 samples and number of repetitions ≥ 1 for PRS Es/Iot ≥ -3 dB and -13 dB.

Proposal 2: Defne PRS-RSRPP absolute accuracy requirements with 1 sample and number of repetitions ≥ 1 for PRS Es/Iot ≥ -6 dB.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | ±1.2 | ±1.5 | ±3.2 |
|  |  |  |  | 4 | 1 | ±1.0 | ±1.2 | ±2.4 |
|  | 52 |  | 128 | 1 | 1 | ±1.4 | ±1.5 | ±2.1 |
|  |  |  |  | 4 | 1 | ±0.9 | ±1.4 | ±1.4 |
|  | 104 |  | 64 | 1 | 1 | ±1.3 | ±1.3 | ±1.5 |
|  |  |  |  | 4 | 1 | ±1.3 | ±1.3 | ±1.1 |

Table 1: PRS-RSRPP simulation results for FR1, SCS = 15 kHz, 2-tap channel.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | ±1.5 | ±1.6 | ±3.2 |
|  |  |  |  | 4 | 1 | ±0.8 | ±1.2 | ±2.3 |
|  | 48 |  | 64 | 1 | 1 | ±0.8 | ±1.6 | ±2.3 |
|  |  |  |  | 4 | 1 | ±0.8 | ±0.8 | ±1.3 |
|  | 132 |  | 32 | 1 | 1 | ±0.7 | ±0.7 | ±1.9 |
|  |  |  |  | 4 | 1 | ±0.7 | ±0.7 | ±1.5 |

Table 2: PRS-RSRPP simulation results for FR1, SCS = 30 kHz, 2-tap channel.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 60 | 64 | 1 | 1 | ±1.6 | ±1.6 | ±3.3 |
|  |  |  |  | 4 | 1 | ±0.8 | ±0.8 | ±2.3 |
|  | 64 |  | 32 | 1 | 1 | ±0.8 | ±1.6 | ±2.5 |
|  |  |  |  | 4 | 1 | ±0.7 | ±1.6 | ±1.8 |
|  | 132 |  | 16 | 1 | 1 | ±0.8 | ±0.8 | ±1.8 |
|  |  |  |  | 4 | 1 | ±0.8 | ±0.8 | ±1.4 |

Table 3: PRS-RSRPP simulation results for FR1, SCS = 60 kHz, 2-tap channel.

# Conclusions

Proposal 1: Defne PRS-RSRPP absolute accuracy requirements with 4 samples and number of repetitions ≥ 1 for PRS Es/Iot ≥ -3 dB and -13 dB.

Proposal 2: Defne PRS-RSRPP absolute accuracy requirements with 1 sample and number of repetitions ≥ 1 for PRS Es/Iot ≥ -6 dB.

# References

[1] R4-2210600, WF on NR Positioning Enhancements (Part 1), RAN4#103-e

[2] RP-2211061, Simulation assumptions for PRS-RSRPP, RAN4#103-e