3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212215

# Electronic Meeting, August 15 – 26, 2022

Source:	LG Electronics

Title:	Discussion on TCI assumption for RSSI measurement for FR2-2

Agenda item:	9.14.6.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on QCL-ed assumption for intra/inter-frequency RSSI measurement for FR2-2 based on RAN1 LS [1].

Discussion

The TCI assumption for intra/inter-frequency RSSI measurement for FR2-2 was agreed in RAN1 [1].

[Agreements in RAN1#108e] For the QCL Type-D of L3-RSSI measurement for unlicensed operation in FR2-2, if explicit TCI state is configured, use the TCI state. - Use the QCL type-D of the latest PDSCH reception or latest CORESET monitoring for RSSI measurement, if the explicit TCI state is not configured. - A dynamic update mechanism for TCI-State in RMTC-Config is not further considered in Rel.17 - The explicit TCI state is configured at least in RMTC-Config - Note: For inter-frequency L3-RSSI measurement, the TCI state configured is with respect to the target frequency TCI state - Note2: For a given L3-RSSI measurement occasion, the UE needs to identify the last PDSCH reception or last configured CORESET monitoring (which ever is later) before the L3-RSSI measurement occasion, and use the QCL Type-D of that for L3-RSSI monitoring

The general principle for QCL-ed assumption for RSSI measurement is

- if explicit TCI state is configured, UE can use the TCI state,

- otherwise, UE can use the QCL type-D of the latest PDSCH reception or latest CORESET monitoring similar to CLI measurement.

Therefore, RAN4 should define QCL-ed assumption for intra/inter-frequency RSSI measurement based on this principle. In that sense, the recommended text proposal from RAN1 generally is acceptable except for the yellow highlighted sentence in the inter-frequency RSSI measurement below. It is not clear under which condition this sentence assumes that UE QCL-ed assumption. “If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell” is one of the conditions of inter-frequency RSSI measurement which has been already defined in RAN4 spec, and UE just uses the TCI state provided in RMTC configuration for QCL type-D like the cyan highlighted sentence already stated. So, to clearly capture QCL-ed assumption for inter-frequency RSSI measurement in the RAN4 specification, the yellow highlighted sentence is redundant.

## *** < Beginning of TP#0 for TS 38.133 v17.5.0> *** ### 9.2A.7 Intra-frequency RSSI and Channel occupancy measurements #### 9.2A.7.1 Intra-frequency RSSI measurements An RSSI measurement is defined as an intra-frequency measurement provided that the RSSI measurement bandwidth is fully contained within the current carrier bandwidth of the UE. The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more serving carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity. For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier. *** < Unchanged parts are ommitted> *** ### 9.3A.8 Inter-frequency RSSI measurements An RSSI measurement is defined as an inter-frequency measurement provided that the RSSI measurement bandwidth is not contained within the current carrier bandwidth of the UE. The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more inter-frequency carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity. The requirements apply if rmtc-SubframeOffset [2] is configured. For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell, UE can assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state of the active BWP of the carrier on which the RMTC configuration is provided. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided. *** < End of TP#0 for TS 38.133 v17.5.0> ***

- Proposal 1: For QCL-ed assumption performing RSSI measurement,

- in FR2-2 intra-frequency,

  - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier.

- in FR2-2 inter-frequency,

  - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided.

Conclusion

In this contribution, we provide our views on QCL-ed assumption for intra/inter-frequency RSSI measurement for FR2-2, and we propose

- Proposal 1: For QCL-ed assumption performing RSSI measurement,

- in FR2-2 intra-frequency,

  - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier.

- in FR2-2 inter-frequency,

  - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided.

Reference

[1] R1-2205582, “LS on TCI assumption for RSSI measurement for FR2-2,” Qualcomm.