3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212227

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Discussion on PUCCH demodulation requirements for NR extended to 71GHz

Agenda item:	9.14.8.3.2

Document for:	Discussion

# Introduction

In previous RAN4#103-e meeting, companies discussed the issues for FR2-2 PUCCH demodulation requirement. Following agreements and open issues are captured in WF [1].

Issue 2-3-1: How many PRBs and symbols are considered for each of the PUCCH format?

Introduce below test cases

- For Format 0:

  - 1RB /1 symbol

  - FFS for [10/16] RB/1 symbol

- For format 1:

  - 1 RB/14 symbols

  - [10/16] RB/14 symbols

- For format 2: Reusing FR2-1

- For format 3: Reusing FR2-1

- For format 4:

  - 1 RB/14 symbols

  - [10/16] RB/14 symbols

Agreed Issue 2-3-4: Parameter list for PUCCH performance requirements (Agreements in the last meeting)

Introduce below test cases

- For Format 0:

  - Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1

  - Test metric: ACK missed detection rate < 1%

- For format 1:

  - Intra-slot frequency hopping: enabled

  - Test metric: ACK missed detection rate < 1% and NACK to ACK probability < 0.1%

- For format 2:

  - Intra-slot frequency hopping: enabled

  - Test metric: ACK missed detection rate < 1%, UCI BLER < 1%

- For format 3:

  - Intra-slot frequency hopping: enabled

  - Test metric: UCI BLER < 1%

- For format 4:

  - Intra-slot frequency hopping: enabled

  - Test metric: UCI BLER < 1%

In this contribution, open issues of FR2-2 PUCCH demodulation are analyzed.

# Discussion

## 2.1	Normal requirement

For format 4, it is not agreed on the DM-RS configuration for it. Note that, FR2 have manufactory declaration for PUCCH format 4 with additional DM-RS configuration. And FR2-1 have defined separate requirement for different DM-RS configurations. To keep the same test coverage as FR2-1, both DM-RS 1+0 and 1+1 could be considered for FR2-2 normal PUCCH format 4 requirements.

Proposal 1: Define FR2-2 normal PUCCH format 4 demodulation requirements for both DM-RS 1+0 and 1+1 configurations.

## 2.2	Multi-PRB requirement

Based on our preliminary analysis on link budget for FR2-2 UL OTA test [3], there might be 10dB more margins could be achieved by reducing CATR chamber area from 20m2 to 2m2. In that case, 100MHz BW is very high possibility to be supported by BS demodulation tests, and 16 PRB could be feasible for PUCCH multi-PRB requirements with up to 480kHz SCS.

Proposal 2: Take 16 PRB for FR2-2 multi-PRB PUCCH demodulation requirements.

For format 4, it is agreed to use 14 symbols for the requirement. The open issue is the DM-RS configuration. As enhanced feature, we think only DM-RS 1+1 configuration should be enough for the multi-PRB requirement. The performance of 1+0 configuration could be covered by normal requirement. The same approach is used in FR2-1 PUCCH enhanced requirements.

Proposal 3: Only define DM-RS 1+1 configuration for FR2-2 multi-PRB PUCCH format 4 demodulation requirements.

# Conclusions

Proposal 1: Define FR2-2 normal PUCCH format 4 demodulation requirements for both DM-RS 1+0 and 1+1 configurations.

Proposal 2: Take 16 PRB for FR2-2 multi-PRB PUCCH demodulation requirements.

Proposal 3: Only define DM-RS 1+1 configuration for FR2-2 multi-PRB PUCCH format 4 demodulation requirements.

# References

[1]	R4-2210664, WF for NR extended to 71GHz BS demodulation, Intel, RAN4#103-e

[2]	R4-2212230, Simulation results for FR2-2 PUCCH demodulation requirement, Ericsson

[3]	R4-2212465, On general aspects relevant for conformance testing up to 71 GHz, Ericsson