3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212235

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Discussion on SAN PUSCH demodulation requirements

Agenda item:	9.11.7.2.1

Document for:	Discussion

# Introduction

In the last RAN4#103-e meeting, companies have discussions on the non-terrestrial network demodulation requirements. Following WF [1] was agreed for SAN PUSCH demodulation requirement issues.

Scope of PUSCH requirements

Agreements

- Not consider 2-step RACH case

### NTN normal PUSCH requirements

SCS/CBW set

Agreements

- One test cases with few PRBs for each SCS

- Reuse Rel-15 test applicability rule defined in clause 8.1.2.1.2 Applicability of requirements for different channel bandwidths

Candidate options

- Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz

- Option 2: 4 PRBs for all SCS

- Other options are not precluded.

Additional antenna configuration besides 2Rx

Agreements

- Both 1Rx and 2Rx shall be considered for NTN SAN demodulation requirements.

- FFS for the test applicability rules.

Modulation order

Agreements

- MCS4 unless any technical issues figured out

Other test parameters

Agreements:

- Use the following test parameters for NTN PUSCH performance requirements definition unless any issues figured out

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

### NTN UL timing adjustment requirements

UE speed

Agreements

- 120km/h

Additional DM-RS position:

Agreements

- pos1

Transform precoding

Candidate options

- Option 1: Same as normal PUSCH

- Option 2: CP-OFDM only

Other test parameters for UL timing adjustment requirement

Agreements

- Agree to use the following test parameters for NTN PUSCH UL TA performance requirements definition unless any issues figured out

| Parameter |  | Value |
| --- | --- | --- |
| Channel bandwidth |  | Same as normal PUSCH |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Depends on normal PUSCH |
|  | Starting PRB index | Based on RB assignment |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | Based on RB assignment |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: Same as PUSCH for NTN-TDLA |
| CP length | Normal |
| A | 15 kHz: 10 ms 30 kHz: 5 ms |
| Dw | Depends on UE speed |

### NTN PUSCH repetition type A requirements

MCS for PUSCH repetition type A

Agreements

- MCS5 in Table 3 same as legacy TN PUSCH repetition type A

Transform precoding for PUSCH repetition type A

Candidate options

- Option 1: Same as normal PUSCH

- Option 2: CP-OFDM only

Other test parameters for PUSCH repetition type A

Agreements

- Agree to use the following parameters for NTN PUSCH repetition type A requirements.

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDL-A, Doppler and delay same as normal PUSCH |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource assignment | RB assignment | Same as normal PUSCH |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

In this contribution, open issues for SAN PUSCH demodulation will be further analyzed.

# Discussion

## 2.1	Normal PUSCH requirement

SCS/CBW set

Based on the link budget analysis [3], it is typical to use a few of PRBs for UL transmission in NTN deployment. There are only 1 or 2 PRBs are considered in link budget calculation, but it is too small for normal scheduling. It might be feasible to take 4 PRBs for PUSCH requirement. In that case, it is possible to apply one requirement for all CBWs.

Proposal 1: Take 4 PRBs for all SCS PUSCH requirements.

Additional antenna configuration besides 2Rx

The Rel-16 applicability rule for different number of antenna connectors are as follows:

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Regarding to the SAN architecture, satellite antenna other than BS antenna are considered as receiver antenna. Furthermore, 1Rx or 2Rx of satellite antenna refers to different polarization types which seems can’t be compatible in current implementation. Then the test could be only applied for declared antenna connector number. If MIMO would be possibly used on satellite in the future, then current statement on covering the lowest and highest connector number could be applied.

Proposal 2: Considering following applicability rule for SAN antenna configuration.

Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy) by same polarization type, the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

## 2.2	UL Timing adjustment requirement

Transform precoding

DFT-s-OFDM is beneficial for NTN deployment to improve coverage. Regarding test coverage, it would be good to introduce requirements for normal PUSCH part. If normal PUSCH requirements include DFT-s-OFDM, there is no need to consider it again in UL timing adjustment requirements due to the same function implementation.

Proposal 3: Consider DFT-s-OFDM in normal PUSCH. Only consider CP-OFDM in UL TA requirement.

Other test parameters

It was agreed that UE speed is 120km/h for SAN UL TA requirement which is the same as Scenario X in normal UL TA. Regarding NTN UE could do pre-compensation on timing in service link, the timing drift caused by satellite motion could be ignored. Then the parameters of Scenario X could be reused here.

Proposal 4: Take Dw  = 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS in SAN UL TA requirement.

## 2.3	PUSCH repetition type A requirement

Transform precoding

Similar as UL TA, only CP-OFDM would be enough to check the performance.

Proposal 5: Only consider CP-OFDM for PUSCH repetition type A requirement.

# Conclusions

Proposal 1: Take 4 PRBs for all SCS PUSCH requirements.

Proposal 2: Considering following applicability rule for SAN antenna configuration.

Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy) by same polarization type, the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Proposal 3: Consider DFT-s-OFDM in normal PUSCH. Only consider CP-OFDM in UL TA requirement.

Proposal 4: Take Dw  = 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS in SAN UL TA requirement.

Proposal 5: Only consider CP-OFDM for PUSCH repetition type A requirement.

# References

[1]	R4-2210662, WF for NTN SAN demodulation requirements, Huawei, HiSilicon

[2]	R4-2212238, Simulation results for SAN PUSCH demodulation requirements, Ericsson

[3] 	TR38.821 v16.0.0