3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212238

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Simulation results for NTN SAN PUSCH demodulation requirements

Agenda item:	9.11.7.2.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NTN NR SAN demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

## 2.1	Normal PUSCH requirement

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Antenna Configuration |  | 1Tx1Rx and 1Tx2Rx |
| Channel model |  | NTN-TDLA NTN-TDLC |
| MCS |  | MCS4 |
| Transform precoding |  | Disabled and enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 4 PRBs and minimum CBW for 15kHz and 30kHz SCS |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Table 2.1-2 Simulation results for AWGN and NLOS channel, 15kHz SCS 4PRBs and 5MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | PRB | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | AWGN | 70% | 4 | A | -1.2 |
|  |  |  | NTN-TDLA100-200 |  |  |  | 2.6 |
|  |  |  | NTN-TDLA150-200 |  |  |  | 2.6 |
|  |  |  | NTN-TDLA100-200 | 70% | 25 | A | 0.8 |
|  |  |  | NTN-TDLA150-200 |  |  |  | 1 |
| 1 | 2 | Normal | AWGN | 70% | 4 | A | -4.2 |
|  |  |  | NTN-TDLA100-200 |  |  |  | -2 |
|  |  |  | NTN-TDLA150-200 |  |  |  | -2 |
|  |  |  | NTN-TDLA100-200 | 70% | 25 | A | -3.2 |
|  |  |  | NTN-TDLA150-200 |  |  |  | -3 |

Table 2.1-3 Simulation results for AWGN and NLOS channel, 30kHz SCS 4PRBs and 10MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | PRB | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | AWGN | 70% | 4 | A | -1.2 |
|  |  |  | NTN-TDLA100-200 |  |  |  | 2.4 |
|  |  |  | NTN-TDLA150-200 |  |  |  | 2.4 |
|  |  |  | NTN-TDLA100-200 | 70% | 24 | A | 0.8 |
|  |  |  | NTN-TDLA150-200 |  |  |  | 0.8 |
| 1 | 2 | Normal | AWGN | 70% | 4 | A | -4.2 |
|  |  |  | NTN-TDLA100-200 |  |  |  | -2 |
|  |  |  | NTN-TDLA150-200 |  |  |  | -2 |
|  |  |  | NTN-TDLA100-200 | 70% | 24 | A | -3.2 |
|  |  |  | NTN-TDLA150-200 |  |  |  | -3 |

Table 2.1-4 Simulation results for LOS channel, 15kHz SCS, PUSCH mapping type A

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | K-factor [dB] | Fraction of maximum throughput | PRB | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC2-200 | 3 | 70 % | 4 | 1.2 |
|  |  |  | NTN-TDLC6-200 |  |  |  | 1.2 |
|  |  |  | NTN-TDLC10-200 |  |  |  | 1.2 |
|  |  |  | NTN-TDLC2-200 | 7 | 70% | 4 | 0 |
|  |  |  | NTN-TDLC6-200 |  |  |  | 0 |
|  |  |  | NTN-TDLC10-200 |  |  |  | 0 |
|  |  |  | NTN-TDLC2-200 | 12 | 70% | 4 | -0.6 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -0.6 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -0.6 |
|  |  |  | NTN-TDLC2-200 | 18 | 70% | 4 | -1 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -1 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -1 |
|  |  |  | NTN-TDLC2-200 | 3 | 70 % | 25 | 0.6 |
|  |  |  | NTN-TDLC6-200 |  |  |  | 0.6 |
|  |  |  | NTN-TDLC10-200 |  |  |  | 0.6 |
|  |  |  | NTN-TDLC2-200 | 7 | 70% | 25 | -0.4 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -0.4 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -0.4 |
|  |  |  | NTN-TDLC2-200 | 12 | 70% | 25 | -1 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -1 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -1 |
|  |  |  | NTN-TDLC2-200 | 18 | 70% | 25 | -1.4 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -1.4 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -1.4 |

Table 2.1-5 Simulation results for LOS channel, 30kHz SCS, PUSCH mapping type A

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | K-factor [dB] | Fraction of maximum throughput | PRB | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC2-200 | 3 | 70 % | 4 | 1 |
|  |  |  | NTN-TDLC6-200 |  |  |  | 1 |
|  |  |  | NTN-TDLC10-200 |  |  |  | 1 |
|  |  |  | NTN-TDLC2-200 | 7 | 70% | 4 | -0.2 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -0.2 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -0.2 |
|  |  |  | NTN-TDLC2-200 | 12 | 70% | 4 | -0.8 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -0.8 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -0.8 |
|  |  |  | NTN-TDLC2-200 | 18 | 70% | 4 | -1 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -1 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -1 |
|  |  |  | NTN-TDLC2-200 | 3 | 70 % | 24 | 0.6 |
|  |  |  | NTN-TDLC6-200 |  |  |  | 0.4 |
|  |  |  | NTN-TDLC10-200 |  |  |  | 0.4 |
|  |  |  | NTN-TDLC2-200 | 7 | 70% | 24 | -0.6 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -0.6 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -0.6 |
|  |  |  | NTN-TDLC2-200 | 12 | 70% | 24 | -1.2 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -1.2 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -1.2 |
|  |  |  | NTN-TDLC2-200 | 18 | 70% | 24 | -1.4 |
|  |  |  | NTN-TDLC6-200 |  |  |  | -1.4 |
|  |  |  | NTN-TDLC10-200 |  |  |  | -1.4 |

Observation 1: Different delay spreads with NTN-TDLA have similar performance for PUSCH with MCS4.

Observation 2: K-factor is the dominate parameter for NTN LOS channel performance. Lower K-factor leads to the worse performance. The performance is almost the same as AWGN when K-factor is larger than 18dB.

# Conclusions

Simulation results for NR SAN PUSCH demodulation requirements are delivered.

# References

[1]	R4-2210662, WF for NTN NR SAN demodulation, Huawei HiSilicon, RAN4#103-e

# Annex A Channel model for NTN

Table A-1 NTN-TDL-A RMS delay spread (resolution 5ns)

| Tap # | Normalized delay | Power in [dB] | Fading distribution | Delay spread |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 100ns | 150ns | 200ns |
| 1 | 0 | 0 | Rayleigh | 0 | 0 | 0 |
| 2 | 1.0811 | -4.675 | Rayleigh | 110 | 160 | 215 |
| 3 | 2.8416 | -6.482 | Rayleigh | 285 | 425 | 570 |
| Recalculated RMS delay [ns] |  |  |  | 149.6 | 222.1 | 298 |

Table A-2 NTN-TDL-C reference model at elevation

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -10.618 | Rayleigh |
| 2 | 14.8124 | -23.373 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 10.224 dB and a mean power of 0 dB. |  |  |  |

Table A-3 NTN-TDL-C channel profile with different delay spread and K-factors

| Channel model | K_factor [dB] | Tap #1 |  |  |  | Tap #2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Delay [ns] | Power [dB] | Delay [ns] | Power [dB] | Delay [ns] | Power [dB] |
| NTN-TDLC2-200 | 3 | 0 | -0.39 | 0 | -3.62 | 15 | -16.37 |
|  | 7 | 0 | -0.39 | 0 | -7.62 | 20 | -20.37 |
|  | 12 | 0 | -0.39 | 0 | -12.62 | 35 | -25.37 |
|  | 18 | 0 | -0.39 | 0 | -18.62 | 70 | -31.37 |
| NTN-TDLC6-200 | 3 | 0 | -0.39 | 0 | -3.62 | 45 | -16.37 |
|  | 7 | 0 | -0.39 | 0 | -7.62 | 65 | -20.37 |
|  | 12 | 0 | -0.39 | 0 | -12.62 | 110 | -25.37 |
|  | 18 | 0 | -0.39 | 0 | -18.62 | 215 | -31.37 |
| NTN-TDLC10-200 | 3 | 0 | -0.39 | 0 | -3.62 | 80 | -16.37 |
|  | 7 | 0 | -0.39 | 0 | -7.62 | 110 | -20.37 |
|  | 12 | 0 | -0.39 | 0 | -12.62 | 185 | -25.37 |
|  | 18 | 0 | -0.39 | 0 | -18.62 | 355 | -31.37 |