3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212240

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Simulation results for NTN SAN PRACH demodulation requirements

Agenda item:	9.11.7.2.3

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the SAN PRACH demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Tow time error tolerances are considered based on the second largest delay tap and the largest delay tap in NTN-TDLA channel.

Table 2-1 Simulation assumptions for NTN PRACH

| PRACH | PRACH SCS | Time error tolerance |  |  |
| --- | --- | --- | --- | --- |
| preamble | (kHz) | AWGN | NTN-TDLA100-200 | NTN-TDLA150-200 |
| 0 | 1.25 | 1.04 us | 1.15 us / 1.33 us | 1.2 us / 1.47 us |
| B4, | 15 | 0.52 us | 0.63 us / 0.81 us | 0.67 us / 0.95 us |
| C2 | 30 | 0.26 us | 0.37 us / 0.55 us | 0.42 us / 0.69 us |

Table 2-2 Test preambles for normal mode in FR1 (Rel-15)

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| A1, A2, A3, | 15 | 23 | 0 | 0 |
| B4, C0, C2 | 30 | 46 | 0 | 0 |

Table 2-3: PRACH missed detection test requirements, 1.25 kHz SCS

| Number of TX | Number of RX | Propagation conditions and correlation matrix (annex G) | Time error tolerance | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| antennas | antennas |  |  |  | Burst format 0 |
| 1 | 1 | AWGN | 1.04 us | 0 | -14 |
|  |  | NTN-TDLA100-200 Low | 1.15 us | 200 Hz | -2 |
|  |  |  | 1.33 us |  | -2 |
|  |  | NTN-TDLA150-200 Low | 1.2 us | 200 Hz | -2.9 |
|  |  |  | 1.47 us |  | -2.9 |
|  | 2 | AWGN | 1.04 us | 0 | -16.6 |
|  |  | NTN-TDLA100-200 Low | 1.15 us | 200 Hz | -9.4 |
|  |  |  | 1.33 us |  | -9.4 |
|  |  | NTN-TDLA150-200 Low | 1.2 us | 200 Hz | -9.6 |
|  |  |  | 1.47 us |  | -9.6 |

Table 2-4: PRACH missed detection test requirements, 15 kHz SCS

| Number of TX | Number of RX | Propagation conditions and correlation matrix (annex G) | Time error tolerance | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| antennas | antennas |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0.52 us | 0 | -16.8 | -11.9 |
|  |  | NTN-TDLA100-200 Low | 0.63 us | 200 Hz | -4.7 | -0.3 |
|  |  |  | 0.81 us |  | -4.7 | -0.3 |
|  |  | NTN-TDLA150-200 Low | 0.67 us | 200 Hz | -5.9 | -1.3 |
|  |  |  | 0.95 us |  | -5.9 | -1.3 |
|  | 2 | AWGN | 0.52 us | 0 | -19.3 | -14.4 |
|  |  | NTN-TDLA100-200 Low | 0.63 us | 200 Hz | -11.6 | -7.2 |
|  |  |  | 0.81 us |  | -11.6 | -7.2 |
|  |  | NTN-TDLA150-200 Low | 0.67 us | 200 Hz | -12.1 | -7.7 |
|  |  |  | 0.95 us |  | -12.1 | -7.7 |

Table 2-5: PRACH missed detection test requirements, 30 kHz SCS

| Number of TX | Number of RX | Propagation conditions and correlation matrix (annex G) | Time error tolerance | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| antennas | antennas |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0.26 us | 0 | -16.7 | -11.8 |
|  |  | NTN-TDLA100-200 Low | 0.37 us | 200 Hz |  |  |
|  |  |  | 0.55 us |  | -6.9 | -2 |
|  |  | NTN-TDLA150-200 Low | 0.42 us | 200 Hz | -7.7 | -3 |
|  |  |  | 0.69 us |  | -7.7 | -3 |
|  | 2 | AWGN | 0.26 us | 0 | -19.2 | -14.3 |
|  |  | NTN-TDLA100-200 Low | 0.37 us | 200 Hz | -11 | -6.3 |
|  |  |  | 0.55 us |  | -12.5 | -7.8 |
|  |  | NTN-TDLA150-200 Low | 0.42 us | 200 Hz | -12.9 | -8.1 |
|  |  |  | 0.69 us |  | -12.9 | -8.1 |

# Conclusions

Simulation results for SAN PRACH demodulation are delivered.

# References

[1]	R4-2210662, WF for NTN NR SAN demodulation, Huawei HI Silicon, RAN4#103-e

# Annex A Channel model delay profile

Table A-1 NTN-TDL-A RMS delay spread (resolution 5ns)

| Tap # | Normalized delay | Power in [dB] | Fading distribution | 100ns | 150ns | 200ns |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh | 0 | 0 | 0 |
| 2 | 1.0811 | -4.675 | Rayleigh | 110 | 160 | 215 |
| 3 | 2.8416 | -6.482 | Rayleigh | 285 | 425 | 570 |