3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212244

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Simulation results for NR coverage enhancement PUCCH demodulation requirements

Agenda item:	9.16.2.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the PUCCH with JCE demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

## 2.1	PUCCH format 1

Table 2.1-1 Simulation parameters for PUCCH format 1 with JCE for FR1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | disabled |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 slots for TDD, 8 slots for FDD |
| Test metric | SNR@NACK->ACK<0.1% SNR@ACK miss<1% |
| JCE cTDW | 2 slots for TDD, 8 slots for FDD |
| JCE aTDW | 2 slots for TDD, 8 slots for FDD |
| TDD UL-DL pattern | 15kHz/30khz SCS: FDD and 7D1S2U, S=6D:4G:4U |
| phase/power error model | disabled |
| Antenna configuration | 1Tx2Rx |

Table 2.1-2 Simulation results for PUCCH format 1 with JCE, 15kHz SCS, 5MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NACK ACK <0.1% | ACK miss <1% |
| 1 | 2 | Normal | TDLA30-10 low | 2 | -6 | -5.3 |
|  |  |  |  | 8 | -12 | -11.3 |

Table 2.1-3 Simulation results for PUCCH format 1 with JCE, 30kHz SCS, 10MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NACK ACK <0.1% | ACK miss <1% |
| 1 | 2 | Normal | TDLA30-10 low | 2 | -5.7 | -5.4 |
|  |  |  |  | 8 | -11.6 | -11.2 |

Table 2.1-4 Simulation parameters for PUCCH format 1 with JCE for FR2

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | disabled |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 slots |
| Test metric | SNR@NACK->ACK<0.1% SNR@ACK miss<1% |
| JCE cTDW | 2 slots |
| JCE aTDW | 2 slots |
| TDD UL-DL pattern | 60kHz/120khz SCS: DDSUU, S=10D:2G:2U |
| phase/power error model | disabled |
| Antenna configuration | 1Tx2Rx |

Table 2.1-5 Simulation results for PUCCH format 1 with JCE, 60kHz SCS, 50MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NACK ACK <0.1% | ACK miss <1% |
| 1 | 2 | Normal | TDLA30-75 low | 2 | -5.3 | -5.2 |

Table 2.1-6 Simulation results for PUCCH format 1 with JCE, 120kHz SCS, 50MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NACK ACK <0.1% | ACK miss <1% |
| 1 | 2 | Normal | TDLA30-75 low | 2 | -5.3 | -5.2 |

## 2.2	PUCCH format 3

Table 2.2-1 Simulation parameters for PUCCH format 3 with JCE for FR1

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | disabled |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Number of slots for PUCCH repetition | 2 |
| Test metric | SNR@BLER<1% |
| JCE cTDW | 2 slots or TDD, 8 slots for FDD |
| JCE aTDW | 2 slots or TDD, 8 slots for FDD |
| PUCCH repetition factor | 2 slots or TDD, 8 slots for FDD |
| TDD UL-DL pattern | 15kHz/30kHz SCS: FDD and 7D1S2U, S=6D:4G:4U |
| phase/power error model | disabled |
| Antenna configuration | 1Tx2Rx |
| Additional DM-RS configuration | Pos0 and pos1 |

Table 2.2-2 Simulation results for PUCCH format 1 with JCE, 15kHz SCS, 5MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | DM-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 low | 2 | 1+0 | -2.2 |
|  |  |  |  |  | 1+1 | -2.6 |
|  |  |  |  | 8 | 1+0 | -8.2 |
|  |  |  |  |  | 1+1 | -8.4 |

Table 2.2-3 Simulation results for PUCCH format 1 with JCE, 30kHz SCS, 10MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | DM-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 low | 2 | 1+0 | -2.3 |
|  |  |  |  |  | 1+1 | -2.6 |
|  |  |  |  | 8 | 1+0 | -8.2 |
|  |  |  |  |  | 1+1 | -8.5 |

Table 2.2-4 Simulation parameters for PUCCH format 3 with JCE for FR2

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | disabled |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Number of slots for PUCCH repetition | 2 |
| Test metric | SNR@BLER<1% |
| JCE cTDW | 2 slots |
| JCE aTDW | 2 slots |
| PUCCH repetition factor | 2 slots |
| TDD UL-DL pattern | 60kHz/120kHz SCS: DDS44, S=10D:2G:2U |
| phase/power error model | disabled |
| Antenna configuration | 1Tx2Rx |
| Additional DM-RS configuration | Pos1 |
| PT-RS configuration | Disabled |

Table 2.2-5 Simulation results for PUCCH format 1 with JCE, 60kHz SCS, 50MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | DM-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-75 low | 2 | 1+1 | -2.6 |

Table 2.2-6 Simulation results for PUCCH format 1 with JCE, 120kHz SCS, 50MHz BW

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | cTDW | DM-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-75 low | 2 | 1+1 | -2.7 |

# Conclusions

Simulation results for PUCCH with JCE demodulation are delivered.

# References

[1]	R4-2210667, WF for NR coverage enhancement PUCCH with JCE demodulation, Huawei HI Silicon, RAN4#103-e