3GPP TSG-RAN WG4 Meeting # 104-e                                                  R4-2212282			R4-22xxxxx

# Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	9.18.4.2.1

Source: 	CMCC

Title: 			      On RRM test cases configurations for FR1 RedCap

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed and agreed the test case list for RedCap RRM performance part [1]. According to the work plan [2], companies will bring phase 1 test cases in this meeting.

In this contribution, we discuss some RRM test configurations for FR1 RedCap and provid our views.

- Discussion

Test configurations

In existing TS 38.133, the supported test configurations for normal UE are as follows.

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

For RedCap UE, since RedCap UE supports channel bandwidths up to 20 MHz, the bandwidth for TDD duplex mode needs to be reduced. In addition, for duplex mode, HD-FDD should be added for RedCap. Hence, we propose the following test configurations for RedCap.

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15KHz SSB SCS, 10 MHz bandwidth, HD-FDD mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |

Proposal 1: Specify following RRM test configurations for RedCap UE.

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15KHz SSB SCS, 10 MHz bandwidth, HD-FDD mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |

BWP configurations

For RedCap UE, since RedCap UE supports up to 20MHz, the bandwidth for TDD duplex mode needs to be reduced. In addition, for duplex mode, HD-FDD should be added for RedCap. Hence, we propose the following test configurations for RedCap.

The existing initial BWP configuration in TS38.133 can be reused for RedCap. But for dedicated BWP configurations, it is proposed to specify BWP with NCD-SSB considering that UE-specific RRC configured DL BWP with NCD-SSB is mandatory supported by NCD-SSB RedCap UE should mandatory supports NCD-SSB.

In addition, in existing spec, the number of RBs for dedicated BWP is 41 for 30KHz, which corresponds to 20MHz. Considering that RedCap UE supports channel bandwidth up to 20MHz, new BWP configuration with smaller number of RBs need to be specifiec, i.e. 24RBs.

Proposal 2: Specify UE dedicated  BWP configuration with NCD-SSB for RedCap .

Proposal 3: Specify UE dedicated BWP configuration with smaller number of RBs for TDD RedCap, i.e. 24RBs.

- Conclusions

In this contribution, we discuss some RRM test configurations for FR1 RedCap and provid our views.

Proposal 1: Specify following RRM test configurations for RedCap UE.

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15KHz SSB SCS, 10 MHz bandwidth, HD-FDD mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |

Proposal 2: Specify UE dedicated  BWP configuration with NCD-SSB for RedCap .

Proposal 3: Specify UE dedicated BWP configuration with smaller number of RBs for TDD RedCap, i.e. 24RBs.

- Reference

[1] R4-2210595, Test case list for RedCap RRM performance part, Ericsson.

[2] R4-2211050, Workplan to RedCap RRM performance requirements, Ericsson.