3GPP TSG-RAN WG4 Meeting #104-e	R4-2212326

e-Meeting, Aug. 2022

Title:	Reply LS to RAN1 on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs

Source:	Qualcomm Incorporated

Agenda item:	13.2.?

Document for:	Approval

# 1.	Introduction and Discussion

The LS from RAN1 [1] seeks to determine RAN4 guidance on the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation.

Based on RAN1 agreements, there is the following broad categorization of UL in this context:

- TDM based multi-DCI uplink transmission: FR1 and FR2

- Simultaneous multi-DCI uplink transmission: FR2 only

There are two concerns: ability of the baseband to process UL with timing misalignment, and the possibility of time conflicts across TAGs.

The baseband ability is already captured in the form of the inter-band CA MTTD and applies to both cases, TDM and simultaneous multi-DCI UL. That parameter value serves equivalently as an upper bound for maximum TA difference.

Observation 1: The inter-band CA MTTD is an upper-bound for TA difference in the multi-DCI multi-TRP case.

In TDD bands, there is no meaningful front-end protection for the receiver. Here, for the TDM case, there is the potential for overlapping Tx symbols when there is a TCI-state switch. In our view, some clarification of prioritization or similar treatment in RAN1 would be useful.

Finally, there is the potential of UL and DL symbol overlap, but this is an old problem and is assumed to have been addressed with the proper choice of ‘F’ symbols in the ‘S’ slot.

Observation 2: For the TDM multi-DCI UL case, there is potential for overlap between UL symbols across TAGs when there is a TCI-state switch.

These conclusions are FR-agnostic and apply to any TDD band.

Proposal: (reply LS draft in the annex.)

# 2. 	References

- R4-2211512, ‘LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs’, RAN1, RAN4#104-e, Aug. 2022

# 3GPP TSG-RAN WG4 Meeting # 104-e 	R4-22xxxxx

Online Meeting, Aug. 2022

Title:	[Draft] Reply LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs

Response to:	(R1-2205593) LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs

Source:	RAN4

To:	RAN1

Contact Person:

Name	: Sumant Iyer

E-mail Address	: sumanti _at_ qti_dot_qualcomm_dot_com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs.

The maximum TA difference that can be supported by a UE in the specified conditions is the MTTD parameter for inter-band NR CA and is captured in TS38.133:

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2 | 8.5 Note1 |
| Between FR1 and FR2 | 26.1 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA. |  |

RAN4 have also identified that in the TDM case, there is potential for UL symbol overlap across TAGs in case of a TCI-state switch, and request RAN1 to kindly take this into account if it has not already done so.

2. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #104Bis-e	Oct. 2022         		online

TSG-RAN WG4 Meeting #105-e	Nov. 2022         		Canada