3GPP RAN WG4 Meeting #104e		R4-2212338

Electronic meeting, August 15 – 26th, 2022

Agenda item:	10.20.1

Source:	Apple

Title:	Overview of the regulatory parameters and potential NS values

WI/SI:	NR_unlic_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which has been approved recently in different regulatory regions. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI [2], RAN#96 meeting approved a new WI [3] with the main motivation of enabling LPI and VLP in countries not covered by previous efforts.

In this discussion paper we present an overview of the 6GHz regulatory rules for the LPI and VLP modes with the intention of identifying whether they can be covered by the existing NS values or new NS values are needed.

# 2	Overview of the 6GHz band regulatory rules

For the sake of further clarity, Table 2-1 below presents an excerpt from TR 38.849 with information on the existing NS values applicable to the unlicensed 6GHz band. As can be seen, while the support for the LPI mode was already added in previous releases covering most geographical regions, not all the countries were addresses. Furthermore, the VLP mode was considered only for South Korea, but not for other countries.

Table 2-1: Existing NS values for PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | TBD |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |

## 2.1	LPI

In this section we provide a brief overview of the 6GHz LPI mode regulatory parameters for countries not covered by the previous 3GPP releases. More detailed technical parameters can be found in Annex A and the corresponding 3GPP TR 37.890.

-	Hong Kong. Hong Kong regulations allow unlicensed operation in the lower part of the 6GHz band, 5925 – 6425MHz, whereupon the LPI regulatory parameters are aligned with the EU/CEPT technical conditions.

-	UK and Australia. Both UK and Australia have the same regulatory conditions allowing unlicensed operation in the lower 6GHz band, 5925 – 6425MHz, but unlike EU/CEPT they allow slightly higher Tx power and higher PSD. In addition to that, there are no out-of-band emission requirements.

-	Morocco. The Moroccan National Telecommunications Regulatory Agency (ANRT) published a new decision on Short Range Devices (SRD) in 5925-6425MHz frequency range allowing 23dBm EIRP for indoor usage only. And it should be noted that there are no additional PSD or out-of-band emission requirements.

-	UAE. The telecommunications Regulatory Authority (TRA) of UAE has designated 5925-6425MHz for indoor use at an EIRP of 250mW (24dBm) under class authorization. Like Morocco, there are no additional PSD or out-of-band emission requirements.

-	Saudi Arabia. This country allows LPI operation on the whole frequency range of 5925-7125MHz with the maximum Tx power of 24dBm and PSD limit of 10dBm/MHz. There are however no additional OOBE limits.

As can be seen from the summary, all countries except Saudi Arabia can be supported with the existing 3GPP band n102. Hong Kong LPI mode can be supported with the existing LPI NS_58 flag that was introduced for EU/CEPT. As for other countries such as Australia and UK, the existing NS_01 flag with the general requirements can be used at least for the PC5 power class. If we assume the minimum NR-U channel size of 20MHz and the PC5 Tx power of 20dBm, then the anticipated PSD will be 7dBm/MHz, which is below e.g. UK and Australia PSD requirements. Even if the interlaced allocations are used, then the anticipated PSD will be around 10dBm/MHz; and the NS_01 flag will anyway allow additional power back-off, if so needed by the UE.

As for Saudi Arabia, since the LPI mode is allowed for the whole range of 5925-7125MHz, the existing 3GPP band n96 can be re-used. Nevertheless, this country does not require a new NS flag because its PSD limit of 10dBm/MHz does not create any restrictions for the PC5 PA.

Proposal 1a:	Re-use 3GPP band n102 for the LPI operation in Morocco, UAE, Australia, UK and Hong Kong.

Proposal 1b:	Re-use 3GPP band n96 for the LPI operation in Saudi Arabia.

Proposal 1c:	An existing NS_58 flag can be used to support LPI operation in Hong Kong (at least for PC5).

Proposal 1d:	An existing NS_01 flag can be used to support LPI operation in UK, Australia, Morocco, Saudi Arabia and UAE (at least for PC5).

Based on the presented considerations, an updated summary of NS values for the LPI mode can be as presented in the Table 2.1-1 below, where the highlighted parts are new countries and NS values.

Table 2.1-1: Updated NS values for the PC5 power class.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | TBD |
| UK | N/A | NS_01 | TBD |
| Morocco | N/A | NS_01 | TBD |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | TBD |
| Australia | N/A | NS_01 | TBD |

## 2.2	VLP

The VLP mode for the 6GHz band was not properly introduced back in Rel-17, except South Korea for which a new NS value NS_67 was preliminary allocated. Thus, in this section we present the corresponding overview of the VLP parameters with an intention to group them under the same NS value(s).

-	Hong Kong. Hong Kong regulations are identical to EU/CEPT allowing VLP unlicensed operation in the lower part of the 6GHz band, 5925 – 6425MHz.

-	Australia. Australian VLP regulations are similar to EU/CEPT, 14dBm max Tx power and 1dBm/MHz PSD, with a difference that 5925-6425MHz can be used for the VLP mode and that there are no OOBE emission requirements.

-	UK, Morocco and Costa Rica. The UK VLP regulations are identical to Australia, but they allow quite high PSD of 11dBm/MHz.  Moroccan and Costa Rican regulations also define VLP mode with the maximum Tx power of 14dBm, but without any PSD or OOBE restrictions.

-	Canada. While Canadian regulations also allow 14dBm Tx power for the VLP mode, there are much stricter requirements on PSD allowing only -8dBm/MHz.

-	Brazil and Chile. Brazil and Chile allow 17dBm Tx power for the VLP mode, whereupon Brazil also limits the maximum PSD to -5dBm/MHz.

Based on the presented considerations it is possible to assume that VLP operation for both EU/CEPT and Hong Kong can be supported with band n102 and under the same NS flag. However, a new NS flag with the associated A-MPR values should be anyway allocated.

Australia can be also supported with the existing band n102, but it will require another NS flag allowing relaxed A-MPR values because there are no OOBE requirements. As for UK, Morocco and Costa Rica, it should be noted that UK allows quite high PSD comparing to Australia and Morocco has no PSD restrictions at all. Thus, these countries may require yet another NS value with "flat" 6dB A-MPR values.

Canada and Brazil VLP mode can be supported with the existing band n96. However, new NS flags are needed for both countries because these countries regulatory requirements are not the same. For instance, Canada allows the VLP operation with 14dBm and -8dBm/MHz PSD. On the contrary to it, Brazil allows higher 17dBm Tx power with a relaxed -5dBm/MHz PSD.

Proposal 2a:	Re-use 3GPP band n102 for the VLP operation in EU/CEPT, Morocco, Australia, UK and Hong Kong.

Proposal 2b:	Re-use 3GPP band n96 for the VLP operation in Canada, Brazil, Chile, Costa Rica, South Korea.

Proposal 2c:	A new NS flag to support VLP operation in EU/CEPT and Hong Kong.

Proposal 2d:	A new NS flag to support VLP operation in UK, Morocco and Costa Rica.

Proposal 2e:	A new NS flag to support VLP operation in Canada.

Proposal 2f:	A new NS flag to support VLP operation in Brazil and Chile.

Proposal 2g:	A new NS flag to support VLP operation in Australia.

Table 2.2-1: Updated NS values for PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | NS_y1 (new) |
| UK | N/A | NS_01 | NS_y2 (new) |
| Morocco | N/A | NS_01 | NS_y2 (new) |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | NS_y3 (new) |
| Brazil | N/A | NS_53 | NS_y4 (new) |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | NS_y4 (new) |
| Costa Rica | N/A | NS_01 | NS_y2 (new) |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | NS_y1 (new) |
| Australia | N/A | NS_01 | NS_y5 (new) |

## 2.3	Extending the maximum range of NS values

Referring to the preliminary summary of new NS values needed to support LPI and VLP modes for new countries, we might need at least four new NS values. And as can be seen from Table 2.3-1 below, we hit the limit for the available NS values at least for band n96, where we will need three NS new values, but there are only two empty slots. In fact, even more NS values will be needed if we account for the fact that the presented summary is only for the PC5 power class. Since the WI goal is to enable support also for the PC3 power class, there are countries that most likely will require a new NS flag and A-MPR values because NS_01 will not ensure sufficient back-off margins.

Table 2.3-1: Mapping of NS values (based on Table 6.2F.3.1-1A from TS 38.101-1).

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  |  |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 | [NS_61] | NS_y2 | NS_y3 |
| n102 | NS_01 | NS_58 | NS_y1 | NS_y2 | NS_y5 |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

The existing additionalSpectrumEmission IE is encoded as a 3-bit value thus allowing 8 different values. In principle, 32 different NS values should be more than sufficient and thus it is suggested to ask RAN WG2 to extend the corresponding IE.

Proposal 3:	Send LS to RAN WG2 asking to extend the existing range of NS values up to 32.

# 3	Conclusions

In this contribution we have presented summary of the regulatory requirements of the 6GHz band LPI and VLP modes for the countries that were not address by previous WIs.  Based on the presented considerations existing band n96 and n102 can be re-used to support the LPI and VLP mode. However, new NS flags are needed for the 3GPP bands n96 and n102 because there are several countries that have quite different regulatory requirements (the associated A-MPR values can be found in [4]).

Finally, we suggest sending the LS to RAN WG2 to extend the range of NS values because there are not enough empty slots. The draft LS document can be found in [5].

Proposal 1a:	Re-use 3GPP band n102 for the LPI operation in Morocco, UAE, Australia, UK and Hong Kong.

Proposal 1b:	Re-use 3GPP band n96 for the LPI operation in Saudi Arabia.

Proposal 1c:	An existing NS_58 flag can be used to support LPI operation in Hong Kong (at least for PC5).

Proposal 1d:	An existing NS_01 flag can be used to support LPI operation in UK, Australia, Morocco, Saudi Arabia and UAE (at least for PC5).

Proposal 2a:	Re-use 3GPP band n102 for the VLP operation in EU/CEPT, Morocco, Australia, UK and Hong Kong.

Proposal 2b:	Re-use 3GPP band n96 for the VLP operation in Canada, Brazil, Chile, Costa Rica, South Korea.

Proposal 2c:	A new NS flag to support VLP operation in EU/CEPT and Hong Kong.

Proposal 2d:	A new NS flag to support VLP operation in UK, Morocco and Costa Rica.

Proposal 2e:	A new NS flag to support VLP operation in Canada.

Proposal 2f:	A new NS flag to support VLP operation in Brazil and Chile.

Proposal 2g:	A new NS flag to support VLP operation in Australia.

Proposal 3:	Send LS to RAN WG2 asking to extend the existing range of NS values up to 32.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-213180, "Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2212341, "VLP A-MPR simulation results for PC5", Apple Inc.

- R4-2212339, "[draft] LS on extending the maximum range for NS values", Apple Inc.

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |