3GPP RAN WG4 Meeting #104-e		R4-2212352

Online, August 15th – 26th, 2022

Agenda item:	4.1.4

Source:	Apple

Title:	Clarifications on LTE P-Max definition

WI/SI:	TEI15

Release:	Rel-15

Document for:	Approval

# 1	Introduction

The IE P-Max has been defined in LTE since Rel-8 [1]. It is used to limit UE's maximum uplink transmission power and corresponds to the parameter PEMAX in TS 36.101 [2]. The UE behaviour based on the original P-Max definition is that when P-Max is absent, the UE applies the maximum power according to its capability as specified in TS 36.101, clause 6.2.2 which should be applicable to all frequency bands. Prior to high power UE (HPUE) was introduced, most networks did not broadcast P-Max as either the PC3 UE maximum output power at 23 dBm was compliant with the regulatory maximum output power limit in certain region/country or there is no specific regulatory maximum output power limit in other regions/countries. However, when PC1 for Band 3, Band 20, and Band 28 was introduced which was after PC1 for Band 14 [3] and PC2 for Band 41 [4] and Band 47 [5], a new approach on specifying HPUE requirements for bands other than Band 41 was proposed and agreed in order to avoid violating the regulation without additional efforts on network side to signal P-Max [6-8]. The proposed approach though might have resolved the potential concern for certain networks, it unfortunately created a split P-Max definitions in TS 36.101. On the other hand, the issue might have been shifted to other networks supporting HPUE for “bands other than Band 41” in the regions/countries without P-Max regulation (such as B14, B31, B38, B40, B42, etc.) if these networks only follow P-Max definition in TS36.331. In this contribution, we provide a summary on how the P-Max definitions have evolved in TS 36.101 and TS 36.331 alongside the introduction of HPUE for certain frequency bands as the background information to draw attention to the potential issue on the networks supporting HPUE. We also propose to have clarifications on the LTE networks supporting HPUE from all the network stakeholders as whether their networks would signal P-Max or not.

# Discussion

Table 2-1 summarizes the evolvement of the P-Max definitions in TS 36.101 and TS 36.331 alongside the introduction of HPUE for certain frequency bands.

| Document | Meeting | Excerpt |
| --- | --- | --- |
| TS 36.101 V8.29.0 | RAN #78 |  |
| TS 36.331 V8.21.0 | RAN #64 | p-Max Pmax to be used in the target cell. If absent the UE applies the maximum power according to the UE capability. |
| R4-126904.zip | RAN4 #65 | CR for LTE B14 HPUE (power class 1) No P-Max associated change |
| R4-168907.zip | RAN4 #80b | Introduction of power class 2 HPUE in Band 41 For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable,and the corresponding requirements for a power class 3 UE shall apply. |
| R4-1702211.zip | RAN4 #82 | CR for Tx RF requirements for high power V2X (Band 47) No P-Max associated change |
| R4-1708015.zip R4-1610826.zip R4-1609871.zip | RAN4 #84 | Introduction of power class 1 HPUE in Band 3, 20 and 28 For each supported frequency band other than Band 41, the UE shall: 1> if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class: 2> if the band is a TDD band whose frame configuration is 0 or 6; or 2> if the IE P-Max as defined in [7] is not provided; or 2> if the IE P-Max as defined in [7] is provided and set to the maximum output power of the default power class or lower; 3> apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.5; 2> else (i.e the IE P-Max as defined in [7] is provided and set to the higher value than the maximum output power of the default power class): 3> apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2.5; |
| R2-1803837.zip R2-1802908.zip | RAN2 #101 | Correction to handling of p-Max procedure for high-power UEs p-Max Pmax to be used in the target cell. If absent the UE applies the maximum power of the default power class for the band used in the target cell, defined according to 36.101[42]. |
| R4-1805695.zip R2-1808882.zip | RAN4 #86b RAN2 #102 | LS to RAN2 on P-max procedure for high-power UEs Correction to handling of p-Max procedure for high-power UEs p-Max Pmax to be used in the target cell. If absent, for the band used in the target cell, the UE applies the maximum power according to its capability as specified in 36.101[42, 6.2.2]. |
| R4-1713999.zip | RAN4 #85 | Introduction of power class 2 HPUE in Band 38 No P-Max associated change |
| R4-1803222.zip | RAN4 #86 | Introduction of power class 2 HPUE in Band 40 and 42 to 36.101 No P-Max associated change |
| TS 36.331 V17.0.0 | RAN #95 | p-Max Pmax to be used in the target cell. If absent, for the band used in the target cell, the UE applies the maximum power according to its capability as specified in 36.101 [42], clause 6.2.2. |

Table 2-1 Evolvement of the P-Max definitions in TS 36.101 and TS 36.331

Based on the information summarized in Table 2-1, we have arrived the following observations:

Observation 1: The new P-Max definition was proposed and agreed in TS 36.101 when PC1 HPUE for Band 3, 20, 28 was introduced which was intended for public safety networks and may not be applicable for commercial networks.

Observation 2: PC1 for Band 14 and PC2 for Band 47 were introduced before the new P-Max definition for “bands other than Band 41”. It is unclear whether Band 14 and Band 47 networks would follow the same P-Max definition as “bands other than Band 41”.

Observation 3: Though PC2 for Band 38, Band 40, and Band 42 were introduced after the new P-Max definition for “bands other than Band 41”, it is unclear whether they would follow the same P-Max definition as “bands other than Band 41” since the networks might already exist prior to the PC2 introduction.

Observation 4: PC1 for Band 5, Band 12, and Band 13 are expected to be introduced in Rel-18 for fixed-wireless/vehicle-mounted use cases. It is unclear whether those networks would follow the same P-Max definition as “bands other than Band 41” to signal P-Max higher than the default power class on the existing networks to support PC1.

If the aforementioned networks for “bands other than Band 41” would not signal P-Max higher than the default power class, HPUE would never be realized based on the expected UE behavior. Therefore, it is necessary for the concerned LTE network stakeholders to clarify as whether their networks would signal P-Max higher than the default power class or not for supporting HPUE.

Proposal: The concerned LTE network stakeholders to clarify as whether their networks would signal P-Max higher than the default power class or not for supporting HPUE.

# 3	Conclusion

In this contribution, we provide a summary on how the P-Max definitions have evolved in TS 36.101 and TS 36.331 and propose the concerned LTE network stakeholders to clarify as whether their networks would signal P-Max higher than the default power class or not for supporting HPUE.

Observation 1: The new P-Max definition was proposed and agreed in TS 36.101 when PC1 HPUE for Band 3, 20, 28 was introduced which was intended for public safety networks and may not be applicable for commercial networks.

Observation 2: PC1 for Band 14 and PC2 for Band 47 were introduced before the new P-Max definition for “bands other than Band 41”. It is unclear whether Band 14 and Band 47 networks would follow the same P-Max definition as “bands other than Band 41”.

Observation 3: Though PC2 for Band 38, Band 40, and Band 42 were introduced after the new P-Max definition for “bands other than Band 41”, it is unclear whether they would follow the same P-Max definition as “bands other than Band 41” since the networks might already exist prior to the PC2 introduction.

Observation 4: PC1 for Band 5, Band 12, and Band 13 are expected to be introduced in Rel-18 for fixed-wireless/vehicle-mounted use cases. It is unclear whether those networks would follow the same P-Max definition as “bands other than Band 41” to signal P-Max higher than the default power class on the existing networks to support PC1.

Proposal: The concerned LTE network stakeholders to clarify as whether their networks would signal P-Max higher than the default power class or not for supporting HPUE.

# 4	References

TS 36.331 V8.21.0 (2014-07-02)

TS 36.101 V8.29.0 (2018-01-17)

R4-126904 “CR for LTE B14 HPUE”, Motorola Solutions et al, 3GPP TSG-RAN WG4 Meeting #64, New Orleans, Louisiana, USA, November 12th – 16th, 2012

R4-168907 “Introduction of power class 2 HPUE in Band 41”, Qualcomm Incorporated et al, 3GPP TSG-RAN WG4 Meeting #80bis, Ljubljana, Slovenia, October 10th – 14th, 2016

R4-1702211 “CR for Tx RF requirements for high power V2X”, CATT and LGE, 3GPP TSG-RAN WG4 Meeting #82, Athens, Greece, February 13th – 17th, 2017

R4-1609871 “P-Max related requirements for HPUE”, NTT DOCOMO, INC., 3GPP TSG-RAN WG4 Meeting #81, Reno, Nevada, USA, November 14th – 18th, 2016

R4-1610826 “Draft CR for HPUE for operating bands other than Band 41”, NTT DOCOMO, INC., 3GPP TSG-RAN WG4 Meeting #81, Reno, Nevada, USA, November 14th – 18th, 2016

R4-1708015 “Introduction of power class 1 HPUE in Band 3, 20 and 28”, Huawei et al, 3GPP TSG-RAN WG4 Meeting #84, Berlin, Germany, August 21st – 25th, 2017