3GPP RAN WG4 Meeting #104-e		R4-2212372

Online, , 15th Aug 2022 - 26th Aug 2022

Agenda item:	9.14.3.1

Source:	Apple

Title:	Discussion on minimum UE EIRP scaling for FR2-2

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Document for:	Decision

# 1  Introduction

At RAN4#103-e various Tx requirements for the new band n263 were introduces in CR [1]. The CR introduced a new table for FR2-2 minimum UE EIRP requirements which are dependent on the actual channel bandwidth. The general scaling aspect was agreed with tentative values for each channel. Due to the short time frame in which the CR had to be crafted during the meeting an oversight was introduced for minimum UE EIRP requirements. This contribution discusses the oversight and proposes a potential correction.

# 2  Discussion

The CR [1] introduced a new table for FR2-2 minimum UE EIRP requirements which are dependent on the actual channel bandwidth. The general scaling aspect was agreed with tentative values for each channel. Due to the short time frame in which the CR had to be crafted during the meeting an oversight was introduced for minimum UE EIRP requirements.

Below is an excerpt from TS 38.101-2 v17.6.0. The UE EIRP levels for 100MHz CBW in Table 6.4.2.1-3a are 3dB tighter compared to the ones listed in Table 6.4.2.1-3 for FR2-1. This is not logical as generally the phase noise contribution tends to increase with higher frequencies. Due to this relation this should result into relaxed values if different values for FR2-2 would be considered compared to FR2-1. Therefore, we propose to correct those values that those start at least at FR2-1 level. The correction is indicated with red highlighting and follows the agreed scaling pattern.

Table 6.4.2.1-3: Parameters for Error Vector Magnitude for power class 2, 3, 4 and 7 in FR2-1 Parameter; Unit; Level. UE EIRP; dBm; -13. UE EIRP for UL 16 QAM; dBm; -10. UE EIRP for UL 64 QAM; dBm; -6. Operating conditions; Normal conditions. Table 6.4.2.1-3a: Parameters for Error Vector Magnitude for power class 3 in FR2-2 Level. Parameter; Unit; 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. UE EIRP; dBm; [ -16 -13]; [ -13 -10]; [ -10 -7]; [ -7 -4]; [ -6 -3]. UE EIRP for UL 16 QAM; dBm; [ -13 -10]; [ -10 -7]; [ -7 -4]; [ -4 -1]; [ -3 0]. UE EIRP for UL 64 QAM; dBm; [ -9 -6]; [ -6 -3]; [ -3 0]; [ 0 3]; [ 1 4]. Operating conditions; Normal Conditions. NOTE 1: PTRS is configured for 16 QAM and 64 QAM.

Observation: Due to the short time frame in which the CR had to be crafted during the meeting an oversight was introduced for minimum UE EIRP requirements. The UE EIRP levels for 100MHz CBW in Table 6.4.2.1-3a are 3dB tighter compared to the ones listed in Table 6.4.2.1-3 for FR2-1. It is not logical that a UE in FR2-2 range can perform better than in FR2-1 range as phase noise contribution tends to increase with higher frequencies. If any a relaxation would be required.

Proposal: Correct the tentative FR2-1 minimum UE EIRP values for 100MHz by replacing those with the values used for FR2-1 and follow the agreed scaling pattern from CR R4-2211196. The changes have been implemented in the companion CR R4-2212373.

# 3  Conclusions

###### This contribution shortly discusses the tentative values for FR2-2 minimum UE EIRP. It was found that there is an oversight leading to tighter requirements compared to FR2-1. It is proposed to correct those values.

Observation: Due to the short time frame in which the CR had to be crafted during the meeting an oversight was introduced for minimum UE EIRP requirements. The UE EIRP levels for 100MHz CBW in Table 6.4.2.1-3a are 3dB tighter compared to the ones listed in Table 6.4.2.1-3 for FR2-1. It is not logical that a UE in FR2-2 range can perform better than in FR2-1 range as phase noise contribution tends to increase with higher frequencies. If any a relaxation would be required.

Proposal: Correct the tentative FR2-1 minimum UE EIRP values for 100MHz by replacing those with the values used for FR2-1 and follow the agreed scaling pattern from CR R4-2211196. The changes have been implemented in the companion CR R4-2212373.

# 4  References

- R4-2211196, “Draft CR to 38.101-2 on band n263 Tx aspects”, Apple, 3GPP RAN4 Meeting #103-e