3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212381

# Electronic Meeting, 15 Aug. – 26 Aug., 2022

Source:	LG Electronics

Title:	Discussion on test cases for cell reselection to intra- and inter-frequency neighbor cell

Agenda item:	9.11.6.2

Document for:	Discussion

Introduction

In RAN4 #103-e meeting, WF on timing requirements and performance requirements for NTN is approved [1]. However, test cases for cell reselection to intra- and inter-frequency neighbor cell is not discussed. We provide our view on test cases for cell reselection.

Discussion

Since TDD and FR2 is not considered in NTN Rel-17, RAN4 only needs to consider FDD test cases, and we think that the procedure for legacy TN cell reselection to FR1 intra- and inter- frequency with configuration 1 (FDD) can be reused to NTN cell reselection test cases.

      - Proposal 1: The test procedure for legacy TN cell reselection to FR1 intra- and inter- frequency with configuration 1 (FDD) can be reused.

However, test parameters, especially for the signal quality and strength parameters, need to be defined to model the satellite communication conditions. For example, in legacy TN cell reselection to FR1 intra-frequency NR case, following parameters are considered as list in Table 1 below.

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
|  | dB | 1 | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -46.12 | -46.12 |  |  |  |

Table 1. Legacy TN test parameters

As shown in Table 1, signal strength is too strong to model satellite communication condition. So, RAN4 needs to study proper values of parameters to model satellite communication condition.

      - Proposal 2: RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition.

For the test cases for relaxed measurement criterion (e.g. A.6.1.1.3, A.6.1.1.4, A.6.1.1.5, A.6.1.1.6 in TS 38.133), GEO measurement relaxation can be considered since the measurement relaxation for GEO is defined in Rel-17 NTN unlike the measurement relaxation for LEO in [2][3]. However, we are not sure the measurement relaxation test case for GEO is actually needed.

      - Proposal 3: Study whether to define the measurement relaxation test case for GEO.

Conclusion

In this contribution, we provide our views,

      - Proposal 1: The test procedure for legacy TN cell reselection to FR1 intra- and inter- frequency with configuration 1 (FDD) can be reused.

      - Proposal 2: RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition.

      - Proposal 3: Study whether to define the measurement relaxation test case for GEO.

Reference

[1] R4-2210612, WF on UE timing requirements and NTN RRM performance requirements

[2] R4-2202637, WF on NR NTN RRM requirements

[3] R4-2207114, WF on NR NTN RRM requirements