3GPP TSG-RAN4 #104-e	R4-2212534

Electronic Meeting, 15th August 2022 – 26th August 2022

Title:	TDD RMC for Intra-band EN-DC

Source:			Anritsu Limited

Agenda Item:			4.1.3.

Document for:			Approval

# 1. Introduction

There are currently no default UL TDD RMC definitions really compatible for intra-band EN-DC in TS 38.101-1 [1] or TS 38.101-3 [2], this has an impact on all the test cases where the TDD configuration is necessary along with the intra-band EN-DC setting. This may lead to different implementations of the test cases for intra-band EN-DC case due to different interpretation of the TS 38.101-1 [1] and other related specifications.

This aim of this contribution is to highlight the issue by explaining the status and propose two solutions to solve the issue.

# 2. Discussion

## 2.1. Current issue

For contiguous and non-contiguous intra-band EN-DC in TDD, the implicit assumption for intra-band is that a UE can only transmit or receive a combined LTE/NR signal at a given time instant, this using one TRx path common to both RATs. That implies in normal operation and in testing as when using TDD RMC for intra-band for intra-band EN-DC:

- an alignment of actual LTE downlink slots and NR downlink slots.

- an alignment of actual LTE uplink slots and NR uplink slots.

The extract below from the TS 38.101-3 [2] sub-clause 4.2 agrees with that:

- “e)  All the requirements for intra-band contiguous and non-contiguous EN-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the NR for the EN-DC.”

However it is not reflected in TS 38.101-3 [2] and TS 38.101-1 [1] as shown in Table 1, there is currently a misalignment between the E-UTRA “Uplink-downlink configuration” and NR “TDD Slot Configuration pattern” specified respectively in Table A.2.2.1.1-1 of the 38.101-3 [2] and Table A.3.3.1-1 of the 38.101-1 [1] : for several subframes of a frame, the two RATs use different links as highlighted in yellow (it could even be extended to the case of subframes using a special subframe S).

Table 1: Misalignment of uplink and downlink subframes between E-UTRA and NR

| RAT | Subframe number |  |  |  |  |  |  |  |  |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| E-UTRA | D | S | U | D | D | D | S | U | D | D | E-UTRA TDD Uplink-Downlink Configuration #2 |
| NR | D | D | D | S | U | D | D | D | S | U | TDD Slot Configuration pattern for 15kHz SCS |

The demonstration of this misalignment follows from “Table A.2.2.1.1-1: Reference Channels for QPSK with full RB allocation” of the 38.101-3 [2], where is shown the use of E-UTRA TDD Uplink-Downlink Configuration #2 (with the use of Special subframe configuration #7) which is defined as having a 10ms frame made up of the concatenation of twice the 5-SF {D, S, U, D, D} as shown in Table 4.2-2 of TS 36.211 [3]. Full UL subframes are allocated to subframe #2 and #7 of the 10ms frame.

Other tables in 38.101-3 [2] give the same indication of Uplink-Downlink Configuration #2 in section “A.2 UL reference measurement channels for E-UTRA TDD Config 2” and section “A.3 DL reference measurement channels for E-UTRA”.

Table 2: "Table A.2.2.1.1-1: Reference Channels for QPSK with full RB allocation" from 38.101-3 [2] showing the use of E-UTRA TDD Uplink-Downlink Configuration #2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/5 | 1/6 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 600 | 1544 | 2216 | 5160 | 4392 | 4584 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 1728 | 4320 | 7200 | 14400 | 21600 | 28800 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

Table 3: “Table 4.2-2: Uplink-downlink configurations” from the TS 36.211[3] – Uplink-downlink configuration #2 is highlighted in yellow corresponding to NOTE 2 of Table 2

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

Whereas as show in Table A.3.3.1-1 Additional reference channels parameters for TDD of the TS 38.101-1 [1] and taking the simplest case of NR 15kHz SCS, the TDD Slot Configuration pattern is {D, D, D, S, U}, full UL subframes are allocated to subframe #4 and #9 of a 10ms frame considering the concatenation of twice that pattern.

Table 4: “Table A.3.3.1-1 Additional reference channels parameters for TDD” from the TS 38.101-1 [1]

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSU | 7DS2U | 14DS1S24U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U | 6D+4G+4U | S1=12D+2G, S2=6G+8U |
| referenceSubcarrierSpacing |  | 15 kHz | 30 kHz | 60 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. NOTE 3: i is the slot index per frame. |  |  |  |  |

Observation 1: The misalignment of UL/DL slots between the E-UTRA “Uplink-downlink configuration” and NR “TDD Slot Configuration pattern” specified respectively in Table A.2.2.1.1-1 of the 38.101-3 [2] and Table A.3.3.1-1 of the 38.101-1 [1] makes the use of a single UE TRx path for both RATs when in TDD intra-band EN-DC not possible.

Observation 2: TS 38.101-1 [1] does not mention if a frame offset can be applied to NR frame for it to be synchronized with the E-UTRA frame in terms of UL/DL slots.

## 2.2. Solution 1: New UL-DL configuration based on TS 38.133 [4].

As shown in 2.1. Current issue, there is a mismatch between the NR frame and E-UTRA frame proposed and specified in 38.101-1 [1] for TDD intra-band EN-DC. One solution is to find compatible E-UTRA “Uplink-downlink configuration” and NR “TDD Slot Configuration pattern”. As shown in Figure 1 and Figure 2, the TS 38.133 [4] does have such compatibility between:

- TDD uplink-downlink configuration #1 is ‘DSUUD’ (with S=9:3:2) proposed in “Table A.3.7.2.1-1: E-UTRAN cell specific test parameters for tests with all NR cells in FR1” of 38.133 [4].

- TDD UL/DL pattern 1 is ‘DSUU’ (with S=’10DL:2GP:2UL’) and TDD UL/DL pattern 2 is ‘D’ proposed in “Table A.3.1.4-1: TDD UL/DL configuration for SCS=15kHz” of 38.133 [4], the concatenation of both patterns resulting to the pattern ‘DSUUD’ (with S=’10DL:2GP:2UL’).

- TDD UL/DL pattern 1 is ‘3D1S4U’ (with S=’6DL:4GP:4UL’) and TDD UL/DL pattern 2 is ‘DD’ proposed in “Table A.3.1.4-2: TDD UL/DL configuration for SCS=30kHz” of 38.133 [4], the concatenation of both patterns resulting to the pattern ‘3D1S4UDD’ (with S=’6DL:4GP:4UL’).

- TDD UL/DL pattern is ‘DDDSU’ (with S=’10DL:2GP:2UL’) proposed in “Table A.3.1.4-3: TDD UL/DL configuration for SCS=60kHz” of 38.133 [4].

Regarding the NR 15kHz SCS, it must be noted that contrary to the current patterns in TS 38.101-1 [1] which has a DL/UL ratio of 3/1, the DL/UL ratio is 2/2 for the patterns specified in 38.133 [4].

Figure 1-NR 15kHz SCS

Figure 2-NR 30kHz SCS

Observation 3: Regarding the NR 15kHz SCS, it must be noted that contrary to the current patterns in TS 38.101-1 [1] which has a DL/UL ratio of 3/1, the DL/UL ratio is 2/2 for the patterns specified in TS38.133 [4].

Option 1: Use in TS 38.101-3 [2] the compatible E-UTRA TDD uplink-downlink configuration #1 and NR TDD UL/DL patterns (SCS of 15kHz, 30kHz and 60kHz) already used in TS 38.133 [4].

## 2.3. Solution 2: Configure NR with some time delay

Starting from the NR frame and E-UTRA frame proposed and specified in 38.101-1 for TDD intra-band EN-DC, another solution is possible which consists simply to apply a time offset to the NR “TDD Slot Configuration pattern”. relative the E-UTRA “Uplink-downlink configuration”.

As can be seen in Figure 3, Figure 4 and Figure 5 by applying a -2ms or +3ms time offset to the NR configuration pattern relative to the E-UTRA UL-DL configuration, both RATs use at the same slot time the same link (or DL slot happens with a Gap slot).

That solution is already in use TS 38.521-3 [5] like in sub-clause 6.2B.1.1.4 and it mentioned in several reports from different authorities.

Figure 3-NR 15kHz SCS

Figure 4-NR 30kHz SCS

Figure 5-NR 60kHz SCS

Observation 4: By applying a -2ms or +3ms time offset to the NR configuration pattern (specified in 38.101-3 [2]) relative to the E-UTRA UL-DL configuration (specified in 38.101-3 [2]), both RATs use at the same slot time the same link (or DL slot happens with a Gap slot) which allows correction operation in TDD Intra-band EN-DC.

Observation 5: Solution #2 is in use already in testing or in the field.

Option 2: Specify in TS 38.101-3 [2] (please see Annex A of this contribution for possible update of TS) the need to apply a +3ms time offset to the NR “TDD Slot Configuration pattern” relative the E-UTRA “Uplink-downlink configuration” for TDD Intra-band EN-DC.

# 3. Conclusion

It was shown that an issue exists in 38.101-1 an 38.101-3, which may haves led to different interpretations.

Based on the discussion above, we propose the following:

Observation 1: The misalignment of UL/DL slots between the E-UTRA “Uplink-downlink configuration” and NR “TDD Slot Configuration pattern” specified respectively in Table A.2.2.1.1-1 of the 38.101-3 and Table A.3.3.1-1 of the 38.101-1 makes the use of a single UE TRx path for both RATs when in TDD intra-band EN-DC not possible.

Observation 2: TS 38.101-1 does not mention if a frame offset can be applied to NR frame for it to be synchronized with the E-UTRA frame in terms of UL/DL slots.

Observation 3: Regarding the NR 15kHz SCS, it must be noted that contrary to the current patterns in TS 38.101-1 which has a DL/UL ratio of 3/1, the DL/UL ratio is 2/2 for the patterns specified in TS38.133.

Observation 4: By applying a -2ms or +3ms time offset to the NR configuration pattern (specified in 38.101-3) relative to the E-UTRA UL-DL configuration (specified in 38.101-3), both RATs use at the same slot time the same link (or DL slot happens with a Gap slot) which allows correction operation in TDD Intra-band EN-DC.

Observation 5: Solution #2 is in use already in testing or in the field.

Proposal 1: Choose one of the following options to solve the identified issue:

- Option 1: Use in TS 38.101-3 the compatible E-UTRA TDD uplink-downlink configuration #1 and NR TDD UL/DL patterns (SCS of 15kHz, 30kHz and 60kHz) already used in TS 38.133.

- Option 2: Specify in TS 38.101-3 (please see Annex A of this contribution for possible update of TS) the need to apply a +3ms time offset to the NR “TDD Slot Configuration pattern” relative the E-UTRA “Uplink-downlink configuration” for TDD Intra-band EN-DC.

# 4. References

[1] 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, version 15.18.0

[2] 38-101-3, “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, version 15.18.0

[3] 36-211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, version 17.2.0

[4] 38-133, “NR; Requirements for support of radio resource management”, version 15.18.0

[5] 38-521-3, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, version 17.5.0

# Annex A - Add annex A.4.1 of 38.101-3

## A.4.1	DL reference measurement channels for TDD

### A.4.1.1	General

Table A.4.1.1-1 Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| TDD Slot Configuration pattern (Note 1, Note 4) |  | DDDSU | 7DS2U | 14DS1S24U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U | 6D+4G+4U | S1=12D+2G, S2=6G+8U |
| referenceSubcarrierSpacing |  | 15 kHz | 30 kHz | 60 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. NOTE 3: i is the slot index per frame. NOTE 4: A -2ms or +3ms time offset to the NR configuration pattern relative to the E-UTRA UL-DL configuration must be apply in the TDD intra-band EN-DC. |  |  |  |  |