3GPP TSG-RAN WG4 Meeting#104-e	R4-2212547

Online, 15th Aug – 26th Aug, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on FeMIMO HST Performance Requirements

Agenda item:	9.17.4.1.1

Document for:	Discussion

- Introduction

After RAN4#103-e meeting, there were some open issues as discussed in [1] and presented below. In this paper, we share our views on whether to introduce performance requirement for Scheme B and assumption of maximum Doppler Shift for 15KHz scenarios.

- Whether to define requirement with HST-SFN Scheme B

Issue 2-1-2: Whether to define PDSCH requireemnt with HST-SFN scheme B

Candidate options:

- Option 1: Introduced the PDSCH requirement with HST-SFN B under test applicability rule

  - FFS on the pre-compensation modeling

- Option 2: Not to introduce the PDSCH requirement with HST-SFN B

According to implementation of scheme B, PDSCH will be Doppler pre-compensated from the network side, therefore it does not impact the UE demod processing in a meaningful way that requires defining requirements for this scheme. Even in the presence of residual Doppler (after pre-compensation), UE should be able to deal with it as the residual Doppler is expected to be small.

Proposal 1: Option 2 (Not to introduce the PDSCH requirement with HST-SFN B)

- Maximum Doppler Shift

Issue 2-2-3: Maximum Doppler shift

Tentative agreements:

- RAN4 will make final decision about the value of Maximum Doppler shift in RAN4#104-E meeting based on the simulation results, e.g. compare the required SNR to achieve 70% of peak rate with MCS17 (or MCS13) with rank 2.

  - Option 1: 870Hz (baseline)

  - Option 2: 972Hz

The difference between the estimated Dopplers for TRP#1 (i.e., estimated from TRS1) and TRP#2 (i.e., estimated from TRS2) should still be within the TRS pull-in range. Furthermore, UE will have to come up with a mechanism to make use of the estimated Dopplers from two TRSs. Therefore, the assumption of single-tap model, for which a higher Doppler is assumed (option 2) may not be applicable here. Hence, we support a lower Doppler, i.e., option 1 for 15KHz SCS.

Proposal 2: (15KHz SCS) Option 1: 870 Hz

- Conclusions

In this paper, we share our views on the remaining open issues for scheme B and maximum Doppler shift. The following has been proposed.

Proposal 1: Option 2 (Not to introduce the PDSCH requirement with HST-SFN B)

Proposal 2: (15KHz SCS) Option 1: 870 Hz

# 6. References

[1] R4-2210670, “WF on demodulation requirement for Enhancement on HST-SFN deployment,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 9 – May 20, 2022.