# 3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212635

# Electronic Meeting, August 15 – August 26, 2022

# Source:	ZTE Corporation

# Title:          Discussion on FR2 UL 256qam results

# Agenda Item:	11.7.2

# Document for:	Discussion

- Introduction

In RAN 96 meeting, a new WID [1] for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved, where for FR2 UL 256QAM, the objections are:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

In this contribution, we provide some initial link level simulation results for CP-OFDM waveform to study the gain, operating SNR for 256QAM.

- Simulation assumptions and results

To study the gain and operating SNR, we compare 256QAM with 64QAM with the same simulation assumptions in link level simulation, which is shown in table 1.

Table 1 Simulation assumptions for FR2 UL 256QAM and 64QAM

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | 30 GHz |  |
| CBW | 100 MHz |  |
| SCS | 120kHz |  |
| Allocated RBs | Full allocation |  |
| Propagation | TDL-A 30ns delay spread, 3kmph TDL-D 30ns delay spread, 3kmph AWGN |  |
| MCS | 64QAM: MCS 23 in TS 38.214 Table 5.1.3.1-1. 256QAM: MCS 21 in TS 38.214 Table 5.1.3.1-2. Baseline: fixed MCSs. |  |
| Symbol type | CP-OFDM DFT-S-OFDM |  |
| HARQ | None |  |
| Antenna configuration | Fading channel: 2x2 for Rank1, Low correlation AWGN: 2x2 for Rank1 |  |
| Channel estimation | Practical |  |
| Receiver type | MMSE |  |
| PUSCH Time domain configuration | PUSCH mapping type | A |
|  | Start symbol index | 0 |
|  | Allocation length | 14 |
| DMRS configuration | 1+1 |  |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |  |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |  |
| Phase noise compensation | Practical based on PTRS |  |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option b): PN model config2: example2 (UE) + example2(BS) |  |
| txEVM + rxEVM excluding phase noise for 256QAM | 2.5% + 2.5% 3.5% + 3.5% 4.0% + 4.0% |  |

Figure 1 shows the performance comparison between 64QAM and 256QAM in TDL-D channel. From figure 1 we can find that 256QAM obtains large gain when SNR is larger than ~21dB and smaller EVM sets can obtain gain at lower SNR value.

Figure 1: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM in TDL-D

Figure 2 shows the performance comparison between 64QAM and 256QAM in TDL-A channel. Figure 2 shows that 256QAM obtains gain when SNR is over ~30dB in the condition of Tx EVM=2.5% and Rx EVM =2.5%, and the throughput gain is smaller compared with TDL-D channel.

Figure 2: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM for TDL-A

Performance comparison between 64QAM and 256QAM in AWGN are shown in figure 3, we can also find the obvious performance gain for 256QAM when SNR is over around 19dB for all the EVM sets in the assumption.

Figure 3: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM in AWGN

From the above initial results, we can find that 256QAM performance gain can be expected large in TDL-D channel while small in TDL-A channel. On the other hand, EVM assumptions will also impact performance gain for FR2 UL 256QAM, for example the 256QAM performance is not good as that of 64QAM in the TDL-A channel in the case of the Tx EVM 4.0 + Rx EVM 4.0.

Observation 1: 256QAM performance gain can be expected in AWGN and TDL-D channel.

Observation 2: 256QAM performance gain is not obvious in TDL-A channel.

Observation 3: EVM assumptions will impact performance gain for FR2 UL 256QAM.

- Conclusion

Based on the analysis, the following observations are given:

Observation 1: 256QAM performance gain can be expected in AWGN and TDL-D channel.

Observation 2: 256QAM performance gain is not obvious in TDL-A channel.

Observation 3: EVM assumption will impact performance gain for FR2 UL 256QAM.

- Reference

RP-221862 revised WID NR RF requirements enhancement for frequency range 2 (FR2), Phase 3.docx