3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2212662

# Electronic Meeting, Aug. 15 – Aug. 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2475 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on R17 RLM and BFD relaxation for UE power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The TSearchDeltaP-Connected should be captured in RAN2 spec 2. The clarification on multi-RS case is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove TSearchDeltaP-Connected. 2. Correct wording of clarification on multi-RS case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. TSearchDeltaP-Connected is captured in RAN4 spec. 2. Unclear UE behaviour when multi-RS is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports rlm-Relaxation-r17and configured with explicit signaling goodServingCellEvaluationRLM, which is always configured to the UE when the network enables RLM relaxation for the UE as specified in TS 38.331 [2], the relaxed requirements defined in clause 8.1.2.4 for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.4 for CSI-RS based radio link monitoring are allowed to apply to the relaxed RLM measurements on SpCell after fulfilling the following conditions:

-	The good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is fulfilled for any resource in the set of resources for radio link monitoring, and

-	for the UE which is not performing intra-band carrier aggregation or for the UE which is performing intra-band carrier aggregation but not configured with SSB-based or CSI-RS based RLM and CSI-RS based BFD on SCell, when

-	the UE has fulfilled good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for the SpCell if the lowMobilityEvaluationConnected is not configured, or

-	the UE is also configured with lowMobilityEvaluationConnected and UE has fulfilled both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for the SpCell.

-	for the UE is performing intra-band carrier aggregation configured with SSB-based or CSI-RS based RLM and CSI-RS based BFD on SCell, when

-	the UE has fulfilled good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for SSB-based or CSI-RS based RLM on SpCell and for CSI-RS based BFD on the serving cell in the intra-band carrier if the lowMobilityEvaluationConnected is not configured, or

-	the UE is configured with lowMobilityEvaluationConnected  and UE has fulfilled both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for SSB-based or CSI-RS based RLM on SpCell and for CSI-RS based BFD on the serving cell in the intra-band carrier.

otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

When more than one RLM-RSs are configured, the UE is allowed to enter relaxation when the radio link quality is better than the threshold (Qin + X dB) for at least one RS.

The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The timer T310 is running.

-	No DRX is used

<End of Change #1>

<Start of Change #2>

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

For the UE supports bfd-Relaxation-r17 and configured with dedicated signaling goodServingCellEvaluationBFD, which is always configured to the UE when the network enables BFD relaxation for the UE as specified in TS 38.331[2], the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection are allowed to apply to the relaxed BFD measuremetns on the serving cell after fulfilling the following conditions:

-	The good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is fulfilled for any resource in the set of resources for beam failure detection, and

-	for the UE which is not performing intra-band carrier aggregation or for the UE which is performing intra-band carrier aggregation but not configured with SSB-based or CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell, when

-	the UE has fulfilled good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for the serving cell configured with BFD-RS if the lowMobilityEvaluationConnected is not configured, or

-	the UE is also configured with lowMobilityEvaluationConnected  and UE has fulfilled both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for the serving cell configured with BFD-RS.

-	for the UE is performing intra-band carrier aggregation configured with SSB-based or CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell, when

-	the UE has fulfilled good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for SSB-based or CSI-RS based RLM on SpCell and for CSI-RS based BFD on the serving cell in the intra-band carrier if the lowMobilityEvaluationConnected is not configured, or

-	the UE is configured with lowMobilityEvaluationConnected, and UE has fulfilled both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2]  for SSB-based or CSI-RS based RLM on SpCell and for CSI-RS based BFD on the serving cell in the intra-band carrier.

otherwise, UE shall apply the requirements defined in clause 8.5.2.2 for SSB based beam failure detection and the requirements defined in clause 8.5.3.2 for CSI-RS based beam failure detection.

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

When more than one BFD-RSs are configured, the UE is allowed to enter relaxation when the radio link quality is better than the threshold (Qin + X dB) for at least one RS.

The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

-	The timer beamFailureDetectionTimer is running.

-	No DRX is used

<End of Change #2>