| 3GPP TR 37.8xx V0.0.1 (2022-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-18 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	TR Maintenance	7

5	General	7

6	Specific Band Combinations	7

6.1	Inter-band uplink CA	7

6.1.1	CA _n3-n40-n41	7

6.1.1.1	Configurations	7

6.1.1.2	Technical analysis	7

6.1.1.3	Conclusion	8

6.1.2	CA_n3-n41-n79	8

6.1.2.1	Configurations	8

6.1.2.2	Technical analysis	8

6.1.2.3	Conclusion	8

6.1.3	CA_n8-n39-n41	8

6.1.3.1	Configurations	8

6.1.3.2	Technical analysis	9

6.1.3.3	Conclusion	9

6.1.4	CA_n8-n41-n79	9

6.1.4.1	Configurations	9

6.1.4.2	Technical analysis	9

6.1.4.3	Conclusion	10

6.1.5	CA_n39-n41-n79	10

6.1.5.1	Configurations	10

6.1.5.2	Technical analysis	10

6.1.5.3	Conclusion	10

6.1.6	CA_n40-n41-n79	10

6.1.6.1	Configurations	10

6.1.6.2	Technical analysis	11

6.1.6.3	Conclusion	11

6.1.7	CA_n3-n40	11

6.1.7.1	Configurations	11

6.1.7.2	Technical analysis	11

6.1.7.3	Conclusion	12

6.1.8	CA_n1-n3-n78	12

6.1.8.1	Configurations	12

6.1.8.2	Technical analysis	12

6.1.8.3	Conclusion	12

Annex A (informative): Change history	14

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for WI “Rel-18 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink” under Rel-18 time frame. The purpose is to gather the relevant background information and studies in order to address the band combinations requested for mandating no DL interruption to conduct dynamic Tx switching in uplink listed in Table 1-1.

Table 1-1: Band combinations requested for mandating no DL interruption

| Configuration | Uplink configuration |
| --- | --- |
| CA_n1-n5-n78 | CA_n1-n78, CA_n5-n78 |
| CA_n3-n5-n78 | CA_n3-n78, CA_n5-n78 |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 		TS38.101-1, NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[3] 		3GPP TS 38.133: "NR; Requirements for support of radio resource management".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>		<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

At 3GPP RAN#96 meeting, a new basket Work Item “Rel-18 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink” was approved. The objectives are as following,

For the core part:

-  	Discuss and specify inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed when dynamic switching between two uplink carriers is conducted.

1)  Based on the requested band combinations from operators, analyze if it is feasible to specify no DL interruption in RAN4 specification.

2)  Based on the feasibility analysis for specific combination, decide to introduce no DL interruption in RAN4 specification.

For the performance part:

- 	For 2CC 1Tx-2Tx switching, the mandating of no DL interruption for new band combinations is release independent from Rel-16.

- 	For 3CC 1Tx-2Tx switching and 2CC or 3CC 2Tx-2Tx switching, the mandating of no DL interruption for new band combinations is release independent from Rel-17.

The present document is the technical report for this basket Work Item.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

5	Specific Band Combinations

5.1	NR band combinations

5.1.x	CA_nX-nY-nZ

5.1.x.1	Configurations

Table 5.1.x.1-1: CA configuration for mandating no DL interruption

| Configuration | Uplink configuration |
| --- | --- |
| CA_nX-nY-nZ | CA nX-nY, … |

5.1.x.2	Technical analysis

5.1.x.3	Conclusion

Table 5.1.x.3-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_nX-nY-nZ | nX, nY, nZ | No for CA nX-nY, … |
| NOTE 4: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [3]. |  |  |

5.2	EN-DC band combinations

5.2.x	DC_xxx

5.2.x.1	Configurations

5.2.x.2	Technical analysis

5.2.x.3	Conclusion

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104-e | R4-2212699 |  |  |  | TR skeleton | 0.0.1 |